| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0054 |
| --- | --- |

| Title: | Improved DBBP in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen1 Qin Yu2 Hongbin Liu1 Siwei Ma2 15775 Morehouse Drive San Diego, CA 92121 USA 2School of EECS, Peking University, Institute of Digital Media, Beijing, China | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated and Peking University |  |  |

_____________________________

The proposed text (Average of depth pixels that belong to 8 fixed positions as well as the corresponding depth region of current partition is used to derive disparity vector) is as follows. Deleted parts are marked as strikethrough and new added parts are highlighted in yellow.

- I.7.3.8.6	Prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if (!dbbp_flag[ x0 ][ y0 ]){ |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- I.7.4.9.5	Coding unit semantics

- dbbp_flag[ x0 ][ y0 ] equal to 1 specifies that depth based block partition is used for the current coding unit. dbbp_flag[ x0 ][ y0 ] equal to 0 specifies that depth based block partition is not used for the current coding unit. When not present, the value of dbbp_flag[ x0 ][ y0 ] is inferred to be equal to 0.

- When dbbp_flag[ x0 ][ y0 ] is equal to 1 and merge_flag[ x0 ][ y0 ] is equal to 0, the inter prediction parameters for the current prediction unit are derived from a depth block used for depth based block parition.

- I.8.4.4.4	Derivation process for a texture predicted contour pattern

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nTbS specifying the transform block size.

Output of this process is:

- binary partition pattern wedgePattern[ x ][ y ], with x, y =0..nTbS − 1.

The variable recTextPic is set equal to the array of the reconstructed luma picture samples of TexturePic.

For x = 0..nTbS − 1 and y = 0..nTbS − 1 the following applies:

  - refSamples[ x ][ y ] = recTextPic[ xTb + x ][ yTb + y ]

The derivation process for an inter-layer predicted contour pattern as specified in subclause  is invoked with the variable sampInt equal to 1, the block size nTbS and the a reference sample array refSamples[ x ][ y ] as inputs, and the output is the binary partition pattern wedgePattern[ x ][ y ] and a threshold value threshVal.

- I.8.5.1	General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause 8.6.1 is invoked with the luma location ( xCb, yCb ) as input.

The variable nCbSL is set equal to 1  <<  log2CbSize and the variable nCbSC is set equal to 1  <<  ( log2CbSize − 1 ).

The decoding process for coding units coded in inter prediction mode consists of following ordered steps:

  - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, following applies:

    - If DepthFlag is equal to 0 the derivation process for disparity vectors as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as inputs.

    - Otherwise (DepthFlag is equal to 1), the derivation process for disparity vectors for depth layers as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as inputs.

  - When dbbp_flag is equal to 1, the derivation process for a modified partitioning mode and derived disparity motion vectors as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ) and the coding block size nCbSL as the inputs.

…

- I.8.5.3.2	Derivation process for motion vector components and reference indices

…

The variables vspModeFlag, ivpMvFlag, subPbMotionFlag, dispDerivedDepthFlag and dispDerivedDepthVal are set equal to 0.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1, as well as predFlagL0 and predFlagL1, the following applies:

- If MergeFlag[ xPb ][ yPb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH, and the partition index partIdx as inputs, and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1,, the flag ivpMvFlag, the flag vspModeFlag, the flag subPbMotionFlag, the flag dispDerivedDepthFlag and the variable dispDerivedDepthVal.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, and refIdxLX, in PRED_LX, and in the syntax elements PuRefIdxLX and MvdLX, the following applies:

    - The variables refIdxLX and predFlagLX are derived as follows:

        - If InterPredIdc[ xPb ][ yPb ] is equal to PRED_LX or PRED_BI,

            - refIdxLX = PuRefIdxLX[ xPb ][ yPb ]	(Error: Reference source not found-1)

            - predFlagLX = 1		(Error: Reference source not found-2)

        - Otherwise, the variables refIdxLX and predFlagLX are specified by:

            - refIdxLX = −1		(Error: Reference source not found-3)

            - predFlagLX = 0		(Error: Reference source not found-4)

    - The variable mvdLX is derived as follows:

            - mvdLX[ 0 ] = MvdLX[ xPb ][ yPb ][ 0 ]	(Error: Reference source not found-5)

            - mvdLX[ 1 ] = MvdLX[ xPb ][ yPb ][ 1 ]	(Error: Reference source not found-6)

    - When predFlagLX is equal to 1, the variable mvpLX is derived as follows.

      - If dbbp_flag[ xPb ][ yPb ] is equal to 1, the following applies:

            - mvpLX[ 0 ] = 0		(Error: Reference source not found-xxx)

            - mvpLX[ 1 ] = 0		(Error: Reference source not found-xxx)

      - Otherwise (dbbp_flag[ xPb ][ yPb ] is equal to 0), the derivation process for luma motion vector prediction in subclause 8.5.3.2.5 is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the variables nPbW, nPbH, refIdxLX, and the partition index partIdx as inputs, and the output being mvpLX.

    - When predFlagLX is equal to 1, the luma motion vector mvLX is derived as follows:

            - uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 216 ) % 216	(Error: Reference source not found-7)

            - mvLX[ 0 ] = ( uLX[ 0 ]  >=  215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(Error: Reference source not found-8)

            - uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 216 ) % 216	(Error: Reference source not found-9)

            - mvLX[ 1 ] = ( uLX[ 1 ]  >=  215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(Error: Reference source not found-10)

NOTE – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1, inclusive.

When ChromaArrayType is not equal to 0 and predFlagLX, with X being 0 or 1, is equal to 1, the derivation process for chroma motion vectors in subclause 8.5.3.2.9 is invoked with mvLX as input, and the output being mvCLX.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xPb.. ( xPb + nPbW − 1 ), y = yPb..( yPb + nPbH− 1 ) (with X being either 0 or 1):

  - IvpMvFlag[ x ][ y ] = ivpMvFlag		(Error: Reference source not found-11)
VspModeFlag[ x ][ y ] = vspModeFlag		(Error: Reference source not found-12)
DispDerivedDepthFlag[ x ][ y ]  = dispDerivedDepthFlag	(Error: Reference source not found-13)
DispDerivedDepthVal[ x ][ y ] = dispDerivedDepthVal		(Error: Reference source not found-14)

- I.8.5.3.3.9 Decoding process for depth based block partition wise inter sample prediction

…

The derivation process for a depth predicted contour pattern as specified in subclause  is invoked with the sampling interval sampInt equal to 1, the sample location ( xTb, yTb ) equal to ( xCb, yCb ), and the block size nTbS equal to nCbS as inputs, and the output is a binary partition pattern segMask[ x ][ y ], a reference depth sample array refSamples of size (nCbs)x(nCbs), and a threshold for the segmentation of refSamples threshVal.

…

- I.8.5.7	Derivation process for a modified partitioning mode and derived disparity motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block.

The derivation process for a depth predicted contour pattern as specified in subclause  is invoked with the sampling interval sampInt equal to 4, the sample location ( xTb, yTb ) equal to ( xCb, yCb ), and the block size nTbS equal to nCbS as inputs, and the output is a binary partition pattern wedgePattern[ x ][ y ], a reference depth sample array refSamples of size (nCbs)x(nCbs), and a threshold for the segmentation of refSamples threshVal.

For p in the range of 0 to 1, inclusive and i in the range of 0 to 1, inclusive, partSum[ p ][ i ] is set equal to 0.

for( y = 0; y < nCbS ; y += 4 )
		for( x = 0; x < nCbS ; x += 4 ) { 
			segFlag = wedgePattern[ x ][ y ]
			partSum[ 0 ][ ( x < ( nCbS  >>  1 ) ) ? segFlag : !segFlag ]++
			partSum[ 1 ][ ( y < ( nCbS  >>  1 ) ) ? segFlag : !segFlag  ]++	(Error: Reference source not found-15)
		}

The variable partFlag is derived as specified in the following:

partFlag = 0
	maxPartSum = 0
	for( p = 0; p < 2; p++ )
		for( i = 0; i < 2; i++ ) {
			if( partSum[ p ][ i ] > maxPartSum ) {	(Error: Reference source not found-16)
				maxPartSum = partSum[ p ][ i ]
				partFlag = p
			}
		}

The variables x0 and y0 are derived and the variable PartMode is modified depending on partFlag as specified in the following:

  - x0 = partFlag ? xCb : xCb + nCbS / 2

  - y0 = partFlag ? yCb + nCbS / 2 : yCb

  - PartMode = partFlag ? SIZE_2NxN : SIZE_Nx2N

The following applies when merge_flag[ xCb ][ yCb + nCbS / 2 ] is equal to 1:

  - MergeIdx[ x0 ][ y0 ] = merge_idx[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-17)

  - MergeFlag[ x0 ][ y0°] = merge_flag[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-18)

  - InterPredIdc[ x0 ][ y0°] = inter_pred_idc[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-19)

For X in the range of 0 to 1, inclusive, the following applies:

  - PuRefIdxLX[ x0 ][ y0°] = ref_idx_lX[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-20)

  - MvpLXFlag[ x0 ][ y0°] = mvp_lX_flag[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-21)

  - MvdLX[ x0 ][ y0°] = MvdLX[ xCb ][ yCb + nCbS/2 ]		(Error: Reference source not found-22)

- When merge_flag[ xCb ][ yCb ] or merge_flag[ xCb ][ yCb + nCbS / 2 ] is equal to 0, the following apply:

- The variables InterPredIdc, PuRefIdxLX and MvdLX are derived as specified in the following steps.

- The variables derIvRefList, derIvRefIdx and derRefViewIdx are derived as follows:

  - Set variable IvRefFound equal to 0.

  - For X in the range of 0 to ( slice_type  = =  P ) ? 0 : 1, inclusive, the following applies:

    - For i in the range of 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

      - if ivRefFound is equal to 0 and ViewIdx( RefPicListX[ i ] ) is not equal to ViewIdx and PicOrderCnt( RefPicListX[ i ] ) is equal to PicOrderCntVal, the following applies:

        - Set variable ivRefFound equal to 1.

        - Set variable derIvRefList equal to X.

        - Set variable derIvRefIdx equal to i.

        - Set variable derRefViewIdx equal to ViewIdx( RefPicListX[ i ] ).

- The variables depthVal[ 0 ] and depthVal[ 1 ] are derived as follows:

  - The variables depthSum[ i ], numSum[ i ], i = 0, 1, are derived as follows:

    - For i being equal to 0 or 1, set depthSum[ i ] equal to 0 and set numSum[ i ] equal to 0.

    - For x in 0, ( nCbs >> 1 ) and ( nCbs – 1 )

      - For y in 0, ( nCbs >> 1 ) and ( nCbs – 1 )

        - If x is not equal to ( nCbs >> 1 ) or y is not equal to ( nCbs >> 1 ), the following applies:

          - depthSum[ wedgePattern[ x ][ y ] ]  +=  refSamples[ x ][ y ] 	(Error: Reference source not found-xxx)

          - numSum[ wedgePattern[ x ][ y ] ]  ++ 	(Error: Reference source not found-xxx)

  - For i being equal to 0 or 1, the variable depthEst[ i ] is set equal to clip3( 0, 255, ( refSamples[ 0 ][ 0 ] > threshVal ) ? threshVal + ( 1 – 2 * i ) : threshVal – ( 1 – 2 * i ) ).

  - For i being equal to 0 or 1, the variable depthVal[ i ] is derived as follows:

        - depthVal[ i ] = ( numSum[ i ] > 0 ) ? depthSum[ i ] / numSum[ i ] : depthEst[ i ] 	(Error: Reference source not found-xxx)

- For i being equal to 0 or 1, variables derDv[ i ] is derived as follows:

        - derDv[ i ][ 0 ] = DepthToDisparityB[ derRefViewIdx ][ depthVal[ i ] ]	(Error: Reference source not found-xxx)

        - derDv[ i ][ 1 ] = 0		(Error: Reference source not found-xxx)

- The variables InterPredIdc, PuRefIdxLX and MvdLX are derived as follows:

  - Set variable Y equal to derIvRefList.

  - For i equal to 0 or 1, the following applies:

    - Set variable x equal to ( i  = =  0 ) ? xCb : x0, set variable y equal to ( i  = =  0 ) ? yCb : y0.

    - When MergeFlag[ x ][ y°] is equal to 0, the following applies:

        - InterPredIdc[ x ][ y°] = PRED_LY		(Error: Reference source not found-xxx)

        - PuRefIdxLY[ x ][ y°] = derIvRefIdx		(Error: Reference source not found-xxx)

        - MvdLY[ x ][ y°] = derDv[i]		(Error: Reference source not found-xxx)

- I.8.5.8	Derivation process for a depth predicted contour pattern

Inputs to this process are:

- a variable sampInt specifying the sampling interval,

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nTbS specifying the transform block size.

Output of this process is:

- binary partition pattern wedgePattern[ x ][ y ], with x, y =0..nTbS − 1.

- reference depth array refSamples[ x ][ y ], with x, y =0..nTbS – 1.

- a threshold value threshVal.

The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations ( xP, yP ) equal to ( xTb, yTb ), the disparity vector mvDisp equal to MvRefinedDisp[ xTb ][ yTb ], the view identifier refViewIdx equal to RefViewIdx[ xTb ][ yTb ], the variables nPbW and nPbH, equal to nTbS, and the variable partIdc equal to 1 as inputs, and the output is the array refSamples of size (nTbS)x(nTbS) and the flag horSplitFlag.

The derivation process for an inter-layer predicted contour pattern as specified in subclause  is invoked with the sampling interval sampInt, the block size nTbS and the reference sample array refSamples[ x ][ y ] as inputs, and the output is the binary partition pattern wedgePattern[ x ][ y ] and a threshold value threshVal.

- I.8.8.1	Derivation process for an inter-layer predicted contour pattern

Inputs to this process are:

- a variable sampInt specifying the sampling interval,

- a variable nTbS specifying the block size,

- a reference sample array refSamples[ x ][ y ], with x, y =0..nTbS − 1.

Output of this process is:

- the binary partition pattern contourPattern[ x ][ y ], with x, y =0..nTbS − 1.

- a threshold value threshVal.

The values of the binary partition pattern contourPattern[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable threshVal specifying a threshold for the segmentation of refSamples is derived as specified in the following:

    - The variable sumRefVals is set equal to 0.

    - For x = 0..nTbS − 1 and y = 0..nTbS − 1, when ( x % sampInt ) and ( y % sampInt ) are both equal to 0, the following applies:

        - sumRefVals  +=  refSamples[ x ][ y ]		(Error: Reference source not found-23)

    - The variable threshVal is set equal to ( sumRefVals  >>  ( 2 * log2( nTbS / sampInt ) ) )

  - The variable contourPattern[ x ][ y ] with x, y =0..nTbS − 1 specifying a binary partition pattern is derived as specified in the following:

    - For x = 0..nTbS − 1 and y = 0..nTbS − 1, the following applies:

        - contourPattern[ x ][ y ] = ( refSamples[ x ][ y ] > threshVal )	(Error: Reference source not found-24)