| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0055 |
| --- | --- |

| Title: | AHG4: Semantics clean-up for advanced residual prediction weighting factor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-HEVC, a weighting factor index is signaled to indicate whether advanced residual prediction (ARP) is enabled for one coding unit (CU), and if yes, what is the weight applied to the predicted residue. The weighting factor index iv_res_pred_weight_idx however has an issue of having unclear semantics. Even when it is not equal to 0, it might happen that ARP doesn’t apply to the current CU. Such a behavior is considered contradictive to the value of this syntax element of iv_res_pred_weight_idx. In this proposal, it is proposed that iv_res_pred_weight_idx has to be set to 0 (at the encoder side) if eventually no ARP applies to the current CU. From the decoder perspective, a simple constraint is added to clean-up the design and make the semantics of iv_res_pred_weight_idx more reasonable. In addition, the iv_res_pred_weight_idx is currently constrained to be 0 when NBDV doesn’t return a valid disparity vector. However, this constraint shouldn’t apply for inter-view ARP thus the constraint is proposed to be modified. The proposed fixes don’t introduce any coding performance change under common test conditions.

# Introduction

The current advanced residual prediction (ARP) design in 3D-HEVC has problems.

When iv_res_pred_weight_idx is unequal to 0, it can happen that after certain derivation processes, ARP is not enabled for the whole coding unit. This is contradictive to the semantics of iv_res_pred_weight_idx. More specifically, in the decoding process, for each reference picture list X, whether to invoke the ARP process is further controlled by the following equation in addition to the decoded ARP weighting factors:

resPredFlag = 	( iv_res_pred_weight_idx  !=  0 ) && RpRefPicAvailFlagLX &&

RefRpRefAvailFlagLX[ RefViewIdx[ xP ][ yP ] ]	(I 198)

wherein RefRpRefAvailFlagLX is derived by checking whether the motion vector in in reference view corresponds to a picture in the DPB and were used as reference pictures and target reference picture used in ARP is available, and RpRefPicAvailFlagLX denotes the availability of target ARP reference picture. Therefore, even the iv_res_pred_weight_idx being unequal to 0, but either RpRefPicAvailFlagLX or RefRpRefAvailFlagLX[ RefViewIdx[ xP ][ yP ] ] is equal to 0, ARP is disabled, i.e., the residual predictor generation process is not invoked which makes the hardware implementation unfriendly.

In addition, in current design, a constraint is added that when the NBDV is unavailable, ARP shall be disabled. This constraint is meaningful for temporal ARP since it requires the NBDV result to identify reference blocks in a reference view. However, for inter-view ARP which doesn’t rely on the NBDV result, such a constraint might cause a performance drop.

- Proposal

It is proposed that the semantics and decoding process related to iv_res_pred_weight_idx should be changed in a way that if iv_res_pred_weight_idx is unequal to 0, the current block has ARP enabled (at least for one prediction direction).

In addition, when NBDV is unavailable, as long as any available prediction direction corresponds to inter-view ARP therefore no temporal ARP applies, iv_res_pred_weight_idx can be non-zero. In other words, when NBDV is unavailable, only when any available motion vector is a temporal motion vector, iv_res_pred_weight_idx shall be 0.

Therefore one constraint (highlighted in green) needs to be added into the working draft text. Meanwhile another constraint needs to be released (highlighted in yellow).

- I.7.4.9.5.2	Coding unit extension semantics

The variable rpEnableFlag is derived as specified in the following:

  - rpEnableFlag = iv_res_pred_flag[ nuh_layer_id ]  &&  RpRefPicAvailFlag 
	( CuPredMode[ x0 ][ y0 ]  !=  MODE_INTRA )  &&  ( PartMode  = =  PART_2Nx2N )	(Error: Reference source not found-1)

iv_res_pred_weight_idx specifies the index of the weighting factor used for residual prediction. iv_res_pred_weight_idx equal to 0 specifies that residual prediction is not used for the current coding unit. iv_res_pred_weight_idx not equal to 0 specifies that residual prediction is used for the current coding unit. When not present, the value of iv_res_pred_weight_idx is inferred to be equal to 0.

- The variable icEnableFlag is set equal to 0 and when slice_ic_enable_flag is equal to 1 and PartMode is equal to 2Nx2N and CuPredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

    - icEnableFlag = ( merge_idx[ x0 ][ y0 ] != 0 )  | |  !slice_ic_disable_merge_zero_idx_flag 	(Error: Reference source not found-2)

- Otherwise (merge_flag[ x0 ][ y0 ] is equal to 0), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to ViewIdx( RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ] ).

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = 	
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L0  &&  refViewIdxL1  !=  ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L1  &&  refViewIdxL0  !=  ViewIdx)	(Error: Reference source not found-3)

ic_flag equal to 1 specifies illumination compensation is used for the current coding unit. ic_flag equal to 0 specifies illumination compensation is not used for the current coding unit. When not present, ic_flag is inferred to be equal to 0.

When DispAvailabilityIdc[ x0 ][ y0 ] is not equal to DISP_AVAILABLE and ((inter_pred_idc[ x0 ][ y0 ] = = Pred_L0 && refViewIdxL0 = = ViewIdx)  ||  (inter_pred_idc[ x0 ][ y0 ] = = Pred_L1 && refViewIdxL1 = = ViewIdx) || (inter_pred_idc[ x0 ][ y0 ] = = Pred_BI && refViewIdxL0 = = ViewIdx && refViewIdxL1 = =  ViewIdx)) is equal to 1, iv_res_pred_weight_idx shall be equal to 0.

iv_res_pred_weight_idx shall be equal to 0 when both of the following conditions are true:

- (predFlagL0[ x0 ][ y0 ] && RpRefPicAvailFlagL0 && RefRpRefAvailFlagL0[ RefViewIdx[ x0 ][ y0 ] ]) is equal to 0.

- (predFlagL1[ x0 ][ y0 ] && RpRefPicAvailFlagL1 && RefRpRefAvailFlagL1[ RefViewIdx[ x0 ][ y0 ]]) is equal to 0.

In above conditions, RefViewIdx[ x0 ][ y0 ] indicates the disparity vector derived in I.8.5.5 for temporal prediction and the reference view index of the reference picture for inter-view prediction.

# Experimental results

The above constraint is realized at the decoder and further enforced in the encoder.

Proposed method is integrated in to 3D-HTM 11.0 software [1] and simulations are performed based on the common test conditions (CTC) [2].

The coding performance in CTC (anchor is HTM 11.0) is provided in Table 1. As reported in Table 1, the fix has negligible influence on coding performance.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux clusters. And these estimates may not be accurate due to parallel simulations.

Table 1: Coding results for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.01% | 0.00% | 0.00% | 0.01% |  |  | 0.01% | 97.8% | 98.8% | 103.5% |
| Kendo | -0.01% | 0.01% | 0.00% | -0.02% |  |  | 0.00% | 97.6% | 97.9% | 100.8% |
| Newspapercc | 0.00% | -0.01% | 0.00% | 0.00% |  |  | -0.01% | 88.7% | 91.6% | 93.3% |
| GhostTownFly | 0.00% | 0.00% | 0.00% | 0.00% |  |  | 0.00% | 81.9% | 88.3% | 88.5% |
| PoznanHall2 | 0.00% | -0.01% | -0.01% | -0.01% |  |  | -0.01% | 100.3% | 98.5% | 103.2% |
| PoznanStreet | 0.00% | 0.00% | 0.00% | 0.00% |  |  | 0.00% | 100.5% | 103.5% | 104.7% |
| UndoDancer | -0.02% | 0.02% | 0.00% | 0.00% |  |  | 0.01% | 84.0% | 85.6% | 87.8% |
| Shark | 0.01% | 0.00% | 0.00% | 0.00% |  |  | 0.00% | 79.5% | 89.4% | 89.1% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% |  |  | 0.00% | 94.7% | 96.1% | 99.2% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% |  |  | 0.00% | 89.2% | 93.0% | 94.6% |
| average | 0.00% | 0.00% | 0.00% |  |  | 0.00% | 0.00% | 91.3% | 94.2% | 96.3% |

# References

- K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, 11–17 Jan. 2014.

- 3DV-HTM version 11.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0/

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).The authors are not aware of Peking University having any current or pending patent rights relating to the technology described in this contribution.