| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0255r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0059r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On persistence of SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Many SEI messages are specified to persist until the end of the CVS (or sometimes until the next SEI message of the same type and applicable to the same layer, whichever is earlier in decoding order). Consequently, when SEI messages are applied to a non-base layer, they may persist across an IDR or BLA picture of that layer. It is suggested that the persistence of each SEI message applying separately for each target layer is specified not to span across IRAP pictures within the target layer and with NoRaslOutputFlag equal to 1. The contribution includes several related sub-proposals.

Revision 1 provides the specification text changes for the segmented rectangular frame packing arrangement SEI message and the mastering display colour volume SEI message, which is asserted to be similar to the changes for the other SEI messages.

Changes in revision 2:

- An asserted bug fix of how the target layers for the nested SEI messages are concluded from a scalable nesting SEI message.

- Text for the structure of pictures SEI message appeared in two locations in the specification, both in Annex D and in subclause F.14. The text was combined and checked for alignment with the phrasing used in this contribution.

- The requirement of having the inter-layer-constrained tile sets (ILCTS) SEI message present with the first picture of a CLVS (if any picture at all in the CLVS) was loosened so that it shall be present in the first picture of the CLVS if the picture is an IRAP picture. This change is asserted to make splicing easier, as the non-base layers of a spliced bitstream might not start with an IRAP picture and may contain ILCTS SEI messages later in the first CLVS of a non-base layer.

# Introduction

The current MV-HEVC/SHVC is asserted the have the following problems related to the persistence of SEI messages within the CVS:

- An SEI message is present for the picture firstPic having nuh_layer_id targetLayerId greater than 0, and an SEI message of the same type is present for another picture secondPic also having the nuh_layer_id equal to targetLayerId and later than firstPic in decoding order. Let us assume that layer down-switching takes place between firstPic and secondPic and the layer with targetLayerId is no longer sent. Let us assume that layer up-switching takes place subsequently, after secondPic in decoding order, but still within the same CVS and thereafter the layer with targetLayerId is sent again. The decoder would (incorrectly) consider the SEI message for the picture firstPic be in effect for the pictures of the targetLayerId after the up-switching.

- An SEI message is present for an independent non-base layer. Subsequent to the SEI message, in decoding order, the independent non-base layer contains a BLA picture (in a non-IRAP access unit). The SEI message continues to persist regardless of the presence of the BLA picture.

# Overview of the proposal

The proposal is summarized as follows, while the detailed proposal is presented with the change-marked text in the accompanying specification text document:

- A coded layer-wise video sequence (CLVS) is defined as follows:

coded layer-wise video sequence (CLVS): A sequence of coded pictures, with the same nuh_layer_id value nuhLayerId, that consists, in decoding order, of an IRAP picture with NoRaslOutputFlag equal to 1 or a picture with FirstPicInLayerDecodedFlag[ nuhLayerId ] equal to 0, followed by all coded pictures, if any, up to but excluding the next IRAP picture with NoRaslOutputFlag equal to 1 or the next picture with FirstPicInLayerDecodedFlag[ nuhLayerId ] equal to 0.

- In subclause D.3.1, the semantics of SEI messages are clarified to apply layer-wise to each layer specified as follows:

–	Unless otherwise specified in the semantics of the particular SEI message, the set of layer among which the semantics of the particular SEI message apply independently to each particular layer is derived as follows:

–	If the partcular SEI message is included in the scalable nesting SEI message and nesting_op_flag is equal to 0, the particular SEI message applies to each nuh_layer_id value in nestingLayerIdList[ 0 ].

–	Otherwise, if the particular SEI message is included in the scalable nesting SEI message and nesting_op_flag is equal to 0, the particular SEI message applies to each nuh_layer_id value in nestingLayerIdList[ i ] for each value of i in the range of 0 to nesting_num_ops_minus1, inclusive.

–	Otherwise, the particular SEI message applies to the nuh_layer_id value of the SEI NAL unit that includes the particular SEI message.

The above derivation of targetLayerId values and the other definitions (e.g. the current layer and the current access unit) given in subclause D.3.1 are clarified to apply to SEI messages in F.14, G.14 and H.14 too.

- The persistence of the SEI messages identified below is proposed to last until the end of the CLVS (of the current layer) rather than until the end of the CVS, as further detailed below:

- The persistence of the following SEI messages are changed to apply (at maximum) until the end the CLVS (of the current layer) rather than until the end of the CVS:

- The pan scan rectangle SEI message

- The scene information SEI message

- The progressive refinement segment start SEI message

- The film grain characteristics SEI message

- The tone mapping information SEI message

- The frame packing arrangement SEI message

- The display orientation SEI message

- The inter-layer constrained tile sets SEI message

- The alpha channel information SEI message

- The depth representation information SEI message

- The segmented rectangular frame packing arrangement SEI message

It is remarked that the scene information SEI message and the display orientation SEI message seem to be access unit specific and might not require semantics specified for picture sequences within a layer and persistence until the end of CLVS. However, it is asserted that the semantics of these SEI messages would require a greater amount of changes if they were made access unit specific.

- It is noted that the following SEI messages would probably also require the persistence to be specified until the end of the CLVS (of the current layer) rather than until the end of the CVS, but the text is not provided with this contribution due to lack of time:

- The overlay information SEI message

- The temporal motion-constrained tile sets SEI message

- The chroma resampling filter hint SEI message

- The knee function information SEI message

- The colour remapping information SEI message

- It is noted that the persistence of the mastering display colour volume SEI message is unclear. It is suggested that it should be allowed to use layer-wise and that the persistence should be specified to last until the end of the CLVS (of the current layer).

In revision 1, the persistence of the mastering display colour volume SEI message is proposed to be specified as follows:

The mastering display colour volume SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. When a previous mastering display colour volume SEI message already persists for the current picture, the mastering display colour volume SEI message for the current picture and the previous mastering display colour volume information SEI message shall have the same content.

- The inter-layer constrained tile sets SEI message is proposed to be modified as follows:

- In MV-HEVC/SHVC, terms identified tile set and motion-constrained tile set are used interchangeably to mean both a tile set in the current layer as well as the respective tile set in each reference layer. Consequently, it is asserted that the text is not very clear and easy to understand. In the proposed text, an identified tile set (within the current layer) and associated reference tile sets (within the reference layers) are defined and used consistently.

- In MV-HEVC/SHVC the identified tile set is associated with the respective tile sets in each reference layer through the use of the same tile set index in the inter-layer constrained tile sets SEI message. In the proposed text, the ilcts_id[ i ] value is used to associate tile sets of different layers with each other. The motivation for this proposed change is as follows:

- In multi-layer scalability (with more than two layers), it possible to have a different number of tile sets between different pairs of a predicted layer and its reference layer. The use of ilcts_id[ i ] provides more flexibility in associating tile sets between each pair of a predicted layer and its reference layer.

- ilcts_id[ i ] is available similarly to the tile set index, hence its use does not cause additional signalling.

- ilcts_id[ i ] value is inferred for each tile, when il_one_tile_per_tile_set_flag is equal to 1.

- It is asserted that the persistence of an alpha picture and the persistence of an alpha channel information SEI message are mixed in MV-HEVC/SHVC. Consequently, the following changes are proposed:

- It is proposed to specify the persistence an alpha picture as follows:

When an access unit contains an auxiliary picture picA with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_ALPHA, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

- It is proposed to specify the persistence of alpha channel information SEI message similarly to many other SEI messages, as follows:

Let currPic be the picture which the alpha channel information SEI message is associated with. The semantics of alpha channel information SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB with nuh_layer_id equal to targetLayerId in an access unit containing an alpha channel information SEI message applicable to nuh_layer_id equal to targetLayerId is output having PicOrderCnt( picB ) greater than PicOrderCnt( currPic ) and alpha_channel_cancel_flag in the alpha channel information SEI message is equal to 1, where PicOrderCnt( picB ) and PicOrderCnt( currPic ) are the PicOrderCntVal values of picB and currPic, respectively, immediately after the invocation of the decoding process for picture order count for picB.

- The temporal motion vector prediction constraints SEI message and the frame-field information SEI message are editorially revised to be more aligned with the phrasing of other SEI messages.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).