| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0274r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0064r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On active parameter sets SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution lists several asserted issues related to the active sequence parameter sets SEI message and its use in SPS activation. Consequently, the contribution proposes the following items:

- active_seq_parameter_set_id[ 0 ] in the active parameter sets SEI message is used for activating the SPS for the base layer (as in version 1 and RExt).

The SEI message extension mechanism to append the syntax of the active parameter sets SEI message to provide a mapping of SPS identifier values to layers. It is asserted that this mapping is needed to fulfill the intent of the SEI message in the multi-layer context and does not affect v1 or RExt encoder or decoder implementations.

- When vps_base_layer_internal_flag is equal to 0 or vps_base_layer_available_flag is equal to 0, the active parameter sets SEI message is not use to activate an SPS for the base layer and active_seq_parameter_set_id[ 0 ] in the active parameter sets SEI message does not specify the active SPS of the base layer.

Revision 2 of the contribution is asserted to contain the specification text document as reviewed and agreed by JCT-VC.

# Introduction

## Background: active parameter sets SEI message (in v1 and RExt)

The syntax of the active parameter sets SEI message is the following:

| active_parameter_sets( payloadSize ) { | Descriptor |
| --- | --- |
| active_video_parameter_set_id | u(4) |
| self_contained_cvs_flag | u(1) |
| no_parameter_set_update_flag | u(1) |
| num_sps_ids_minus1 | ue(v) |
| for( i = 0; i <= num_sps_ids_minus1; i++ ) |  |
| active_seq_parameter_set_id[ i ] | ue(v) |
| } |  |

The semantics of the last two syntax elements are the following:

num_sps_ids_minus1 plus 1 indicates and shall be equal to the number of SPSs that are referred to by the VCL NAL units of the access unit associated with the active parameter sets SEI message. In bitstreams conforming to this version of this Specification, num_sps_ids_minus1 shall be equal to 0. Although the value of num_sps_ids_minus1 is required to be equal to 0 in this version of this Specification, decoders shall allow other values of num_sps_ids_minus1 in the range of 0 to 15, inclusive, to appear in the syntax.

active_seq_parameter_set_id[ 0 ] indicates and shall be equal to the value of the sps_seq_parameter_set_id of the SPS that is referred to by the VCL NAL units of the access unit associated with the SEI message. The value of active_seq_parameter_set_id[ 0 ] shall be in the range of 0 to 15, inclusive. The syntax elements active_seq_parameter_set_id[ i ] for i greater than 0 are reserved for future use by ITU-T | ISO/IEC. Although the value of num_sps_ids_minus1 is required to be equal to 0 in bitstreams conforming to this version of this Specification, decoders shall allow values of active_seq_parameter_set_id[ i ] in the range of 0 to 15, inclusive, to appear in the syntax for i greater than 0, and shall ignore the presence and values of syntax elements active_seq_parameter_set_id[ i ] for i greater than 0.

## Asserted issues in MV-HEVC/SHVC

It is asserted that MV-HEVC/SHVC includes the following issues related to the active parameter sets SEI message:

- The active parameter sets SEI message does not indicate in which layer(s) a particular SPS identifier is active and hence cannot be used for layer-wise SPS activation.

The SPS activation is specified in subclause 7.4.2.4.2 as follows (with changes specified in MV-HEVC/SHVC indicated by turquoise highlights):

When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for a particular layer and it is referred to by activation of a PPS RBSP (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value) referred to by the particular layer or is referred to by an SEI NAL unit containing an active parameter sets SEI message (in which one of the active_seq_parameter_set_id[ i ] values is equal to the sps_seq_parameter_set_id value), it is activated for the particular layer.

The problem of the text above is that it specifies SPS activation for a particular layer, but the active sequence parameter sets SEI message does not provide information of the one or more nuh_layer_id values of the layers for which the SPS with sps_seq_parameter_set_id equal to active_seq_parameter_set_id[ i ] is active. Consequently, active parameter sets SEI message cannot be used to activate SPSs for non-base layers.

- When the base layer is external (vps_base_layer_internal_flag is equal to 0) or the base layer is not available (vps_base_layer_available_flag is equal to 0), no SPS for the base layer should be required to be present in the bitstream or provided by external means and no SPS for the base layer should be activated.

- The semantics of active_seq_parameter_set[ i ] have not been updated for MV-HEVC/SHVC.

The semantics of num_sps_ids_minus1 is updated in MV-HEVC/SHVC as follows:

num_sps_ids_minus1 plus 1 indicates and shall be equal to the number of SPSs that are referred to by the VCL NAL units of the access unit associated with the active parameter sets SEI message. The value of num_sps_ids_minus1 shall be in the range of 0 to 15, inclusive.

However, the semantics of active_seq_parameter_set[ i ] have not been updated in MV-HEVC/SHVC, which is suspected to be an editorial oversight. (The current semantics basically disallow the use of active_seq_parameter_set[ i ] with i greater than 0 and contradict with nun_sps_ids_minus1: "The syntax elements active_seq_parameter_set_id[ i ] for i greater than 0 are reserved for future use by ITU-T | ISO/IEC.")

- The bitstream may be subject to sub-bitstream extraction after which some layers are no longer present in the bitstream. Consequently, some of the active_seq_parameter_set[ i ] values might correspond to the layers that have been removed during a sub-bitstream extraction operation. It is therefore asserted that the semantics cannot indicate that active_seq_parameter_set[ i ] shall be a value that is referred to by a VCL NAL unit of the access unit associated with the SEI message.

# Proposal

## Overview

The proposal is summarized as follows:

- active_seq_parameter_set_id[ 0 ] in the active parameter sets SEI message is used for activating the SPS for the base layer (as in version 1 and RExt).

To enable activation of SPSs for particular non-base layers, an indicated mapping between active_seq_parameter_set_id[ i ] values and nuh_layer_id values is proposed to be added into the active parameter sets SEI message using the SEI message extension mechanism.

- When vps_base_layer_internal_flag is equal to 0 or vps_base_layer_available_flag is equal to 0, the active parameter sets SEI message is not use to activate an SPS for the base layer and active_seq_parameter_set_id[ 0 ] in the active parameter sets SEI message does not specify the active SPS of the base layer.

## Specification text

The change-marked text is provided in the accompanying document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).