| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0066 |
| --- | --- |

| Title: | CE2 related: Simplification of DMM4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle Detlev Marpe | Email: | philipp.merkle@hhi.fraunhofer.de, detlev.marpe@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes two modification of the DMM4 depth intra coding mode: simplification of threshold calculation as well as encoder optimization and simplification.

# Introduction

DMM4 derives a contour segmentation from the co-located texture luma block (CTLB). Based on the proposals to the last meeting, regarding simple threshold derivation for contour segmentation in DBBP as well as complexity and performance of DMM, we propose simplifications and optimization of DMM4.

# Proposed methods

## Simple 4-corner threshold calculation

Currently the threshold value for contour segmentation derivation is calculated as the mean value of all depth samples, requiring N²-1 additions and one shift operation for an NxN block.

Based on the proposal in JCT3V-H0058, the threshold calculation is simplified by using the mean of the four corner values as the threshold value for contour segmentation, requiring 3 additions and one shift, independent of the block size.

| block size | original method | proposed method |
| --- | --- | --- |
| 4x4 | 15 ADDs + 1 SHIFT | 3 ADDs + 1 SHIFT |
| 8x8 | 63 ADDs + 1 SHIFT |  |
| 16x16 | 255 ADDs + 1 SHIFT |  |
| 32x32 | 1023 ADDs + 1 SHIFT |  |

Specification text:

- I.8.4.4.4	  Derivation process for a texture predicted contour pattern

- …

The values of wedgePattern[ x ][ y ] are derived as specified by the following ordered steps:

  - The variable threshVal specifying a threshold for the segmentation of refSamples is derived as specified in the following:

      - thresVal  =  ( refSamples[ 0 ][ 0 ] + refSamples[ nTbS − 1 ][ 0 ] + refSamples[ 0 ][ nTbS − 1 ] + refSamples[ nTbS − 1 ][ nTbS − 1 ] )  >>  2		(Error: Reference source not found-xxx)

  - The variable wedgeletPattern[ x ][ y ] with x, y =0..nTbS − 1 specifying a binary partition pattern is derived as specified in the following:

    - For x = 0..nTbS − 1 and y = 0..nTbS − 1, the following applies:

        - wedgeletPattern[ x ][ y ] = ( refSamples[ x ][ y ] > threshVal )	(Error: Reference source not found-xxx)

## Encoder optimization

Currently, intra modes in inter slices are only tested under certain conditions in order to limit the encoder complexity. It is proposed that this encoder restriction is removed for DMM4 mode, i.e. always testing it in inter slices.

Moreover, encoder complexity is reduced by removing the additional test of DMM modes with deltaDCs in case both deltaDC values are estimated as being equal to zero.

# Experimental results

According to the CE2 description in JCT3V-H1102 [1] and the common test conditions in JCT3V-G1100 [2], the proposed method is evaluated for random access (CTC) and all-intra configuration with HTM-11.0.

Table 1. Result for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.01% | 104.0% | 98.8% | 97.9% |
| Kendo | 0.0% | 0.1% | -0.13% | 104.0% | 98.6% | 98.7% |
| Newspapercc | 0.0% | 0.1% | -0.08% | 105.4% | 100.6% | 99.5% |
| GhostTownFly | 0.0% | 0.2% | 0.08% | 105.2% | 100.1% | 98.6% |
| PoznanHall2 | -0.1% | 0.0% | 0.01% | 104.5% | 98.1% | 100.2% |
| PoznanStreet | 0.0% | 0.0% | 0.02% | 105.2% | 99.9% | 98.9% |
| UndoDancer | -0.1% | 0.0% | -0.26% | 105.7% | 101.5% | 100.1% |
| Shark | -0.1% | 0.1% | -0.33% | 104.2% | 98.7% | 98.7% |
| 1024x768 | 0.0% | 0.1% | -0.07% | 104.5% | 99.3% | 98.7% |
| 1920x1088 | 0.0% | 0.1% | -0.10% | 104.9% | 99.6% | 99.3% |
| average | 0.0% | 0.1% | -0.09% | 104.8% | 99.5% | 99.1% |

Table 2. Result for 3-view scenario under all-intra configuration

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.02% | 98.3% | 98.6% | 100.2% |
| Kendo | 0.0% | 0.0% | -0.04% | 98.9% | 102.7% | 100.9% |
| Newspapercc | 0.0% | 0.0% | -0.03% | 99.2% | 99.1% | 99.7% |
| GhostTownFly | 0.0% | 0.0% | -0.07% | 98.4% | 96.4% | 99.7% |
| PoznanHall2 | 0.0% | 0.0% | -0.05% | 99.4% | 100.2% | 100.7% |
| PoznanStreet | 0.0% | 0.0% | -0.03% | 98.8% | 100.4% | 100.1% |
| UndoDancer | 0.0% | 0.0% | -0.02% | 99.0% | 99.9% | 100.0% |
| Shark | 0.0% | -0.1% | -0.28% | 97.4% | 102.1% | 100.1% |
| 1024x768 | 0.0% | 0.0% | -0.03% | 98.8% | 100.1% | 100.3% |
| 1920x1088 | 0.0% | 0.0% | -0.09% | 98.6% | 99.8% | 100.1% |
| average | 0.0% | 0.0% | -0.07% | 98.7% | 99.9% | 100.2% |

The results for individual modifications and the overall performance are summarized in table 3.

Table 3. Results overview

|  | synth PSNR / total bitrate |  |
| --- | --- | --- |
|  | CTC | all-intra |
| simple threshold | 0.02% | -0.07% |
| enc. intra in inter | -0.07% | 0.00% |
| enc. deltaDC test | 0.00% | 0.01% |
| JCT3V-I0066 | -0.09% | -0.07% |

# Cross check

The cross check of the proposed method was done by Aachen University.

# Option: extension to DBBP

As the simplification of contour threshold calculation was originally proposed in JCT3V-H0058 for DBBP, this method can be identically applied to both DMM4 and DBBP, such that the specification text of the common process I.8.8.1 is simplified, including removal of the sampling factor.

Specification text:

- I.8.8.1  Derivation process for an inter-layer predicted contour pattern

Inputs to this process are:

- a variable nTbS specifying the block size,

- a reference sample array refSamples[ x ][ y ], with x, y =0..nTbS − 1.

Output of this process is:

- the binary partition pattern contourPattern[ x ][ y ], with x, y =0..nTbS − 1.

The values of the binary partition pattern contourPattern[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable threshVal specifying a threshold for the segmentation of refSamples is derived as specified in the following:

        - -

      - thresVal  =  ( refSamples[ 0 ][ 0 ] + refSamples[ nTbS − 1 ][ 0 ] + refSamples[ 0 ][ nTbS − 1 ] + refSamples[ nTbS − 1 ][ nTbS − 1 ] )  >>  2		(Error: Reference source not found-2

      - The variable contourPattern[ x ][ y ] with x, y =0..nTbS − 1 specifying a binary partition pattern is derived as specified in the following:

    - For x = 0..nTbS − 1 and y = 0..nTbS − 1, the following applies:

        - contourPattern[ x ][ y ] = ( refSamples[ x ][ y ] > threshVal )	(Error: Reference source not found-3)

Results:

Table 4 shows the results for the extension to DBBP under CTC (all-intra results identical to Table 2).

Table 4. Result for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | -0.01% | 101.6% | 101.9% | 102.7% |
| Kendo | 0.0% | 0.1% | -0.03% | 100.9% | 99.7% | 102.3% |
| Newspapercc | 0.0% | 0.1% | 0.00% | 102.8% | 102.9% | 103.2% |
| GhostTownFly | 0.0% | 0.2% | 0.06% | 103.9% | 101.9% | 99.5% |
| PoznanHall2 | 0.0% | 0.0% | 0.01% | 102.8% | 98.3% | 99.3% |
| PoznanStreet | 0.0% | 0.1% | 0.04% | 103.9% | 101.5% | 99.6% |
| UndoDancer | -0.1% | 0.0% | -0.25% | 104.7% | 100.6% | 99.9% |
| Shark | 0.0% | 0.1% | -0.33% | 103.8% | 99.1% | 99.0% |
| 1024x768 | 0.0% | 0.1% | -0.01% | 101.7% | 101.5% | 102.7% |
| 1920x1088 | 0.0% | 0.1% | -0.09% | 103.8% | 100.3% | 99.5% |
| average | 0.0% | 0.1% | -0.06% | 103.0% | 100.7% | 100.7% |

# Conclusion

The results show that the proposed simplification and optimization of DMM4 leads to a consistent coding gain with a reduction of decoder complexity and is therefore recommended for adoption to 3D-HEVC.

The extension of simple contour threshold calculation to DBBP leads to losses for Kendo and Newspaper such that the total gain is somewhat smaller, but also reduces the encoder complexity and simplifies the specification in a unified way with a common process for DMM4 and DPPB.

# References

- Y. Chen, Y.-W. Chen, “Description of Core Experiment 2 (CE2) on depth intra coding,” JCT3V-H1102, Valencia, Spain, April 2014.

- K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, San Jose, USA, January 2014.

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).