| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0069 |
| --- | --- |

| Title: | AHG7: Complexity Assessment on Depth Intra Modes with Simplified Data Flows |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chun-Fu Chen Chun-Hsi Huang Gwo Giun Lee No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University/Industrial Technology Research Institute |  |  |

_____________________________

# Abstract

This proposal provides a case study of complexity assessment on depth intra modes in 3D-HEVC. The complexity metrics, including number of operations, data storage requirement, and data transfer rate are assessed for evaluating depth intra modes. With respect to the previous proposal, this proposal use another data flow which simplified segmental depth coding through the segment-based processing; moreover, the complexities of de-quantization and inverse transform are also included when segment-wise DC coding (SDC) is disabled. Therefore, the analysis results might be considered as anchor complexity to assist JCT3V in evaluating the proposals coming from several proponents in a fair manner. This proposal also presents the experimental results with various configurations, such as enabling/disabling depth modeling modes (DMM), SDC, etc. When SDC, DMM, and DLT are all disabled, the results reveal that the coding loss is 0.0%, 1.8%, and 10.3% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively in all intra coding condition; on the other hand, in CTC coding condition, the results reveal that the coding loss is 0.1%, 0.7%, and 3.6% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively. Consequently, SDC not only reduces the complexity in depth coding but also increases the coding performance of 3D-HEVC since it provides a reasonable approach to handle depth data.

# Introduction and Problem Statement

The complexity issue has been studied for several decades; however, these studies may analyze the complexity based on the specific platforms. Thus, the assessed complexity would be the platform-dependent and would be varied on different platforms since the analyzed results cannot be transparent to another platform. The proposals in previous JCT3V meetings [1]-[9] provide a guideline to evaluate the intrinsic complexity of an algorithm. Thus, this proposal follows the analysis method to extract the intrinsic algorithmic complexity that is platform-independent and transparent to platforms. The coding tools, depth intra mode, including depth modeling mode 1 (DMM 1), DMM 4m and angular mode, would be considered as an exercise to assess its complexity in this proposal. On the other hand, the issue of wedgelet pattern would also be discussed in this proposal.

# Method of Complexity Assessment

To assess the complexity, the flowchart for completing one data granularity should be established first to reveal data causality. One important issue should be concerned during the flowchart construction; the major processing or key modules should be identified in this stage in order to achieve the further analysis work. However, the major processing identification would be affected by the analyzer; thus, this proposal also provides a general concept to construct the flowchart. Subsequently, the complexity metrics, such as data storage requirement, and data transfer rate, can be assessed in accordance with the constructed flowchart and the identified major processes or key modules at various data granularities. The data granularity means that amount of data should be performed or produced within one process.

- Number of operations

To estimate the number of operations of an algorithm, the starting point would be mathematical expressions due to the fact that mathematical expressions are more intrinsic to algorithms and mathematical expressions would not be affected by platform. When encountering some if-else conditions, the worst case of conditions would be considered. Some basic operation types are considered only, such as addition, multiplication, and division, due to the fact that some equivalent operations would be treated as identical complexity. E.g., the subtraction can be easily implementation by the addition with 2’s complement representation plus a NOT logic and an incermentor and the shift operations can be implemented with the wire reconnection.

LUT access is widely used in depth intra coding; thus, here this proposal assumes that one LUT access with few data, e.g., just two elements in storage, would be considered as 1-bit comparison in the following complexity assessment.

- Data storage requirement

The data storage requirement was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and generated output data should be stored to complete one data granularity and some intermediate data would be stored according to the algorithm. Since the assessed results would like to be transparent to different platforms, the data storage configuration shall not be considered at the initial stage.

- Data transfer rate

The data transfer rate was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and produced output data should be transferred during the process of one data granularity. The intermediate data would not be taken into consideration since these data would be disappeared after finishing the process. In addition, transfer rate of input data and output data shall be discussed individually since the characteristic of sending data and fetching data are different.

In this proposal, the flowchart is constructed according to the specification [10] and the test model [11] and the major process is defined according to that is there any data to be processed or not. The data granularities studied in this proposal is one 6464 block with all supported PU sizes. (When SDC is enabled, DMM could support 6464 block since one 6464 block will be split into four 3232 blocks.)

# Complexity Assessment

In the subsection of number of operations, two kinds of assessment are provided. One is assessed according to the spec text, and the other one is derived according to the simplified expressions. (The simplifications are marked by the symbol of strikethrough.) The major different is that in the spec, pixel-wise operations are presented; however, when SDC is enabled, it is more reasonable to use segment-wise procedure instead of computing pixel by pixel. Hence, a dramatic difference on number of operations would be found at the following subsections when comparing the simplified data flow to the original one. Furthermore, the complexity of de-quantization and inverse transform at various TU size are also illustrated in the appendix section of this proposal.

## DMM 1

### Algorithmic description

According to the specification text and the test model, the flowchart of DMM 1 is shown in Fig. 1. The major processing of DMM 1 is the processes containing derivation and assignment of depth partition value and segmental depth intra coding.

Fig. 1 Flowchart of DMM 1

According to flowchart, the complexity of DMM 1 is assessed at various data granularities and the details are described in the following sections.

### Number of operations (based on spec)

- Derive and assign depth partition value: (I.8.4.4.2.9):
DLT ON: 8+N2 comparison (1-bit), 1 comparison (pixel), 5 additions/subtractions, 4 LUT access
DLT OFF: 8+N2 comparison (1-bit), 1 comparison (pixel), 5 additions/subtractions, 0 LUT access

vertEdgeFlag = (partitionPattern[0][0] != partitionPattern[N-1][0])? 1 : 0;
horEdgeFlag = (partitionPattern[0][0] != partitionPattern[0][N-1])? 1 : 0

If (vertEdgeFlag == horEdgeFlag)

dcValLT = (neiSamples[-1][0] + neiSamples[0][-1]) >> 1;

If (horEdgeFlag == 0), and neiSamples[-1][2N-1] and neiSamples[2N-1][-1] are both available,

dcValBR = abs(neiSamples[-1][2N-1] - neiSamples[-1][0]) > 
abs(neiSamples[2N-1][-1] - neiSamples[0][-1]) ? 
neiSamples[-1][2N-1]: neiSamples[2N-1][-1]

Otherwise (horEdgeFlag != 0 or neiSamples[-1][2N-1] or neiSamples[2N-1][-1] is unavailable)

dcValBR = 2BitDepth-1

Otherwise (vertEdgeFlag!= horEdgeFlag)

dcValLT = horEdgeFlag ? neiSamples[(N-1)>>1][-1]: neiSamples[-1][(N-1)>>1]
dcValBR = horEdgeFlag ? neiSamples[-1][N-1]: neiSamples[N-1][-1]

If(DltFlag == 1)

dcOffset[0] = dcOffsetAvailFlag ? DcOffset[xTb][yTb][0] : 0
  dcOffset[1] = dcOffsetAvailFlag ? DcOffset[xTb][yTb][1] : 0
  dcVal[0] = Idx2DepthValue[DepthValue2Idx[dcValLT]+dcOffset[0]]
  dcVal[1] = Idx2DepthValue[DepthValue2Idx[dcValBR]+dcOffset[0]]

Otherwise (DltFlag == 0)

dcOffset[0] =dcOffsetAvailFlag ? DcOffset[xTb][yTb][0] : 0
  dcOffset[1] =dcOffsetAvailFlag ? DcOffset[xTb][yTb][1] : 0
  dcVal[0] = dcValLT + dcOffset[0]
  dcVal[1] = dcValBR + dcOffset[1]

For 0<= x, y < N:

predSamples[x][y] = dcVal[wedgePattern[x][y]]

where neiSamples are left and above reference samples, xTb and yTb are the top-left sample position of current block, and N is PU size.

- Segmental depth intra coding is ON: (I.8.4.4.3): 
DLT ON: 1+2N2 comparison (1-bit), N2 additions, 2N2 LUT access
DLT OFF: 1+N2 comparison (1-bit), N2 additions, 0 LUT access

dcPred[0] = dcVal[0]
dcPred[1] = dcVal[1]

If (DltFlag == 1)

For 0<= x, y < N:
dltIdxPred = DepthValue2Idx[dcPred[wedgePattern[x][y]]]
dltIdxResi = DcOffset[xTb][yTb][wedgePattern[x][y]]
Recblock[x][y] = Idx2DepthValue[dltIdxPred + dltIdxResi]

Otherwise (DltFlag == 0)

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + DcOffset[xTb][yTb][wedgePattern[x][y]]

, where xTb, yTb are the top-left sample position of current block and Recblock is reconstructed block.

- Segmental depth intra coding is OFF: 
DLT ON and OFF: De-quantization and inverse transform are applied. The corresponding complexity is illustrated in appendix.

### Number of operations (simplified, segment-wise processing)

- Depth partition value derivation and assignment process (I.8.4.4.2.9): 
DLT ON: 8 comparison (1-bit), 1 comparison (pixel), 5 additions/subtractions, 4 LUT access
DLT OFF: 8 comparison (1-bit), 1 comparison (pixel), 5 additions/subtractions, 0 LUT access

vertEdgeFlag = (partitionPattern[0][0] != partitionPattern[N-1][0])? 1 : 0;
horEdgeFlag = (partitionPattern[0][0] != partitionPattern[0][N-1])? 1 : 0

If (vertEdgeFlag == horEdgeFlag)

dcValLT = (neiSamples[-1][0] + neiSamples[0][-1]) >> 1;

If (horEdgeFlag == 0), and neiSamples[-1][2N-1] and neiSamples[2N-1][-1] are both available,

dcValBR = abs(neiSamples[-1][2N-1] - neiSamples[-1][0]) > 
abs(neiSamples[2N-1][-1] - neiSamples[0][-1]) ? 
neiSamples[-1][2N-1]: neiSamples[2N-1][-1]

Otherwise (horEdgeFlag != 0 or neiSamples[-1][2N-1] or neiSamples[2N-1][-1] is unavailable)

dcValBR = 2BitDepth-1

Otherwise (vertEdgeFlag!= horEdgeFlag)

dcValLT = horEdgeFlag ? neiSamples[(N-1)>>1][-1]: neiSamples[-1][(N-1)>>1]
dcValBR = horEdgeFlag ? neiSamples[-1][N-1]: neiSamples[N-1][-1]

If(DltFlag == 1)

dcOffset[0] = dcOffsetAvailFlag ? DcOffset[xTb][yTb][0] : 0
  dcOffset[1] = dcOffsetAvailFlag ? DcOffset[xTb][yTb][1] : 0
  dcVal[0] = Idx2DepthValue[DepthValue2Idx[dcValLT]+dcOffset[0]]
  dcVal[1] = Idx2DepthValue[DepthValue2Idx[dcValBR]+dcOffset[0]]

Otherwise (DltFlag == 0)

dcOffset[0] =dcOffsetAvailFlag ? DcOffset[xTb][yTb][0] : 0
  dcOffset[1] =dcOffsetAvailFlag ? DcOffset[xTb][yTb][1] : 0
  dcVal[0] = dcValLT + dcOffset[0]
  dcVal[1] = dcValBR + dcOffset[1]

For 0<= x, y < N:
  predSamples[x][y] = dcVal[wedgePattern[x][y]]

, where xTb, yTb are the top-left sample position of current block.

- Segmental depth intra coding is ON (I.8.4.4.3): 
DLT ON: 5+N2 comparison (1-bit), 2 additions, 4 LUT access
DLT OFF: 5+N2 comparison (1-bit), 2 additions, 0 LUT access

dcPred[0] = dcVal[0]
dcPred[1] = dcVal[1]

If (DltFlag == 1)

For 0<= x, y < N:
dltIdxPred = DepthValue2Idx[dcPred[wedgePattern[x][y]]]
dltIdxResi = DcOffset[xTb][yTb][wedgePattern[x][y]]
Recblock[x][y] = Idx2DepthValue[dltIdxPred + dltIdxResi]

Otherwise (DltFlag == 0)

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + DcOffset[xTb][yTb][wedgePattern[x][y]]

If (DltFlag == 1)

RecPixel[0] = Idx2DepthValue[DepthValue2Idx[dcPred[0]] + DcOffset[xTb][yTb][0]]
RecPixel[1] = Idx2DepthValue[DepthValue2Idx[dcPred[1]] + DcOffset[xTb][yTb][1]]

Otherwise (DltFlag == 0)

RecPixel[0] = dcPred[0] + DcOffset[xTb][yTb][0] 
RecPixel[1] = dcPred[1] + DcOffset[xTb][yTb][0]

For 0<= x, y < N:
Recblock[x][y] = RecPixel[wedgePattern[x][y]]

- Segmental depth intra coding is OFF: 
DLT ON and OFF: De-quantization and inverse transform are applied. The corresponding complexity is illustrated in appendix.

For 0<= x, y < N:
  predSamples[x][y] = dcVal[wedgePattern[x][y]] (Assign the value to prediction block for inverse quantization if segmental depth intra coding is OFF since we skip this procedure at “Depth partition value derivation and assignment process”.)

The number of operations of DMM 1 with different configurations for one PU is recorded in the algebraic way in Table 1 and Table 2, and the attached spreadsheet illustrated the numeric results in detail.

Table 1 Number of operations of one NN PU (SDC ON + DLT ON)

|  | Original, based on spec |  |  |  | Simplified, segment-wise processing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processes | Comp. (1-bit) | Comp. (pixel) | Add/Sub | LUT access | Comp. (1-bit) | Comp. (pixel) | Add/Sub | LUT access |
| Derive and assign depth partition value | N2+8 | 1 | 5 | 4 | 8 | 1 | 5 | 4 |
| Segmental depth intra coding | 2N2+1 | 0 | N2 | 2N2 | N2+5 | 0 | 2 | 4 |
| Total | 3N2+9 | 1 | N2+5 | 2N2+4 | N2+13 | 1 | 7 | 8 |

Table 2 Number of operations of one NN PU (SDC ON + DLT OFF)

|  | Original, based on spec |  |  |  | Simplified, segment-wise processing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processes | Comp. (1-bit) | Comp. (pixel) | Add/Sub | LUT access | Comp. (1-bit) | Comp. (pixel) | Add/Sub | LUT access |
| Derive and assign depth partition value | N2+8 | 1 | 5 | 0 | 8 | 1 | 5 | 0 |
| Segmental depth intra coding | 2N2+1 | 0 | N2 | 0 | N2+5 | 0 | 2 | 0 |
| Total | 3N2+9 | 1 | N2+5 | 0 | N2+13 | 1 | 7 | 0 |

### Storage requirement

Input buffer of DMM 1 should store wedgelet pattern, wedgelet pattern index, above and left reference samples, dcOffset and output buffer stores the reconstructed block. On the other hand, during the processing of DMM 1, some intermediate data should be stored, such as predicted DC (dcValXX and dcPred). In the analysis, N denotes the PU size. Consequently, the algebraic expression of storage requirement according PU size is displayed in Table 3, and the attached spreadsheet show the numeric results of different configurations in detail. (The result in table is the case of that SDC and DLT both are enabled.)

Table 3 Algebraic expressions of storage requirement (N is PU size)

| Data | Number of bits |
| --- | --- |
| Wedgelet pattern | N×N×1 |
| Wedgelet pattern index | 11 |
| Above and left reference samples | 8×8 |
| Predicted DC | 2×8 |
| dcOffset | 2×2×8 |
| Reconstructed block | N×N×8 |

### Data transfer rate

Input data and output transferred between DMM 1 and other modules would be taken into consideration. Hence, wedgelet pattern, wedgelet pattern index, above and left reference samples, dcOffset, and reconstructed block are calculated. Consequently, the algebraic expression of data transfer rate according PU size is displayed in Table 4, and the attached spreadsheet show the numeric results of different configurations in detail. (The result in table is the case of that SDC and DLT both are enabled.)

Table 4 Algebraic expressions of data transfer rate (N is PU size)

| Data | Bits/PU |
| --- | --- |
| Wedgelet pattern (Input) | N×N×1 |
| Wedgelet pattern index (Input) | 11 |
| Above and left reference samples (Input) | 8×8 |
| dcOffset (Input) | 2×2×8 |
| Reconstructed block (Output) | N×N×8 |

## DMM 4

### Algorithmic description

According to the specification text and the test model, the flowchart of DMM 4 is shown in Fig. 2. The major processing of DMM 4 is the processes including contour pattern generation, derivation and assignment of depth partition value and segmental depth intra coding. According to flowchart, the complexity of DMM 4 is assessed at various data granularities and the details are described in the following sections.

Fig. 2 Flowchart of DMM 4

### Number of operations

Due to the fact that the part of deriving and assigning depth partition value and segmental depth intra coding of DMM 4 are identical to DMM 1, this section would only display the process which derives contour pattern via reconstructed Luma picture. However, the assessment result includes all procedures of DMM 4.

- Contour pattern generation: N2 comparisons (pixel), N2 additions/subtractions

wedgePatterm[x][y] = ( refSamples[x][y] > ThresVal ), for 0<= x, y < N

where refSamples is the co-located reconstructed Luma picture, and N is block size.

The number of operations of DMM 4 with different configurations for one PU is recorded in the algebraic way in Table 5 and Table 6, and the attached spreadsheet records the numeric results of all configurations in detail.

Table 5 Number of operations of one NN PU (SDC ON + DLT ON)

|  | Original, based on spec |  |  |  | Simplified, segment-wise processing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processes | Comp. (1-bit) | Comp. (pixel) | Add/Sub | LUT access | Comp. (1-bit) | Comp. (pixel) | Add/Sub | LUT access |
| Contour pattern generation | 0 | N2 | N2 | 0 | 0 | N2 | N2 | 0 |
| Derive and assign depth partition value | N2+8 | 1 | 5 | 4 | 8 | 1 | 5 | 4 |
| Segmental depth intra coding | 2N2+1 | 0 | N2 | 2N2 | N2+5 | 0 | 2 | 4 |
| Total | 3N2+9 | N2+1 | 2N2+5 | 2N2+4 | N2+13 | N2+1 | N2+7 | 8 |

Table 6 Number of operations of one NN PU (SDC ON + DLT OFF)

|  | Original, based on spec |  |  |  | Simplified, segment-wise processing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processes | Comp. (1-bit) | Comp. (pixel) | Add/Sub | LUT access | Comp. (1-bit) | Comp. (pixel) | Add/Sub | LUT access |
| Contour pattern generation | 0 | N2 | N2 | 0 | 0 | N2 | N2 | 0 |
| Derive and assign depth partition value | N2+8 | 1 | 5 | 0 | 8 | 1 | 5 | 0 |
| Segmental depth intra coding | 2N2+1 | 0 | N2 | 0 | N2+5 | 0 | 2 | 0 |
| Total | 3N2+9 | N2+1 | 2N2+5 | 0 | N2+13 | N2+1 | N2+7 | 0 |

### Storage requirement

Input buffer of DMM 4 should store contour pattern, above and left reference samples, co-located Luma samples, dcOffset and output buffer stores the reconstructed block. On the other hand, during the processing of DMM 4, some intermediate data should be stored, such as predicted DC (dcValXX and dcPred). In the analysis, N denotes the PU size. Consequently, the algebraic expression of storage requirement according PU size is displayed in Table 7, and the attached spreadsheet show the numeric results of different configurations in detail. (The result in table is the case of that SDC and DLT both are enabled.)

Table 7 Algebraic expressions of storage requirement (N is PU size)

| Data | Number of bits |
| --- | --- |
| Reconstructed Luma samples | N×N×8 |
| Contour pattern | N×N×1 |
| Above and left reference samples | 8×8 |
| Predicted DC | 2×8 |
| dcOffset | 2×2×8 |
| Reconstructed block | N×N×8 |

### Data transfer rate

Input data and output transferred between DMM 4 and other modules would be taken into consideration. Hence, above and left reference samples, co-located Luma samples, dcOffset, and reconstructed block are calculated. Consequently, the algebraic expression of data transfer rate according PU size is displayed in Table 8, and the attached spreadsheet show the numeric results of different configurations in detail. (The result in table is the case of that SDC and DLT both are enabled.)

Table 8 Algebraic expressions of data transfer rate (N is PU size)

| Data | Bits/PU |
| --- | --- |
| Reconstructed Luma samples (Input) | N×N×8 |
| Above and left reference samples (Input) | 8×8 |
| dcOffset (Input) | 2×2×8 |
| Reconstructed block (Output) | N×N×8 |

## Conventional intra prediction (Angular mode)

### Number of operations (based on spec)

- Angular intra prediction: 4N2+2N addition/subtractions, 4N2 multiplications

For the case of predModeIntra <= 18, the other case is similar.

If (intraPredAngle < 0)
  If (( N * intraPredAngle )  >>  5 < −1)
	ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ], for x = −1..( N * intraPredAngle )  >>  5
Otherwise, 
  ref[ x ] = p[ −1 + x ][ −1 ], with x = N + 1..2 * N

For 0 <= x, y < N-1:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5
iFact = ( ( y + 1 ) * intraPredAngle ) & 31

If (iFact != 0)
  predSamples[ x ][ y ] = ( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5
Otherwise (iFact == 0)
  predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]

If (predModeIntra == 26 (Vertical mode) && cIdx == 0 && N < 32)
For 0 <= x < N-1 and y= 0:
  predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )

- Segmental depth intra coding is ON: (I.8.4.4.3): 
DLT ON: 1+N2 comparison (1-bit), 3+3N2 additions, 2N2 LUT access
DLT OFF: 1+N2 comparison (1-bit), N2 additions

If (DltFlag == 1)

dcPred[0] = ( predSamples[0][0] + predSamples[0][N − 1] + 
predSamples[N − 1][ 0 ] + predSamples[N − 1][N − 1] + 2 )  >>  2

For 0<= x, y < N:
dltIdxPred = DepthValue2Idx[dcPred[0]]
dltIdxResi = DcOffset[xTb][yTb][0]
Recblock[x][y] = predSamples[x][y] + Idx2DepthValue[dltIdxPred + dltIdxResi] − dcPred[0]

Otherwise (DltFlag == 0)

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + DcOffset[xTb][yTb][0]

where xTb, yTb are the top-left sample position of current block and Recblock is the reconstructed block.

- Segmental depth intra coding is OFF: 
DLT ON and OFF: De-quantization and inverse transform are applied. The corresponding complexity is illustrated in appendix.

### Number of operations (simplified, segment-wise processing)

- Angular intra prediction: 4N2+2N addition/subtractions, 4N2 multiplications (unchanged)

- Segmental depth intra coding is ON (I.8.4.4.3): 
DLT ON: 2 comparison (1-bit), 5+N2 additions, 2 LUT access
DLT OFF: 2comparison (1-bit), N2 additions, 0 LUT access

If (DltFlag == 1)

dcPred[0] = ( predSamples[0][0] + predSamples[0][N − 1] + 
predSamples[N − 1][ 0 ] + predSamples[N − 1][N − 1] + 2 )  >>  2

For 0<= x, y < N:
dltIdxPred = DepthValue2Idx[dcPred[0]]
dltIdxResi = DcOffset[xTb][yTb][0]
Recblock[x][y] = predSamples[x][y] + Idx2DepthValue[dltIdxPred + dltIdxResi] − dcPred[0]

dltIdxPred = DepthValue2Idx[dcPred[0]]
dltIdxResi = DcOffset[xTb][yTb][0]
Resi = Idx2DepthValue[dltIdxPred + dltIdxResi] − dcPred[0]

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + Resi

Otherwise (DltFlag == 0)

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + DcOffset[xTb][yTb][0]

Resi = DcOffset[xTb][yTb][0]

For 0<= x, y < N:
Recblock[x][y] = predSamples[x][y] + Resi

where xTb, yTb are the top-left sample position of current block and Recblock is the reconstructed block.

- Segmental depth intra coding is OFF: 
DLT ON and OFF: De-quantization and inverse transform are applied. The corresponding complexity is illustrated in appendix.

The number of operations of angular mode with different configurations for one PU is recorded in the algebraic way in Table 9 and Table 10, and the attached spreadsheet illustrated the numeric results in detail.

Table 9 Number of operations of one NN PU (SDC ON + DLT ON)

|  | Original, based on spec |  |  |  | Simplified, segment-wise processing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processes | Comp. (1-bit) | Mul. | Add/Sub | LUT access | Comp. (1-bit) | Mul. | Add/Sub | LUT access |
| Angular intra prediction | 0 | 4N2 | 4N2+2N | 0 | 0 | 4N2 | 4N2+2N | 0 |
| Segmental depth intra coding | 2N2+1 | 0 | 3N2+3 | 2N2 | 2 | 0 | N2+5 | 2 |
| Total | 2N2+1 | 4N2 | 7N2+2N+3 | 2N2 | 2 | 4N2 | 5N2+2N+5 | 2 |

Table 10 Number of operations of one NN PU (SDC ON + DLT OFF)

|  | Original, based on spec |  |  |  | Simplified, segment-wise processing |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processes | Comp. (1-bit) | Mul | Add/Sub | LUT access | Comp. (1-bit) | Mul | Add/Sub | LUT access |
| Angular intra prediction | 0 | 4N2 | 4N2+2N | 0 | 0 | 4N2 | 4N2+2N | 0 |
| Segmental depth intra coding | N2+1 | 0 | N2 | 0 | 2 | 0 | N2 | 0 |
| Total | N2+1 | 4N2 | 5N2+2N | 0 | 2 | 4N2 | 5N2+2N | 0 |

### Storage requirement

Input buffer of angular mode should stores above and left reference samples and dcOffset and output buffer stores the reconstructed block. On the other hand, during the processing of angular mode, some intermediate data should be stored, such as predicted DC (predDCVal and dcPred). In the analysis, N denotes the PU size. Consequently, the algebraic expression of storage requirement according PU size is displayed in Table 11, and the attached spreadsheet show the numeric results of different configurations in detail. (The result in table is the case of that SDC and DLT both are enabled.)

Table 11 Algebraic expressions of storage requirement (N is PU size)

| Data | Number of bits |
| --- | --- |
| Above and left reference samples | (2N+2N)×8 ((2N+N)×8 for 64×64 block) |
| Predicted DC | 1×8 |
| dcOffset | 1×8 |
| Reconstructed block | N×N×8 |

### Data transfer rate

Input data and output transferred between angular mode and other modules would be taken into consideration. Hence, above and left reference samples, dcOffset and reconstructed block are calculated. Consequently, the algebraic expression of data transfer rate according PU size is displayed in Table 12, and the attached spreadsheet show the numeric results of different configurations in detail. (The result in table is the case of that SDC and DLT both are enabled.)

Table 12 Algebraic expressions of data transfer rate (N is PU size)

| Data | Bits/PU |
| --- | --- |
| Above and left reference samples (Input) | 2N+2N (2N+N for 64×64 block) |
| dcOffset (Input) | 1×8 |
| Reconstructed block (Output) | N×N×8 |

## Overall Summary

The following tables illustrated complexity on 6464 CTU level at various PU size of angular mode, DMM 1, and DMM 4 under various configurations. (Enable or disable SDC and DLT.) Table 13, Table 14, and Table 15 display the number of operations required by angular mode, DMM 1, DMM 4 at 6464 CTU at various PU size, respectively. On the other hand, Table 16 and Table 17 record storage requirement for one 6464 CTU and data transfer rate per 6464 CTU at various PU size, respectively. (For de-quantization and inverse transform, TU size is equal to PU size for current evaluation; in addition, for one 6464 block, four 3232 blocks are applied.)

From the perspective of number of operations, when SDC is enabled, the operations are dramatically reduced since de-quantization and inverse transform did not need anymore. The number of multiplications reduces about 7 times of the worst case and division is not required; furthermore, additions/subtractions/comparisons are down to about 19% when SDC is enabled. This influence on DMM 1 and DMM 4 are similar when SDC is enabled. On the other hand, when comparing angular, DMM 1, and DMM 4 at the same configuration, DMM 1 and DMM 4 are simpler than angular mode due to the fact that the complicated intra prediction is not required; hence, the number of multiplications is zero for both DMM 1 and DMM 4. In addition, because DMM 4 uses the co-located texture data to predict wedge pattern; thus, DMM 4 requires more operations than DMM 1 does.

From the perspective of data storage size and data transfer rate, in the assessment, all the required input data and output data should be buffered for the core process and these data are also transferred between modules. When SDC is enabled, the scaling coefficient of de-quantization is not required anymore; hence, the storage size is reduced for three cases concurrently. On the other hand, when evaluating the complexity among angular mode, DMM 1, and DMM 4 at identical configuration, storage requirement and data transfer rate of DMM 4 are the largest since one N by N texture block is required to derive wedge pattern. It is a huge data as compared to miscellaneous data, such as wedge pattern, dcOffset, etc. On the other hand, the data storage and data transfer rate of DMM 1 and angular mode are comparable since DMM 1 requires wedge pattern and angular mode requires more reference pixels to perform intra prediction.

Consequently, the complexity of configurations with and without SDC are significantly different and the complexity of the case which enables SDC is lower than the one disables SDC whatever intra modes are used.

Table 13 Number of operations of “Angular mode” for one 6464 CTU

|  | SDC OFF + DLT OFF |  |  |  | SDC ON + DLT OFF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | Mul. | Div. | Add/Sub /Comp | LUT access | Mul. | Div. | Add/Sub /Comp | LUT access |
| 64×64 | 112,640 | 4,096 | 120,064 | 8,192 | 16,384 | 0 | 21,000 | 0 |
| 32×32 | 112,640 | 4,096 | 120,320 | 8,192 | 16,384 | 0 | 21,000 | 0 |
| 16×16 | 69,632 | 4,096 | 76,800 | 8,192 | 16,384 | 0 | 21,536 | 0 |
| 8×8 | 49,152 | 4,096 | 55,296 | 8,192 | 16,384 | 0 | 22,656 | 0 |
| 4×4 | 43,008 | 4,096 | 51,200 | 8,192 | 16,384 | 0 | 25,088 | 0 |
|  | SDC OFF + DLT ON |  |  |  | SDC ON + DLT ON |  |  |  |
| PU size | Mul. | Div. | Add/Sub /Comp | LUT access | Mul. | Div. | Add/Sub /Comp | LUT access |
| 64×64 | N/A |  |  |  | 16,384 | 0 | 21,020 | 8 |
| 32×32 |  |  |  |  | 16,384 | 0 | 21,020 | 8 |
| 16×16 |  |  |  |  | 16,384 | 0 | 21,616 | 32 |
| 8×8 |  |  |  |  | 16,384 | 0 | 22,976 | 128 |
| 4×4 |  |  |  |  | 16,384 | 0 | 26,368 | 512 |

Table 14 Number of operations of “DMM 1” for one 6464 CTU

|  | SDC OFF + DLT OFF |  |  |  | SDC ON + DLT OFF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | Mul. | Div. | Add/Sub /Comp | LUT access | Mul. | Div. | Add/Sub /Comp | LUT access |
| 64×64 | N/A | N/A | N/A | N/A | 0 | 0 | 4,180 | 16 |
| 32×32 | 96,256 | 4,096 | 103,480 | 8,192 | 0 | 0 | 4,180 | 16 |
| 16×16 | 53,248 | 4,096 | 59,616 | 8,192 | 0 | 0 | 4,432 | 64 |
| 8×8 | 32,768 | 4,096 | 37,760 | 8,192 | 0 | 0 | 5,440 | 256 |
| 4×4 | 26,624 | 4,096 | 34,304 | 8,192 | 0 | 0 | 9,472 | 1,024 |
|  | SDC OFF + DLT ON |  |  |  | SDC ON + DLT ON |  |  |  |
| PU size | Mul. | Div. | Add/Sub /Comp | LUT access | Mul. | Div. | Add/Sub /Comp | LUT access |
| 64×64 | N/A | N/A | N/A | N/A | 0 | 0 | 4,180 | 32 |
| 32×32 | 96,256 | 4,096 | 103,480 | 8,208 | 0 | 0 | 4,180 | 32 |
| 16×16 | 53,248 | 4,096 | 59,616 | 8,256 | 0 | 0 | 4,432 | 128 |
| 8×8 | 32,768 | 4,096 | 37,760 | 8,448 | 0 | 0 | 5,440 | 512 |
| 4×4 | 26,624 | 4,096 | 34,304 | 9,216 | 0 | 0 | 9,472 | 2,048 |

Table 15 Number of operations of “DMM 4” for one 6464 CTU

|  | SDC OFF + DLT OFF |  |  |  | SDC ON + DLT OFF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | Mul. | Div. | Add/Sub /Comp | LUT access | Mul. | Div. | Add/Sub /Comp | LUT access |
| 64×64 | N/A | N/A | N/A | N/A | 0 | 0 | 12,372 | 16 |
| 32×32 | 96,256 | 4,096 | 111,672 | 8,192 | 0 | 0 | 12,372 | 16 |
| 16×16 | 53,248 | 4,096 | 67,808 | 8,192 | 0 | 0 | 12,624 | 64 |
| 8×8 | 32,768 | 4,096 | 45,952 | 8,192 | 0 | 0 | 13,632 | 256 |
| 4×4 | 26,624 | 4,096 | 42,496 | 8,192 | 0 | 0 | 17,664 | 1,024 |
|  | SDC OFF + DLT ON |  |  |  | SDC ON + DLT ON |  |  |  |
| PU size | Mul. | Div. | Add/Sub /Comp | LUT access | Mul. | Div. | Add/Sub /Comp | LUT access |
| 64×64 | N/A | N/A | N/A | N/A | 0 | 0 | 12,372 | 32 |
| 32×32 | 96,256 | 4,096 | 111,672 | 8,208 | 0 | 0 | 12,372 | 32 |
| 16×16 | 53,248 | 4,096 | 67,808 | 8,256 | 0 | 0 | 12,624 | 128 |
| 8×8 | 32,768 | 4,096 | 45,952 | 8,448 | 0 | 0 | 13,632 | 512 |
| 4×4 | 26,624 | 4,096 | 42,496 | 9,216 | 0 | 0 | 17,664 | 2,048 |

Table 16 Storage requirement of “Angular mode”, “DMM 1,” and “DMM 4” for one 6464 CTU (bits)

|  | Angular |  | DMM 1 |  | DMM 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| PU size | SDC OFF + DLT OFF | SDC ON + DLT ON/OFF | SDC OFF + DLT ON/OFF | SDC ON + DLT ON/OFF | SDC OFF + DLT ON/OFF | SDC ON + DLT ON/OFF |
| 64×64 | 67,136 | 34,344 | N/A | 37,083 | N/A | 69,840 |
| 32×32 | 17,472 | 9,232 | 17,579 | 9,339 | 25,760 | 17,520 |
| 16×16 | 4,672 | 2,576 | 4,523 | 2,427 | 6,560 | 4,464 |
| 8×8 | 1,344 | 784 | 1,259 | 699 | 1,760 | 1,200 |
| 4×4 | 448 | 272 | 443 | 267 | 560 | 384 |

Table 17 Data transfer rate of “Angular mode”, “DMM 1,” and “DMM 4” for one 6464 CTU. (bits/6464 CTU)

|  | Angular |  | DMM 1 |  | DMM 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| PU size | SDC OFF + DLT OFF | SDC ON + DLT ON/OFF | SDC OFF + DLT ON/OFF | SDC ON + DLT ON/OFF | SDC OFF + DLT ON/OFF | SDC ON + DLT ON/OFF |
| 64×64 | 67,088 | 34,368 | N/A | 37,067 | N/A | 65,728 |
| 32×32 | 69,696 | 36,928 | 70,060 | 37,292 | 98,688 | 65,920 |
| 16×16 | 73,984 | 41,216 | 71,344 | 38,576 | 99,840 | 67,072 |
| 8×8 | 82,944 | 50,176 | 76,480 | 43,712 | 104,448 | 71,680 |
| 4×4 | 102,400 | 69,632 | 97,024 | 64,256 | 122,880 | 90,112 |

## Experimental Results

This section provides the simulation results which enables partial functions in depth intra coding and anchor of all simulations is HTM-11.0. Table 18 illustrates coding condition for each case. Case 0, case 1, and case 2 demonstrate the capability of each coding tool, including DMM, SDC, and DLT in 3D-HEVC. Case 0 shows that the DLT did not significantly influence coding results due to the fact that only four test sequences are applied DLT. Case 1 displays the benefits brought from SDC, the coding gain is dropped when SDC is disabled; in addition, SDC also significantly reduces the number of operations that required by de-quantization and inverse transform. Case 2 studies the advantage of DMM. When DMM is disabled, 5.7% and 2.4% BD-rate increasing on synth PSNR vs. total bitrate for all intra and CTC configurations, respectively. DMM exactly models the depth information better than the angular mode in most situations.

Case 3 increases 10.3% and 3.6% BD-rate on synth PSNR vs. total bitrate for all intra and CTC configurations. Since DMM models the depth information as segments, SDC would bring advantages on top of DMM. Therefore, when SDC and DMM are disabled concurrently, the coding performance is dropped dramatically. In the attachments, more coding condition combinations are provided.

Table 18 Coding condition for each case

|  | DMM (1 and 4) | SDC | DLT |
| --- | --- | --- | --- |
| Anchor (HTM-11.0) | ON | ON | ON |
| Case 0 | ON | ON | OFF |
| Case 1 | ON | OFF | ON |
| Case 2 | OFF | ON | ON |
| Case 3 | OFF | OFF | OFF |

Table 19 All intra configuration: coding results of case 0.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 1.1% | 99.2% | 99.6% | 99.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 1.1% | 99.0% | 99.2% | 100.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 1.2% | 100.2% | 98.9% | 100.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 100.1% | 99.6% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.3% | 101.2% | 97.0% | 100.0% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 99.8% | 99.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 101.6% | 101.5% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 100.8% | 100.5% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 1.1% | 99.5% | 99.2% | 100.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 100.3% | 99.8% | 100.2% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.5% | 100.0% | 99.6% | 100.2% |

Table 20 CTC configuration: coding results of case 0.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.2% | 99.5% | 100.5% | 99.9% |
| Kendo | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.2% | 99.0% | 101.8% | 106.8% |
| Newspaper_CC | 0.0% | -0.2% | 0.1% | 0.0% | 0.1% | 0.6% | 100.0% | 100.0% | 99.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 100.4% | 100.2% |
| Poznan_Hall2 | 0.0% | -0.1% | 0.3% | 0.0% | -0.1% | 0.0% | 100.0% | 100.0% | 100.0% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.4% | 99.9% | 99.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 99.1% | 100.5% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 99.1% | 100.3% |
| 1024x768 | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.4% | 99.5% | 100.8% | 102.0% |
| 1920x1088 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.2% | 99.7% | 100.1% |
| average | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 99.9% | 100.1% | 100.8% |

Table 21 All intra configuration: coding results of case 1.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 2.5% | 2.0% | 70.0% | 111.3% | 100.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 3.8% | 3.1% | 69.6% | 113.5% | 99.9% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 2.0% | 1.7% | 70.1% | 111.2% | 100.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 1.4% | 1.7% | 74.0% | 106.6% | 99.7% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 1.2% | 1.4% | 72.2% | 115.6% | 100.0% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.7% | 1.0% | 72.5% | 107.2% | 99.4% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.3% | 72.2% | 105.3% | 101.6% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 2.0% | 1.9% | 72.9% | 106.3% | 102.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 2.8% | 2.3% | 69.9% | 112.0% | 100.1% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 1.1% | 1.2% | 72.8% | 108.2% | 100.7% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 1.8% | 1.6% | 71.7% | 109.6% | 100.5% |

Table 22 CTC configuration: coding results of case 1.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | 0.0% | 0.0% | 0.8% | 0.8% | 95.1% | 107.2% | 106.8% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.0% | 1.8% | 1.8% | 95.4% | 104.2% | 99.8% |
| Newspaper_CC | 0.0% | 0.0% | 0.1% | 0.0% | 1.5% | 1.3% | 94.4% | 97.8% | 99.0% |
| GT_Fly | 0.0% | -0.8% | -0.7% | -0.2% | 0.4% | 0.6% | 92.4% | 100.2% | 100.1% |
| Poznan_Hall2 | 0.0% | 0.1% | 0.1% | 0.0% | 0.6% | 0.8% | 93.3% | 102.1% | 99.8% |
| Poznan_Street | 0.0% | -0.4% | -0.4% | -0.1% | 0.3% | 0.5% | 93.2% | 99.9% | 100.0% |
| Undo_Dancer | 0.0% | -0.3% | -0.1% | 0.0% | 0.1% | 0.2% | 94.2% | 99.5% | 99.8% |
| Shark | 0.0% | -0.6% | -0.3% | -0.1% | 0.5% | 0.7% | 93.9% | 100.0% | 100.1% |
| 1024x768 | 0.0% | -0.2% | 0.0% | 0.0% | 1.3% | 1.3% | 94.9% | 103.1% | 101.9% |
| 1920x1088 | 0.0% | -0.4% | -0.3% | -0.1% | 0.4% | 0.6% | 93.4% | 100.3% | 100.0% |
| average | 0.0% | -0.3% | -0.2% | -0.1% | 0.7% | 0.8% | 94.0% | 101.4% | 100.7% |

Table 23 All intra configuration: coding results of case 2.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 4.2% | 64.1% | 100.7% | 100.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 4.5% | 64.5% | 100.8% | 100.8% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.7% | 7.6% | 63.1% | 100.3% | 100.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | 4.7% | 63.7% | 99.3% | 98.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | 6.1% | 63.3% | 99.4% | 100.3% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | 2.2% | 64.4% | 101.1% | 100.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 8.2% | 62.7% | 99.3% | 100.4% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 7.9% | 67.4% | 96.5% | 99.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | 5.4% | 63.9% | 100.6% | 100.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 5.8% | 64.3% | 99.1% | 99.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 5.7% | 64.2% | 99.7% | 99.9% |

Table 24 CTC configuration: coding results of case 2.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.0% | -0.3% | 1.4% | 96.8% | 101.3% | 106.4% |
| Kendo | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 1.3% | 96.1% | 101.3% | 101.9% |
| Newspaper_CC | 0.0% | 0.0% | 0.1% | 0.0% | -0.7% | 4.2% | 95.5% | 100.9% | 102.9% |
| GT_Fly | 0.0% | 0.9% | 0.8% | 0.2% | -0.3% | 0.7% | 95.5% | 100.0% | 100.1% |
| Poznan_Hall2 | 0.0% | -0.2% | -0.1% | -0.1% | -0.5% | 2.1% | 95.4% | 100.1% | 99.8% |
| Poznan_Street | 0.0% | 0.4% | 0.4% | 0.1% | -0.3% | 1.3% | 95.2% | 99.7% | 99.8% |
| Undo_Dancer | 0.0% | 1.2% | 1.3% | 0.3% | 0.5% | 4.1% | 96.3% | 100.0% | 99.9% |
| Shark | 0.0% | 1.6% | 1.6% | 0.3% | 0.4% | 4.0% | 96.7% | 99.7% | 100.0% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | -0.4% | 2.3% | 96.1% | 101.1% | 103.8% |
| 1920x1088 | 0.0% | 0.8% | 0.8% | 0.2% | -0.1% | 2.4% | 95.8% | 99.9% | 99.9% |
| average | 0.0% | 0.5% | 0.5% | 0.1% | -0.2% | 2.4% | 95.9% | 100.4% | 101.4% |

Table 25 All intra configuration: coding results of case 3.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 2.5% | 7.5% | 39.6% | 107.3% | 100.7% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 3.9% | 9.1% | 39.0% | 107.5% | 101.3% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 1.4% | 12.7% | 39.8% | 107.5% | 100.5% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 1.8% | 10.5% | 39.9% | 108.1% | 98.6% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.7% | 11.3% | 39.3% | 101.2% | 99.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 4.6% | 40.7% | 118.3% | 104.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 12.4% | 39.9% | 105.4% | 101.6% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 3.5% | 14.3% | 43.4% | 105.0% | 99.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 2.6% | 9.8% | 39.4% | 107.4% | 100.9% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 1.3% | 10.6% | 40.6% | 107.6% | 101.0% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 1.8% | 10.3% | 40.2% | 107.5% | 100.9% |

Table 26 CTC configuration: coding results of case 3.

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.0% | 0.6% | 2.3% | 92.8% | 107.8% | 109.2% |
| Kendo | 0.0% | -0.2% | 0.0% | 0.0% | 1.5% | 3.1% | 93.1% | 99.0% | 100.6% |
| Newspaper_CC | 0.0% | 0.1% | 0.1% | 0.0% | 0.8% | 6.3% | 91.0% | 95.9% | 98.2% |
| GT_Fly | 0.0% | 0.3% | 0.3% | 0.1% | 0.4% | 2.1% | 88.8% | 100.4% | 99.8% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.4% | -0.1% | 0.0% | 2.9% | 89.7% | 101.0% | 100.1% |
| Poznan_Street | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 2.1% | 89.5% | 99.9% | 99.8% |
| Undo_Dancer | 0.0% | 1.3% | 1.2% | 0.3% | 0.8% | 5.0% | 91.6% | 100.0% | 100.0% |
| Shark | 0.0% | 1.0% | 1.3% | 0.2% | 1.2% | 5.4% | 91.1% | 100.4% | 100.3% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | 1.0% | 3.9% | 92.3% | 100.9% | 102.7% |
| 1920x1088 | 0.0% | 0.5% | 0.5% | 0.1% | 0.5% | 3.5% | 90.1% | 100.3% | 100.0% |
| average | 0.0% | 0.3% | 0.3% | 0.1% | 0.7% | 3.6% | 90.9% | 100.6% | 101.0% |

# Conclusion

From Table 13 to Table 26, the complexity of depth intra mode is assessed to assist the JCT3V team in evaluating the proposals related to depth intra modes. The results can be considered as anchor complexity. In addition, this proposal presents the complexity with different data flows to show that the complexity might be varied for different data flows. Thus, with the help of complexity analysis, this proposal would assist the designer to evaluate the developed coding tools. This proposal provides not only the evaluated complexity of coding tools but also an instruction on that how to assess the intrinsic algorithmic complexity. Due to the fact that the assessed complexity is independent from any platform or architecture, these numbers provide a fair complexity comparison.

In sum, the coding tools of depth intra mode introduced in 3D-HEVC, they did not significantly increase complexity at decoder side but provide good coding performance since they are designed according to the characteristics of depth data. On the other hand, from the perspective of complexity assessment, this proposal also justify that number of operations could be significantly reduced when SDC is enabled.

# Reference

- JCT3V-H0008, “JCT-3V AHG Report: Complexity Assessment (AHG8),” Valencia, Spain, 29 Mar. - 04 Apr. 2014.

- JCT3V-H0127, “AHG8: Complexity Assessment on Depth Intra Modes,” Valencia, Spain, 29 Mar. - 04 Apr. 2014.

- JCT3V-G0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” San Jose, USA, 11 - 17 Jan. 2014.

- JCT3V-G0114, “AHG 10: Complexity Assessment on Illumination Compensation (IC),” San Jose, USA, 11 - 17 Jan. 2014.

- JCT3V-F0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” Geneva, CH, 25 Oct. - 1 Nov. 2013.

- JCT3V-E0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” Vienna, AT, 27 July - 2 Aug. 2013.

- JCT3V-E0132, “AHG 10: Complexity Assessment on Simplified Depth Coding (SDC),” Vienna, AT, 27 July - 2 Aug. 2013.

- JCT3V-D0010, “JCT-3V AHG Report: Complexity assessment (AHG10),” Incheon, KR, 20-26 April 2013.

- JCT3V-D0301, “AHG 10 Overview of Algorithmic Intrinsic Complexity Measures,” Incheon, KR, 20-26 April 2013.

- JCT3V-H1001, “3D-HEVC Draft Text 4,” Valencia, Spain, 29 Mar. - 04 Apr. 2014.

- JCT3V-H1003, “Test Model 8 of 3D-HEVC and MV-HEVC,” Valencia, Spain, 29 Mar. - 04 Apr. 2014.

# Appendix

## Complexity of de-quantization and inverse transform (SDC OFF)

When SDC disables, de-quantization and inverse transform are required to derive the residuals of predicted samples; hence, the block could be reconstructed. The complexity of de-quantization, inverse transform, and block reconstruction are illustrated in the following subsections.

### Number of operations

- Inverse quantization: N2 comparison (1-bit), 2N2 LUT access, 2N2 multiplications, N2 divisions

For 0 <= x, y < N
   m = (scaling_list_enabled_flag == 0)? 16 : ScalingFactor[ sizeId ][ matrixId ][ x ][ y ]
   ScaledTransCoeff[x][y] = TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * m * 			levelScale[ qP%6 ]  <<  (qP / 6 )

- Inverse transform: please refer to the attached spreadsheet; the value cannot be expressed associated with block size.

The complexity of inverse transform is assessed according to reference software, that is, partial butterfly is applied. After this process, Resiblock, residual of each pixel, is obtained.

- Block reconstruction: N2 additions

For 0 <= x, y < N
Recblock [ x ][ y ] = predSamples[ x ][ y ] + Resiblock[ x ][ y ]

### Storage requirement

1. LUT of scaling factor (ScalingFactor): MN8 + 68

One MN block which is recorded the scaling factor for each pixel. Bit-width of scaling factor is 8-bit.

2. level scale (levelScale): one storage with six contents. Bit-width of each content is 8-bit.

### Data transfer rate

1. LUT of scaling factor (ScalingFactor): MN8

One MN block which is recorded the scaling factor for each pixel. Bit-width of scaling factor is 8-bit.