| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0070r1 |
| --- | --- |

| Title: | Simplification of partition derivation in DBBP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ce Zhu1, Yu-Hua Zhang1, Yong-Bing Lin2, Jian-Hua Zheng2, Jian-Jun Song1 1 School of Electronic Engineering University of Electronic Science and Technology of China, Chengdu, P. R. China,611731 2 HiSilicon technologies, Beijing, P. R. China, 100095 | Tel: Email: | +1-822-808-8676 eczhu@uestc.edu.cn +1-500-840-0575 jieruo@yeah.net +86-10-60612372 linyongbing@hisilicon.com +1-316-192-6232 zhengjianhua@hisilicon.com +1-588-446-9103 tsjj.123@163.com |
| Source: | UESTC, HiSilicon |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the depth-based block partitioning (DBBP) [1] comprises steps of partitioning derivation, segment mask generation, bi-segment compensation, and merging of prediction signals. The partitioning derivation process, with derived virtual depth as input, is relatively high complex, as it needs to perform sub-sample mean value calculation and to search the most matched partition mode. In this contribution, two methods are proposed to simplify the partition derivation for DBBP. Method #1 proposes to use default partition mode in order to remove the existing process of DBBP partition derivation. Method #2 proposes to derive the partition by simply comparing the mask value of two cornor pixels. Experimental results reportedly show the proposed simplifications bring almost no coding efficiency loss.

- Introduction

In DBBP of the current 3D-HEVC[2], the decoder requires five steps: virtual depth derivation, partitioning derivation, segment mask generation, bi-segment compensation，and merging of prediction signals. The virtual depth derivation step utilizes the disparity vector from neighboring blocks (NBDV) to address a virtual depth block from the coded depth picture in the dependent depth view. One partition mode is selected from 2NxN, Nx2N in the partitioning derivation process to derive two predictive motion vectors (PMVs), which are utilized for the compensation procedure for the to-be-divided two segments. The segment mask generation step divides the current texture block into two segments in forms of segment mask according to the virtual depth block. The bi-segment compensation step utilizes the PMVs and MVDs from the block partitioning step and the derived segment mask from the block segmentation step to fetch the predicted samples and then completes the compensation by two-time 2Nx2N compensations with their own decoded motion vectors. In the final bi-segment compensation step, the compensation is done under the control of the calculated segment mask.

Thereinto, the partition derivation process consists of two steps. Firstly, the threshold is obtained by calculating the mean value of top-left pixels of all the 4x4 sub-blocks in the derived virtual depth block, which are marked in Figure 1. Secondly, 4x4 block-wise segmentation map is derived by using the calculated threshold in the first step, and then matched filtering is performed to derive the best matching PU partition mode. As it needs to perform sub-sample mean value calculation and matched filtering, which consumes a certain memory access, hardware resources and computation complexity, the partition derivation is relatively high complex.

Figure 1. Example of used depth samples to derive the PU partition mode in the current method

(CU size: 8x8).

- Proposed methods

The main idea behind the proposed methods is to reduce the complexity by omitting some operations, such as sub-sample mean value calculation and matched filtering, as is done in the partition derivation process of current 3D-HEVC.

- Method #1: Setting the default partition mode for DBBP.

- 2NxN or Nx2N is directly set as the default partition mode for DBBP.

- Method #2: Using the calculated segment mask to derive partition mode for DBBP.

- The proposed partition derivation simply uses two segment mask values (Mask1 and Mask2 in Figure 2), calculated in segment mask generation step, to derive the partition mode for DBBP. It compares the value size between Mask1 and Mask2, and then directly decides the PU partition mode between 2NxN and Nx2N. If Mask1 is equal to Mask2, 2NxN is selected to be the final partition mode. Otherwise, Nx2N will be selected.

Figure 2. Example of used segment mask to derive the PU partition mode in the proposed method

(CU size: 8x8).

- Experimental Results

The proposed methods were implemented on top of HTM-11.0[3] and were carried out on the common test condition [4]. The results are summarized in Table 1 and Table 2.

Table 1 shows the results of Method #1. The proposed method provides negligible coding loss (0.03% bit-saving for synthesized views).

Table 1. Comparison results between CTC anchor and Method #1

In addition, Table 1.a also provides the test result for method#1 when the default partition size is Nx2N.

Table 1.a Comparison results between CTC anchor and Method #1 with default partition Nx2N

Table 2 shows the results of Method #2. The proposed method provides no coding loss (-0.01% bit-saving for synthesized views).

Table 2. Comparison results between CTC anchor and Method #2

- Conclusions

In this contribution, two methods are proposed to reduce the complexity of the partitioning derivation process in DBBP. Method 1 proposes to directly use the default partition mode and Method 2 proposes to derive partition mode for DBBP by simply comparing the segment mask values of two pixels. Each of the two proposed methods can reduce the memory and computational complexity. The experiments results show almost no coding efficiency loss.

- Patent rights declaration(s)

UESTC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

HiSilicon Technologies and Huawei technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

- F. Jäger, J. Konieczny, G. Cordara, “CE3: Results on Depth-based Block Partitioning (DBBP),” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G0106, 7th Meeting: San José, US, Jan. 11–17, 2014.

- Y. Chen, G. Tech, K. Wegner, S. Yea, “Test Model 8 of 3D-HEVC and MV-HEVC,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-H1003, 8th Meeting: Valencia, ES, Mar. 29–Apr. 4, 2014.

- HTM-11.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0

- K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, Jan. 11–17, 2014.