| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0072 |
| --- | --- |

| Title: | Problem fix for MV scaling in inter-view ARP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current inter-view ARP, a problem exists in the scaling process when the MV in the reference view is in inter-view direction (IBP configuration) which results in the software crash. This contribution proposes to fix this problem by setting the derived MV directly as (0, 0) for that situation to avoid the problematic scaling.

# Introduction

In inter-view ARP of current 3D-HEVC, the derived MV in the reference view is scaled to the current temporal reference picture according to the POC distance. However, when the MV in reference view is in inter-view direction in IBP configuration, the scaling process becomes meanlingless and will result in software crash since the denominator (the POC distance) equal to 0.

# Proposed method

In order to solve the problem, it is proposed to scale the MV in the reference view only when the MV is in temporal direction. If the MV in reference view is in inter-view direction, the derived MV is set to the default value (0, 0).

The related part in the draft text [1] is modified as follows: (added part is highlighted, and the deleted part is strikethrough)

- I 8.5.3.3.7.4 Derivation process for a motion vector from a reference block for residual prediction

…

When the variable CuPredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following applies for X in the range of 0 to 1, inclusive:

  - The variable refPredFlagLX is set equal to the prediction utilization flag predFlagLX of the prediction unit refPU.

  - When availFlag is equal to 0 and refPredFlagLX is equal to 1, the following applies:

    - Let refPicListRefX be the reference picture list X of refPic.

    - Let mvLX and refIdxLX be the motion vector and reference index of the prediction unit refPU corresponding to refPicListRefX, respectively.

    - When PicOrderCnt(refPicListRefX[ refIdxLX ]) is not equal to PicOrderCnt(refPic) refPicListRefX[ refIdxLX ] is a temporal reference picture of refPic and RpRefIdxLX is not equal to −1, availFlag is set to 1, Y is set equal to X and the residual prediction motion vector scaling process as specified in subclause Error: Reference source not found is invoked with the prediction list utilization variable equal to X, the motion vector mvLX, and the reference picture refPicListRefX[ refIdxLX ] as inputs, and the output being mvT.

With this modification, the software with IBP configuration can run normally without crash.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H1001, Apr. 2014.