| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0073 |
| --- | --- |

| Title: | MV sharing for depth coding in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a MV sharing method between the depth and texture components is proposed. MVs needed by temporal motion vector prediction for depth coding are fetched from the corresponding texture picture instead of a depth picture. MV storage can be halved in the proposed approach. In addition, the times of loading the MV buffer from external memory to on-chip memory can also be halved. The experimental results reportedly show that the proposed simplification brings no coding loss.

# Introduction

Temporal motion vector prediction (TMVP) is applied on both texture and depth component in 3D-HEVC [1]. When TMVP is used, a motion data is fetched from the MV buffer of a reference picture, which is the collocated picture, as depicted in Fig.1(a). For each reference picture in the reference list, a picture buffer is needed to store the motion information. Moreover, the MV buffers for TMVP in depth and texture component are manipulated separately.

# Proposed method

It is proposed that the MV storage for TMVP are shared by the depth and texture components. When TMVP is used in a depth picture, the MV is fetched from the MV buffer of the texture picture corresponding to the reference depth picture, i.e., the collocated picture, as illustrated in Fig.1(b). Several benefits can be attained with this modification. First, MVs for depth reference pictures are not required to be stored any longer. Thus the MV storage for TMVP can be halved in 3D-HEVC. Second, the times of loading MV buffers from the external memory to the on-chip memory can also be halved as depicted in Fig.2. In addition, the motion compression process for TMVP is no more required by a depth picture.

# Experimental results

The proposed method is integrated on HTM-11.0 [2]. Table 1 shows the results under the common test condition [3]. The experiments results show that the proposed simplification brings no coding loss.

| (a) | (b) |
| --- | --- |

Fig. 1 MV storage in the current 3D-HEVC (a), and in the proposed method (b).

| (a) | (b) |
| --- | --- |

Fig. 2 MV buffer loading in the current 3D-HEVC (a), and in the proposed method (b).

Table 1.  Performance for the proposed MV sharing (CTC)

# Conclusion

A MV sharing method between the depth and texture components is proposed in this contribution. MV storage and times of loading the MV buffer from the external memory to the on-chip memory can be reduced as proposed. Experimental results reportedly show no coding performance loss.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H1001, Apr. 2014.

- HTM-11.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0.

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.