| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0075 |
| --- | --- |

| Title: | Segmental prediction for Inter-SDC in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a segmental prediction method for Inter-SDC is proposed in depth coding. Besides the current Inter-SDC mode, three segmental Inter-SDC modes with 1, 2 and 3 segments are added. A single depth value is assigned to all reconstructed samples in a segment. The experimental results reportedly show that proposed method can achieve 0.4% BD-rate saving for the synthesized views in the common test condition.

# Introduction

Inter-SDC is a special inter-prediction mode for depth coding in 3D-HEVC [1]. When Inter-SDC is applied, a normal inter-prediction is conducted first. Then a coded offset is added to all prediction samples to get the reconstructed sample values.

In a depth picture, the foreground and background usually hold very different brightness with sharp edges between them. Fig. 1 and Fig. 2 demonstrate two examples. Because the coding process loses high-frequency information generally, the quality of reconstruction is deteriorated more when there are acute pixel value changes. The coding efficiency of Inter-SDC will be reduced when such reconstructed pixels are referred to as the prediction samples.

Fig. 1 A block with two segments with sharp sample value changes.

Fig. 2 A block with three segments with sharp sample value changes.

# Proposed method

In this contribution, a segmental prediction method for Inter-SDC (segInter-SDC) is proposed. A flag is signaled whether to use the normal Inter-SDC or the segInter-SDC for a CU coded with Inter-SDC mode. When segInter-SDC is applied, two steps are applied to obtain the reconstructed block from the prediction block. Fig. 3 illustrates an example of segInter-SDC with 2 segments for an 8x8 block.

In the first step, samples in the current block are classified into one, two or three segments. All samples belong to segment 0 when the predication block possesses only one segment. When the prediction block is divided into two segments, a sample is classified to segment 0 if its value is smaller than the average value of all the samples in the prediction block. Otherwise, it is classified into segment 1. When the prediction block is divided into three segments, two thresholds T1 and T2 are calculated as T1= (T+Vmin)/2 and T2= (Vmax+T)/2, where T is the average value of all the samples in the prediction block. Vmax and Vmin are the minimum sample value and the maximum sample value in the prediction block respectively. A sample is classified into segment 0 if its value is smaller than T1; it is classified into segment 2 if its value is larger than T2; otherwise it is classified into segment 1.

In the second step, a single value is assigned for each segment in the reconstructed block. A single value denoted as V is calculated as V = E + O, where E is an estimated value for the segment of the prediction block and O is an offset transmitted to the decoder. If there is only one segment, E is set equal to average value of all the samples in the prediction block. Otherwise, E = (Vsegmax+Vsegmin)/2, where Vsegmax and Vsegmin are the minimum sample value and the maximum sample value in the segment of the prediction block respectively.

At encoder, the offset for a segment is calculated by subtracting the average value of all the original sample values in the segment of the current block and E.

If segInter-SDC is on, the number of segments, 1, 2 or 3 is signaled explicitly. And the offset is signaled in the same way as DcOffset in the current 3D-HEVC.

Fig. 3 An example of segInter-SDC with 2 segments for an 8x8 block.

# Experimental results

The proposed method is integrated on HTM-11.0 [2]. Table 1 shows the results under the common test condition [3]. The experiments results show that the proposed segInter-SDC can achieve 0.4% BD-rate saving for the synthesized views.

Table 1.  Performance for the proposed MV sharing (CTC)

# Conclusion

A segmental prediction method for Inter-SDC is proposed in this contribution. Three more Inter-SDC modes are added, which use a single reconstructed sample value for each segment. The experimental results show that proposed method can achieve 0.4% BD-rate saving for the synthesized views in the common test condition.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H1001, Apr. 2014.

- HTM-11.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0.

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.