| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0076 |
| --- | --- |

| Title: | Simplification of Threshold Derivation for DBBP and DMM4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, DBBP and DMM4 need to generate a binary segmentation mask used for the segmentation based prediction. The binary segmentation mask is generated by using the threshold derived by averaging all the samples in the corresponding depth or texture block. However, accessing all the samples could cause increasing the computation complexity. Therefore, in order to reduce the complexity, this contribution proposes to use four corner samples in the corresponding block to derive the threshold. The proposed method reportedly provides no coding loss with 96.7% encoding time under the common test conditions. For DMM4, the results based on the all intra test conditions reportedly show 0.1% bit-saving for synthesized views.

# Introduction

In the current 3D-HEVC [1], DBBP (Depth Based Block Partitioning) and DMM4 (Depth Modeling Mode 4) generate a binary segmentation mask used for the segmentation based prediction. The binary segmentation mask is generated based on the depth samples in the corresponding depth block for DBBP and texture samples in the corresponding texture block for DMM4, and the threshold derived by averaging all the samples in the corresponding block. However, accessing all the samples in the corresponding block could cause increasing the computation complexity.

# Proposed Methods

In this contribution, in order to reduce the computational complexity of the threshold derivation process in DBBP and DMM4, it is proposed to use four corner depth or texture samples (a, b, c, and d in Figure 1) in the corresponding depth or texture block to derive the threshold used in DBBP and DMM4.

Figure 1. Example of 4 corner samples used for proposed derivation

The current and proposed threshold derivation methods can be illustrated as equations in Figure 2 (Here, N indicates size of the current CU and d(x, y) indicates a sample in the corresponding block). While the threshold (Thcurr) of the current method is calculated by using the average of all the samples, the threshold (Thprop) of the proposed method is calculated by using the average of four corner samples.

Figure 2. Comparison between the current and proposed derivation methods

# Experimental Results

The simulation was carried out based on the common test condition [2] and HTM 11 reference software. There are three test cases; applying proposed derivation to the DBBP mode (Test 1), applying proposed derivation to the DMM4 mode (Test 2), and combing Test 1 and Test 2 (Test 3).

Table 1 shows the results of applying simplified threshold derivation to DBBP. The proposed method provides no coding loss with 96.9% encoding time.

Table 1. Comparison results between CTC anchor and Test 1 (DBBP)

Table 2 shows the results of applying simplified threshold derivation to DMM4. The proposed method provides no coding loss.

Table 2. Comparison results between CTC anchor and Test 2 (DMM4)

For Test 2, the simulation based on the all intra (AI) test condition was also performed. The proposed method provides 0.1% bit-saving for synthesized views.

Table 3. Comparison results between AI anchor and Test 2 (DMM4)

Table 4 shows the results of applying simplified threshold derivation to both DBBP and DMM4. The propose method provides no coding loss with 96.7% encoding time.

Table 4. Comparison results between CTC anchor and Test 3 (DBBP & DMM4)

In the current 3D-HEVC, the partition derivation for DBBP also has the threshold derivation process. So, the proposed method was additionally applied to the partition derivation process (Test 4). The results with Test 3 show that the proposed method also provides no coding loss as shown in Table 5.

Table 5. Comparison results between CTC anchor and Test 4

# Conclusion

In this contribution, it is proposed to simply the threshold derivation for DBBP and DMM4. With the proposed method, the memory and computational complexity can be reduced. The proposed method provides no coding loss based on the common test condition with about 97% encoding time. For DMM4, the simulation results based on the all intra test condition shows 0.1% bit-saving on synthesized views. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC draft text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” JCT3V-H1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Mar. 2014, Valencia, ES.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.