| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0077_WD2_r1 |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Working Draft for JCT3V-I0077 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Working Draft. The changes are highlighted in green.

Revision 1: the redundant condition is removed to align with software

I.7.3.8.5  Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && ( PartMode == PART_2NxN ¦ ¦ PartMode == PART_Nx2N ) ) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |
| … |  |

I.7.4.9.5  Coding unit semantics

…

- dbbp_flag[ x0 ][ y0 ] equal to 1 specifies that depth based block partition is used for the current coding unit. dbbp_flag[ x0 ][ y0 ] equal to 0 specifies that depth based block partition is not used for the current coding unit. When not present, the value of dbbp_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When dbbp_flag[ x0 ][ y0 ] is equal to 1, PartMode is set equal to PART_2NxN.

…

I.8.5.1  General decoding process for coding units coded in inter prediction mode

…

The decoding process for coding units coded in inter prediction mode consists of following ordered steps:

  - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, following applies:

    - If DepthFlag is equal to 0 the derivation process for disparity vectors as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as inputs.

    - Otherwise (DepthFlag is equal to 1), the derivation process for disparity vectors for depth layers as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as inputs.

  - When dbbp_flag is equal to 1, the derivation process for a modified partitioning mode as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ) and the coding block size nCbSL as the inputs.

  - The inter prediction process as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the outputs are three arrays predSamplesL, predSamplesCb, and predSamplesCr.

I.8.5.7  Derivation process for a modified partitioning mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block.

The derivation process for a depth predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sampling interval sampInt equal to 4, the sample location ( xTb, yTb ) equal to ( xCb, yCb ), and the block size nTbS equal to nCbS as inputs, and the output is a binary partition pattern wedgePattern[ x ][ y ].

For p in the range of 0 to 1, inclusive and i in the range of 0 to 1, inclusive, partSum[ p ][ i ] is set equal to 0.

for( y = 0; y < nCbS ; y += 4 )
		for( x = 0; x < nCbS ; x += 4 ) { 
			segFlag = wedgePattern[ x ][ y ]
			partSum[ 0 ][ ( x < ( nCbS  >>  1 ) ) ? segFlag : !segFlag ]++
			partSum[ 1 ][ ( y < ( nCbS  >>  1 ) ) ? segFlag : !segFlag  ]++	(Error: Reference source not found-1)
		}

The variable partFlag is derived as specified in the following:

partFlag = 0
	maxPartSum = 0
	for( p = 0; p < 2; p++ )
		for( i = 0; i < 2; i++ ) {
			if( partSum[ p ][ i ] > maxPartSum ) {	(Error: Reference source not found-2)
				maxPartSum = partSum[ p ][ i ]
				partFlag = p
			}
		}

The variables x0 and y0 are derived and the variable PartMode is modified depending on partFlag as specified in the following:

  - x0 = partFlag ? xCb : xCb + nCbS / 2

  - y0 = partFlag ? yCb + nCbS / 2 : yCb

  - PartMode = partFlag ? SIZE_2NxN : SIZE_Nx2N

The following applies:

  - MergeIdx[ x0 ][ y0 ] = merge_idx[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-3)

  - MergeFlag[ x0 ][ y0°] = merge_flag[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-4)

  - InterPredIdc[ x0 ][ y0°] = inter_pred_idc[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-5)

For X in the range of 0 to 1, inclusive, the following applies:

  - PuRefIdxLX[ x0 ][ y0°] = ref_idx_lX[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-6)

  - MvpLXFlag[ x0 ][ y0°] = mvp_lX_flag[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-7)

  - MvdLX[ x0 ][ y0°] = MvdLX[ xCb ][ yCb + nCbS/2 ]		(Error: Reference source not found-8)