| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0077 |
| --- | --- |

| Title: | Partition Derivation for DBBP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, when the DBBP mode is enabled, the partition mode used for storing the motion parameters needs to be decided between PART_2NxN and PART_Nx2N at the decoder side. However, the partition derivation has a relatively high complexity since it needs to derive a threshold and then to generate a 4x4 block-wise segmentation map by using the derived threshold. To reduce the complexity of the partition derivation, two options of simplified methods are proposed. The first option is to use three corner samples in the corresponding depth block at decoder side and to signal the enabling flag only when the partition mode is PART_2NxN to remove redundant signaling of the flag. The first option reportedly provides 0.07% bit-saving on coded views. And the other option is to completely remove the partition derivation process from the decoder and to signal the enabling flag only when the partition mode is PART_2NxN and PART_Nx2N so that received partition mode can be used as is to store the motion parameters. The second option reportedly provides 0.03% bit-saving on coded views.

# Introduction

In the current 3D-HEVC [1], DBBP needs to decide the partition mode between PART_2NxN and PART_Nx2N used for storing the motion parameters at the decoder side. But, this derivation has a relatively high complexity since it needs to derive a threshold and then to generate a 4x4 block-wise segmentation map. To be specific, the current partition derivation process consists of two steps. Firstly, the current CU is partitioned into 4x4 blocks and then the threshold is calculated by averaging top-left pixels of all the 4x4 blocks, which are indicated in Figure 1. Secondly, a 4x4 block-wise segmentation mask is derived by the derived threshold at the first step, and then the best matching PU partition mode is finally derived between PART_2NxN and PART_Nx2N.

Figure 1. Example of used depth samples to derive the PU partition mode in the current method
 (CU size: 16x16).

# Proposed Methods

In this contribution, in order to reduce the complexity of the partition derivation, two options of the derivation methods are presented.

## Simplification of Partition Derivation and dbbp_flag Signaling

In this section, it is proposed to use three corner depth samples (a, b and c in Figure 2) in the corresponding depth block to derive the PU partition mode of the current CU as shown in Figure 2.

Figure 2. Three corner depth samples used in the proposed partition derivation

The proposed partition derivation method can be illustrated as shown in Figure 3. The partition mode can be decided between PART_2NxN and PART_Nx2N based on comparison of gradients between ‘a’ and ‘b’ between ‘a’ and ‘c’. If the absolute value of gradient between ‘a’ and ‘b’ is greater than the one between ‘a’ and ‘c’, then the partition mode is set to PART_Nx2N. Otherwise, the partition mode set to PART_2NxN.

Figure 3. Proposed partition derivation method

Additionally, dbbp_flag which indicates whether DBBP is enabled is currently signaled in every partition mode and if dbbp_flag is enabled, the partition mode is set to PART_2NxN and then motion parameters are sent on the assumption that the current partition mode is PART_2NxN. However, it seems to be redundant and complicated since every partition mode has to signal dbbp_flag with ‘0’ even though DBBP is not used for the current CU and the coded partition mode in the bitstream has to be changed when dbbp_flag is enabled. Therefore, it is also proposed to signal dbbp_flag only when the partition mode is equal to PART_2NxN. With the proposed method, the redundant signaling of dbbp_flag and the partition mode setting process can be removed.

## Removal of Partition Derivation

In this section, it is proposed to remove the partition derivation from the decoder and to signal dbbp_flag only when the partition mode is PART_2NxN or PART_Nx2N, which is used in the DBBP mode. With the proposed method, the complexity for the partition derivation at decoder side is completely removed, and the coded partition mode in the bitstream is always identical with the one really used in the DBBP mode.

# Experimental Results

The simulation was carried out based on the common test condition [2] and HTM 11 reference software. There are five test cases; simplified the derivation method in section 2.1 (Test 1), optimization of dbbp_flag signaling in section 2.1 (Test 2), combining Test 1 and Test 2 (Test 3), removal of partition derivation in section 2.2 (Test 4), and combining Test 1 and Test 4 (Test 5).

Table 1 shows the results of simplified the derivation method in section 2.1. The proposed method provides no coding loss.

Table 1. Comparison results between CTC anchor and Test 1

Table 2 shows the results of dbbp_flag signaling in section 2.1. The proposed method provides 0.2% bit-saving for dependent views, and 0.06% and 0.04 bit-saving on coded views and synthesized views, respectively.

Table 2. Comparison results between CTC anchor and Test 2

Table 3 shows the combining results of Test 1 and Test 2. The proposed method provides 0.2% bit-saving for dependent views, and 0.07% and 0.04% bit-saving on coded views and synthesized views, respectively.

Table 3. Comparison results between CTC anchor and Test 3

Table 4 shows the results of the partition derivation removal in section 2.2. The proposed method provides 0.2% bit-saving for dependent views, and 0.03% bit-saving on coded views.

Table 4. Comparison results between CTC anchor and Test 4

Table 5 shows the combining results of Test 1 and Test 4. Here, Test 1 is only used at the encoder side since the decoder does not need to perform the partition derivation anymore. The proposed method provides 0.2% bit-saving for dependent views, and 0.03% and 0.02% bit-saving on coded views and synthesized views, respectively.

Table 5. Comparison results between CTC anchor and Test 5

# Conclusion

In this contribution, two options of the partition derivation methods for DBBP are proposed. The first option is to simplify the partition derivation and to optimize dbbp_flag signaling, and it provides 0.07% bit-saving on coded views. The second option is to remove the partition derivation from the decoder and to unify the coded partition mode and the used partition mode in DBBP, and it provides 0.03% bit-saving on coded views. In terms of the computational complexity, the second option is simpler than the first one since it can completely remove the partition derivation process from the decoder so it is recommended to adopt the second option into next 3D-HEVC draft text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” JCT3V-H1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Mar. 2014, Valencia, ES.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.