| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0078 |
| --- | --- |

| Title: | Memory Complexity for DBBP and VSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, the memory complexity analysis for DBBP and VSP is presented. In the current 3D-HEVC, DBBP uses DoNBDV to derive a binary segmentation map. However, using DoNBDV can cause increasing an external memory bandwidth requirement since it needs to fetch two depth blocks in addition to the motion compensation. Therefore, this contribution proposes two methods to reduce the worst case memory bandwidth of DBBP. The first method is to use NBDV instead of DoNBDV. With this method, the worst case memory bandwidth is reportedly reduced from 120% to 110% with a coding loss of 0.06%. The second method is to allow DBBP only when the current CU size is greater than 8. With this method, the worst case memory bandwidth is reportedly reduced from 120% to 79% without any coding loss (0.00%). Additionally, VSP allows only one depth block fetch to minimize the memory complexity. However, the worst case memory bandwidth for VSP is only 50%. So, it is proposed to allow two depth block fetches for VSP to improve the coding efficiency. With this method, the worst case memory bandwidth is reportedly increased from 50% to 59%, but a coding gain of 0.05% can be achieved.

# Introduction

This contribution provides the memory complexity analysis for DBBP and VSP in 3D-HEVC [1] based on the method used in [2]. Table 1 shows the worst case of each coding tools in terms of external memory bandwidth. The analysis is done by comparing to the worst case of HEVC. The worst case of HEVC occurs when bi-predictive 8x8 PU (Predictive Unit) is applied.

Table 1. Analysis of external memory bandwidth in the worst case

| Coding Tool | Memory bandwidth (1x1 pattern) | Memory bandwidth (4x2 pattern) | Memory bandwidth (4x4 pattern) | Memory bandwidth (8x2 pattern) |
| --- | --- | --- | --- | --- |
| HEVC MC | 100% | 100% | 100% | 100% |
| DBBP | 120% | 125% | 123% | 128% |
| VSP | 50% | 66% | 61% | 81% |

As can be seen in Table 1, the worst case memory bandwidth of DBBP is 120% to 128% depending on used memory pattern since DBBP needs to fetch two depth blocks in addition to the motion compensation. So, the memory bandwidth for DBBP needs to be reduced. And the worst case memory bandwidth of VSP is only 50% to 81% since VSP allows only one depth block fetch.

# Proposed Methods

DBBP uses the depth-based disparity vector (DoNBDV) to derive a binary segmentation mask, so it needs to fetch two depth blocks; one is for deriving DoNBDV and the other is for generating the binary segmentation mask. To reduce the memory complexity of DBBP, two options of the simplification methods are proposed. The first option is to use NBDV instead of DoNBDV to reduce the number of depth block fetches. The second option is to disallow DBBP in 8x8 CU which is the worst case.

Currently, VSP allows only one depth block fetch to minimize the memory complexity and its worst case memory complexity is only 50% to 81%. Therefore, it is proposed to allow two depth block fetches for VSP to improve the coding efficiency. That is, DoNBDV is used instead of NBDV when finding a corresponding depth block used for 2nd DV derivation.

# Experimental Results

The simulation was carried out based on the common test conditions [3] and HTM 11 reference software. There are three test cases; using NBDV instead of DoNBDV in DBBP (Test 1), disallowing DBBP in 8x8 CU (Test 2) and using DoNBDV instead of NBDV in VSP (Test 3).

Table 2 shows the results of the method for using NBDV instead of DoNBDV in DBBP. In this method, there is a loss of 0.06% on both coded views and synthesized views.

Table 2. Comparison results between CTC anchor and Test 1

Table 3 shows the results of the method for disallowing DBBP in 8x8 CU. The proposed method provides no coding loss.

Table 3. Comparison results between CTC anchor and Test 2

Table 4 shows the results of the method for using DoNBDV instead of NBDV in VSP. The proposed method provides 0.1% and 0.2% bit-saving for dependent views, and 0.04% and 0.05% on coded and synthesized views, respectively.

Table 4. Comparison results between CTC anchor and Test 3

In this section, the memory complexity analysis for the proposed methods was also conducted.

Table 5 shows the memory bandwidth analysis for Test 1 and Test 2. The memory bandwidth of Test 1 is reduced by 10% to 14% with a coding loss of 0.06% and the memory bandwidth of Test 2 is reduced by 41% to 59% without any coding loss.

Table 5. Analysis of external memory bandwidth for DBBP

| Coding Tool | Memory bandwidth (1x1 pattern) | Memory bandwidth (4x2 pattern) | Memory bandwidth (8x2 pattern) | Coding gain for synthesized views |
| --- | --- | --- | --- | --- |
| DBBP (HTM) | 120% | 125% | 128% | n/a |
| DBBP (Test 1) | 110% | 113% | 114% | 0.06% |
| DBBP (Test 2) | 79% | 71% | 69% | 0.00% |

Table 6 shows the memory bandwidth analysis for Test 3. The memory bandwidth of Test 3 is increased by 9% to 13%, but it still has a less memory bandwidth than the HEVC in the worst case. And the proposed method provides 0.05% bit-saving for synthesized views.

Table 6. Analysis of external memory bandwidth for VSP

| Coding Tool | Memory bandwidth (1x1 pattern) | Memory bandwidth (4x2 pattern) | Memory bandwidth (8x2 pattern) | Coding gain for synthesized views |
| --- | --- | --- | --- | --- |
| VSP (HTM) | 50% | 66% | 81% | n/a |
| VSP (Test 3) | 59% | 78% | 94% | –0.05% |

# Conclusion

In this contribution, in order to reduce the memory bandwidth for DBBP, two options of the simplification methods are proposed. The second methods for DBBP can be reduced the worst case memory bandwidth by 41% to 59% without any coding loss. Additionally, to improve the coding performance for VSP, it is proposed to use DoNBDV instead of NBDV. The proposed method is increased by 9% to 13% with a coding gain of 0.05%. Therefore, it is recommended to adopt the proposed methods into next 3D-HEVC draft text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” JCT3V-H1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Mar. 2014, Valencia, ES.

[2] E. Alshina, E. François, “BoG Report on SHVC complexity assessment,” JCTVC-M0455, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Apr. 2013, Incheon, KR.

[3] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.