| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0078_WD3 |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Working Draft for JCT3V-I0078 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Working Draft. The changes are highlighted in green.

I.8.5.3.2.13  Derivation process for a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current prediction block.

Outputs of this process are

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

- the reference indices refIdxL0VSP and refIdxL1VSP ,

- the prediction list utilization flags predFlagL0VSP and predFlagL1VSP,

- the motion vectors mvL0VSP and mvL1VSP.

- The variable availableFlagVSP is set equal to 0, the variables predFlagL0VSP and predFlagL1VSP are set equal to 0, and the variables refIdxL0VSP and refIdxL1VSP are set equal to −1.

- For X in the range of 0 to 1, inclusive, the following applies:

  - For i in the range of 0 to NumRefPicsLX − 1, inclusive, the following applies:

    - When availableFlagVSP is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xPb ][ yPb ], the following applies:

      - The variable predFlagLXVSP, mvLXVSP, refIdxLXVSP, and availableFlagVSP are modified as specified in the following:

          - predFlagLXVSP = 1		(Error: Reference source not found-1)

          - mvLXVSP = MvRefinedDsipMvDisp[ xPb ][ yPb ]		(Error: Reference source not found-2)

          - refIdxLXVSP = i		(Error: Reference source not found-3)

          - availableFlagVSP = 1		(Error: Reference source not found-4)

      - The derivation process for the luma motion vectors for view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the reference index refIdxLXVSP, the disparity vector mvLXVSP, and the view order index RefViewIdx[ xPb ][ yPb ] as inputs.