| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0078_WD2 |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Working Draft for JCT3V-I0078 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Working Draft. The changes are highlighted in green.

I.7.3.8.5  Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && CuPredMode[ x0 ][ y0 ] != MODE_INTRA && log2CbSize > 3 ) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |
| … |  |