| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0079_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Working Draft for JCT3V-I0079 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee In Kwon Choi Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com ik21.choi@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Working Draft. The changes are highlighted in green.

I.8.5.3.2.1  Derivation process for luma motion vectors for merge mode

…

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

  - The derivation process for the base merge candidate list as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the luma location ( xPb, yPb ), the variables nCbS, nPbW, nPbH, and the partition index partIdx as inputs, and the output being a modified luma location ( xPb, yPb ), the modified variables nPbW and nPbH, the modified variable partIdx, the luma location ( xOrigP, yOrigP ), the variables nOrigPbW and nOrigPbH, the merge candidate list baseMergeCandList, the luma motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N, and the prediction list utilization flags predFlagL0N and predFlagL1N, with N being replaced by all elements of baseMergeCandList.

  - For N being replaced by A1, B1, B0, A0 and B2, the following applies:

    - If N is an element in baseMergeCandList, availableFlagN is set equal to 1.

    - Otherwise (N is not an element in baseMergeCandList), availableFlagN is set equal to 0.

  - Depending on iv_mv_pred_flag[ nuh_layer_id ] and DispAvailabilityIdc[ xPb ][ yPb ], the following applies:

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0 or DispAvailabilityIdc[ xPb ][ yPb ] is not equal to DISP_NONE, the flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH as inputs, and the output is assigned to the availability flags availableFlagIvMC, availableIvMCShift and availableFlagIvDC, the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC, predFlagLXIvMCShift and predFlagLXIvDC, and the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC (with X being 0 or 1, respectively).

  - Depending on view_synthesis_pred_flag[ nuh_layer_id ], DispAvailabilityIdc[ xPb ][ yPb ], and , dbbp_flag[ xPb ][ yPb ] the following applies:

    - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0, DispAvailabilityIdc[ xPb ][ yPb ] is equal to DISP_NONE, or dbbp_flag[ xPb ][ yPb ] is equal to 1, the flag availableFlagVSP is set equal to 0.

    - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, DispAvailabilityIdc[ xPb ][ yPb ] is not equal to DISP_NONE, and dbbp_flag[ xPb ][ yPb ] is equal to 0), the derivation process for availability of a view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma locations ( xPb, yPb ) and the variables nPbW and nPbH as inputs, and the outputs isare the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

…

  - The variable vspModeFlag is derived as specified in the following:

- vspModeFlag = (mergeCandIsVspFlag  	&&  !ic_flag  &&  
	( iv_res_pred_weight_idx  = = 0 )  &&  availableFlagVSP) ? mergeCandIsVspFlag : 0	(Error: Reference source not found-1)

  - If vspModeFlag is not equal to 0, derivation process for a view synthesis prediction merge candidate in subclause I.8.5.3.2.14 is invoked with the luma locations ( xPb, yPb ) and the variables nPbW and nPbH as inputs, and the outputs are the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list  utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

…

I.8.5.3.2.13  Derivation process for availability of a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

Outputs of this process are

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

The variable availableFlagVSP is set equal to 0.

- For X in the range of 0 to 1, inclusive, the following applies:

  - For i in the range of 0 to NumRefPicsLX − 1, inclusive, the following applies:

    - When availableFlagVSP is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xPb ][ yPb ], the following applies:

      - The variable availableFlagVSP is modified as specified in the following:

          - availableFlagVSP = 1		(Error: Reference source not found-2)

I.8.5.3.2.14  Derivation process for a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current prediction block,

- the variable vspMode.

Outputs of this process are

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

- the reference indices refIdxL0VSP and refIdxL1VSP ,

…

- For X in the range of 0 to 1, inclusive, the following applies:

  - For i in the range of 0 to NumRefPicsLX − 1, inclusive, the following applies:

    - When availableFlagVSP is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xPb ][ yPb ], the following applies:

      - The variable predFlagLXVSP, mvLXVSP, refIdxLXVSP, and availableFlagVSP are modified as specified in the following:

          - predFlagLXVSP = 1		(Error: Reference source not found-3)

          - mvLXVSP = MvDisp[ xPb ][ yPb ]		(Error: Reference source not found-4)

          - refIdxLXVSP = i		(Error: Reference source not found-5)

          - availableFlagVSP = 1		(Error: Reference source not found-6)

      - The derivation process for the luma motion vectors for view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the reference index refIdxLXVSP, the disparity vector mvLXVSP, vspMode, and the view order index RefViewIdx[ xPb ][ yPb ] as inputs.

I.8.5.3.2.13.1  Derivation process for luma motion vectors for view synthesis prediction merge candidate

- Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

- a prediction list indication X,

- a reference index refIdx,

- a disparity vector mvDisp,

- a variable vspMode,

…

The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xPb, yPb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPbW , the variable nPbH the variable Mode, the variable partIdc equal to 2 as inputs, and the output is the array disparitySamples of size (nPbW)x(nPbH) and the flag horSplitFlag.

…

I.8.5.3.2.17   Derivation process for a view synthesis prediction flag

…

- The variable mergeCandIsVspFlag is derived as specified in the following:

- If N is equal to VSP, mergeCandIsVspFlag is set equal to 21,

- Otherwise, if N is equal to A1, B1, B0, A0, or B2, the following applies:

  - The luma position ( xN, yN ) is specified in Error: Reference source not found depending on N.

  - If one of the following conditions is true, the variable mergeCandIsVspFlag is set equal to (VspModeFlag[ xN ][ yN ] ? (N is equal to A1 or B1 ? 1 : N is equal to B0 ? 2 : 3) : 0).

    - N is equal to A1 or A0

    - N is equal to B0, B1, or B2 and ( yN  >>  Log2CtbSizeY ) is equal to ( yCb >> Log2CtbSizeY )

  - Otherwise, ,mergeCandIsVspFlag is set equal to 0.

- Otherwise, mergeCandIsVspFlag is set equal to 0.

…

I.8.5.5.2  Derivation process for a disparity sample array

- Inputs to this process are:

…

- a variable vspMode.

…

The array disparitySamples is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH ) − 1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW ) − 1 ), inclusive, the following applies:

    - The variable maxDep is set equal to −1 and modified as specified in the following:

xSubB = sBx * nSubBlkW
			ySubB = sBy * nSubBlkH
			xP0 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB )
			yP0 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB ) 
			xP1 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB + nSubBlkW − 1 )
			yP1 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB + nSubBlkH − 1 )
			if(vspMode = = 1) {
				maxDep = ( refDepPels[ xP0 ][ yP0 ] + refDepPels[ xP0 ][ yP1] + refDepPels[ xP1][ yP0 ] +
				          refDepPels[ xP1][ yP1 ] ) >> 2
			}
			else if(vspMode = = 2) {
				maxDep = Max( maxDep, refDepPels[ xP0 ][ yP0 ] )
				maxDep = Max( maxDep, refDepPels[ xP0 ][ yP1] )
				maxDep = Max( maxDep, refDepPels[ xP1][ yP0 ] )
				maxDep = Max( maxDep, refDepPels[ xP1][ yP1 ] )
			}
			else if(vspMode = = 3) {
				xP2 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB + nSubBlkW / 2 – 1 )
				yP2 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB + nSubBlkH /2 – 1 )
				maxDep = refDepPels[ xP2 ][ yP2 ];
			}

…