| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0080 |
| --- | --- |

| Title: | Parameter Derivation for Illumination Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current IC process, if the reference position is equal to or less than zero then neighboring samples of the reference block are not used to derive the IC parameter, and if the reference position is greater than zero then neighboring samples are always used and the proper description of this is omitted in the draft text. Therefore, this contribution proposes to clip the position of neighboring samples and to remove the useless condition from the IC parameter derivation process. This proposed method reportedly provides no coding loss. Additionally, even though the position of the top-left pixel of the reference block is in fractional sample position, the nearest integer samples are used to derive the IC parameter. To obtain more accurate IC parameter, it is proposed to use the fraction samples derived by the bi-linear interpolation. This proposed method reportedly provides 0.1% bit-saving for coded views.

# Introduction

Figure 1. Availability of the neighboring samples depending on the position of reference block

In the current IC process [1], if the reference position is equal to or less, then zero then neighboring samples of the reference block are not used for the IC parameter derivation, and if the reference position is greater than zero then neighboring samples are always used for the IC parameter derivation as shown in Figure 1, and the proper description of this is omitted in the 3D-HEVC draft text.

Moreover, even though the position of the top-left pixel of the reference block is in fractional sample position, the nearest integer samples are used to derive the IC parameter.

# Proposed Methods

## Clipping Neighboring Sample Position

In this contribution, it is proposed to clip the position of neighboring samples of the reference block and to remove the useless condition, which the neghboring samples are not used if the reference position is equal to or less than zero, from the IC parameter derivation process so that the neighboring samples are always used for the IC parameter derivation. The derivation process for illumination compensation mode availability and parameters in 3D-HEVC draft text can be modified as below.

I.8.5.3.3.6.1 Derivation process for illumination compensation mode availability and parameters … - 3. .If puIvPredFlagLX is equal to 0, the variable puIcFlagLX is set equal to 0, otherwise (puIvPredFlagLX is equal to 1 ) the following applies: - The luma location (xRLX, yRLX) specifying the top-left sample of the reference block in refPicLX is derived as - xRLX = xC + ( ( mvLX[ 0 ] + ( cIdx ? 4 : 2 ) ) >> ( 2 + ( cIdx ? 1 : 0 ) ) ) (Error: Reference source not found-1) - yRLY = yC + ( ( mvLX[ 1 ] + ( cIdx ? 4 : 2 ) ) >> ( 2 + ( cIdx ? 1 : 0 ) ) ) (Error: Reference source not found-2) - The variable availFlagAboveRowLX specifying whether the above neighbouring row samples of the current block and the reference block are available is derived as specified in the following: - availFlagAboveRowLX = ( yRLX > 0 ) && availFlagCurAboveRow (Error: Reference source not found-3) - The variable availFlagLeftColLX specifying whether the left neighbouring column samples of the current block and the reference block are available is derived as specified in the following: - availFlagLeftColLX = ( xRLY > 0 ) && availFlagCurLeftCol (Error: Reference source not found-4) - The variable puIcFlagLX is derived as follows: - puIcFlagLX = availFlagAboveRowLX ¦ ¦ availFlagLeftColLX (Error: Reference source not found-5) … - The lists curNeighSampleListLX and refNeighSampleListLX specifying the neighbouring samples in the current picture and the reference picture are derived as specified in the following: - The variable numNeighSamplesLX specifying the number of elements in curNeighSampleListLX and in refNeighSampleLX is set equal to 0. - The variable leftNeighOffLX specifying the offset of the left neighbouring samples in curNeighSampleListLX and refNeighSampleLX is derived as - leftNeighOffLX = availFlagAboveRowLX ? 0 : nCS (Error: Reference source not found-6) - For i ranging from 0 to nCS − 1, inclusive the following applies: - When availFlagAboveRowLX is equal to 1 the following applies: - xP = Clip3( 0, pic_width_in_luma_samples – 1, xRLX + i ) - yP = Clip3( 0, pic_height_in_luma_samples – 1, yRLY − 1 ) - curNeighSampleListLX[ i ] = curRecSamples[ xC + i][ yC − 1 ] (Error: Reference source not found-7) - refNeighSampleListLX[ I ] = refRecSamples[ xP xRLX + i ][ yPyRLY − 1 ] (Error: Reference source not found-8) - numNeighSamplesLX += 1 (Error: Reference source not found-9) - When availFlagLeftColLX is equal to 1 the following applies: - xP = Clip3( 0, pic_width_in_luma_samples – 1, xRLX – 1 ) - yP = Clip3( 0, pic_height_in_luma_samples – 1, yRLY + i) - curNeighSampleListLX[ i + leftNeighOffLX ] = curRecSamples[ xC − 1][ yC + i ] (Error: Reference source not found-10) - refNeighSampleListLX[ i + leftNeighOffLX ] = refRecSamples[ xPxRLX−1][ yP yRLY + i ] (Error: Reference source not found-11) - numNeighSamplesLX += 1 (Error: Reference source not found-12) …

## Allowing Fractional Sample Position for Neighboring Sample

In this contribution, to obtain more accurate IC parameter, it is proposed to use the fraction samples derived by the bi-linear interpolation.

Figure 2. Example of fractional sample position

When the fractional sample position is represented as shown in Figure 2, the fractional sample can be calculated by following equations (NBL: luma sample, NBC: chroma sample).

NBL = ((4 − xFracL) * (4 − yFracL) * A + xFracL * (4 − yFracL) * B + (4 − xFracL) * yFracL * C + xFracL * yFracL * D + 8 ) >> 4 NBC = ((8 − xFracC) * (8 − yFracC) * A + xFracC * (8 − yFracC) * B + (8 − xFracC) * yFracC * C + xFracC * yFracC * D + 32 ) >> 6

The worst case complexity in terms of the number of multiplication is conducted. The worst case of HEVC is 57 multiplications per pixel and occurs when the bi-predictive 8x8 PU is applied. The worst case of the current IC (59.7 per pixel) is 105% compared to the worst case of HEVC since the IC parameter derivation needs 42 multiplications and applying the illumination compensation to the reference block needs 128 multiplications. And the worst case of the proposed method (60.7 per pixel) is 106% compared to the worst case of HEVC since the IC parameter derivation additionally needs 64 multiplications. Table 1 shows the summary of the complexity comparison in terms of multiplication.

Table 1. Complexity comparison in terms of multiplication

|  | Multiplications per pixel | Compared to HEVC |
| --- | --- | --- |
| HEVC MC | 57.0 | 100% |
| Current | 59.7 | 105% |
| Proposed | 60.7 | 106% |

# Experimental Results

The simulation was carried out based on the common test condition [2] and HTM 11 reference software. There are two test cases; clipping the neighboring samples position (Test 1), allowing fractional sample position (Test 2), and the combining of Test 1 and Test 2 (Test 3).

Table 2 shows the results of clipping the neighboring samples position. The proposed method provides no coding loss.

Table 2. Comparison results between CTC anchor and Test 1

Table 3 shows the results of allowing fraction sample position for the neighboring samples. The proposed method provides 0.2% bit-saving for dependent views, and 0.1% bit-saving for the coded views.

Table 3. Comparison results between CTC anchor and Test 2s

Table 4 shows the combining results of Test 1 and Test 2. The proposed methods provide 0.2% bit-saving for dependent views, and 0.1% bit-saving for the coded views.

Table 4. Comparison results between CTC anchor and Test 3

# Conclusion

In this contribution, it is proposes to clip the position of neighboring samples and to remove the useless condition from the IC parameter derivation process to have a design consistency. Additionally, it is also proposed to use the fraction samples derived by the bi-linear interpolation in order to obtain more accurate IC parameter. The proposed methods provide 0.1% bit-saving for the coded views. Therefore, it is recommended to adopt the proposed methods into next 3D-HEVC draft text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” JCT3V-H1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Mar. 2014, Valencia, ES.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.