| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0081_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Working Draft for JCT3V-I0081 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Working Draft. The changes are highlighted in green.

I.8.5.3.2.10  Derivation process for inter-view merge candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0 and mpi_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagIvMC, availableFlagIvMCShift and availableFlagIvDC specifying whether the inter-view merge candidates are available,

- the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC,

- the prediction list utilization flags predFlagLXIvMC, predFlagLXIvMCShift and predFlagLXIvDC,

- the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC.

The availability flags availableFlagIvMC, availableFlagIvMCShift and availableFlagIvDC are set equal to 0, and for X in the range of 0 to 1, inclusive, the variables predFlagLXIvMC, predFlagLXIvMCShift, predFlagLXIvDC are set equal to 0, the variables refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC are set equal to −1,  and both components of mvLXIvMC, mvLXIvMCShift and mvLXIvDC are set equal to 0.

When ic_flag is equal to 0, the temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - Depending on PartMode, the following applies:

    - If PartMode is equal to PART_2Nx2N, the derivation process for inter layer predicted sub prediction block motion vector candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ] and the disparity vector ( iv_res_pred_weight_idx ) ? MvDisp[ xPb ][ yPb ]  : MvRefinedDisp[ xPb ][ yPb ] ) as inputs, and the outputs are, with X being in the range of 0 to 1, inclusive, the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

    - Otherwise (PartMode is not equal to PART_2Nx2N), the derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector ( iv_res_pred_weight_idx ) ? MvDisp[ xPb ][ yPb ]  : MvRefinedDisp[ xPb ][ yPb ] ) as inputs, and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

      - availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(Error: Reference source not found-1)

      - predFlagL0IvMC = availableFlagL0IvMC		(Error: Reference source not found-2)

      - predFlagL1IvMC = availableFlagL1IvMC		(Error: Reference source not found-3)

When DepthFlag is equal to 0 and ic_flag is equal to 0, the shifted temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies:

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector ( iv_res_pred_weight_idx ) ? MvDisp[ xPb ][ yPb ]  : MvRefinedDisp[ xPb ][ yPb ] ) + ( nPbW *2 + 4, nPbH *2 + 4 ) as inputs, and the outputs are the flag availableFlagLXIvMCShift, the motion vector mvLXIvMCShift and the reference index refIdxLXIvMCShift.

  - The availability flag availableFlagIvMCShift, and the prediction utilization flags predFlagL0IvMCShift and predFlagL1IvMCShift are derived by

      - availableFlagIvMCShift = availableFlagL0IvMCShift | | availableFlagL1IvMCShift	(Error: Reference source not found-4)

      - predFlagL0IvMCShift = availableFlagL0IvMCShift	(Error: Reference source not found-5)

      - predFlagL1IvMCShift = availableFlagL1IvMCShift	(Error: Reference source not found-6)

When DepthFlag is equal to 0, the disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies:

    - The derivation process for a disparity inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector ( iv_res_pred_weight_idx ) ? MvDisp[ xPb ][ yPb ]  : MvRefinedDisp[ xPb ][ yPb ] ), and the prediction list indication X, as inputs, and the outputs are the flag availableFlagLXIvDC, the motion vector mvLXIvDC, and the reference index refIdxLXIvDC.

  - The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

      - availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC 	(Error: Reference source not found-7)

      - predFlagL0IvDC = availableFlagL0IvDC		(Error: Reference source not found-8)

      - predFlagL1IvDC = availableFlagL1IvDC		(Error: Reference source not found-9)