| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0081 |
| --- | --- |

| Title: | 3D-CE1 related: Reduction of Worst Case Memory Bandwidth for ARP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The inter-view candidates use the DoNBDV, which is derived from the corresponding depth block, so a depth block fetch is always needed for the every merge candidates including the candidate coded with ARP. Thus, the memory bandwidth for ARP in the worst case is reportedly 132% to 172% depending on the used memory pattern. Therefore, in this contribution, to reduce the worst case memory bandwidth for ARP, it is proposed to use NBDV instead of DoNBDV for the inter-view merge candidates when ARP is enabled so that the depth memory fetch is not needed anymore for ARP in the worst case. So, the worst case memory bandwidth is reportedly reduced by 10% to 14% as an amount of bandwidth used for accessing depth block. The proposed method reportedly provides a minor coding gain of 0.03% on both coded and synthesized views.

# Introduction

In the current 3D-HEVC [1], the inter-view candidates such as IVMC (Inter-View Motion vector Candidate), IVDC (Inter-View Disparity Candidate), shifted IVMC and shifted IVDC use the DoNBDV, which is derived from the corresponding depth block, and the availability of IVMC, which is the first candidate in the merge list, is decided by the DoNBDV and the IVDC uses DoNBDV as its motion parameters, so a depth block fetch is always need for the every merge candidates including the candidate coded with ARP. Thus, the worst case memory bandwidth for ARP is 132% to 172% depending on the used memory pattern when compared to the worst case of HEVC. The worst case of HEVC occurs when the bi-predictive 8x8 PU is applied. Table 1 shows the worst case memory bandwidth of ARP.

Table 1. Worst case memory bandwidth for ARP

| Memory Pattern | Memory Bandwidth (compared to worst case of HEVC) |
| --- | --- |
| 1x1 | 132% |
| 4x2 | 148% |
| 8x2 | 172% |

# Proposed Method

Therefore, it is proposed to use NBDV instead of DoNBDV for the inter-view merge candidates when the ARP is enabled so that depth memory fetch is not needed anymore for ARP in the worst case. With the proposed method, the worst case memory bandwidth for ARP can be reduced by 10% to 14% as an amount of bandwidth used for accessing depth block. Table 2 shows the comparison the worst case memory bandwidth between the current and proposed method.

Table 2. Comparison the worst case memory bandwidth between the current and proposed method

| Memory Pattern | Memory Bandwidth for ARP | Memory Bandwidth for Proposed |
| --- | --- | --- |
| 1x1 | 132% | 122% |
| 4x2 | 148% | 135% |
| 8x2 | 172% | 158% |

# Experimental Results

The simulation was carried out based on the common test condition [3] and HTM 11 reference software. Table 3 shows the results of the proposed method. The proposed method provides 0.1% bit-saving for dependent views, and the 0.03% bit-saving for coded and synthesized views in spite of the simplification.

Table 3. Comparison results between CTC anchor and proposed method

# Conclusion

In this contribution, it is proposed to use NBDV instead of DoNBDV for the inter-view candidates to reduce the worst case memory bandwidth for ARP. With the proposed method, the worst case memory bandwidth for ARP can be reduced by 10% to 14%. And also the proposed method provides 0.03% bit-saving on both coded and synthesized views in spite of the simplification. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC draft text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” JCT3V-H1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Mar. 2014, Valencia, ES.

[2] E. Alshina, E. François, “BoG Report on SHVC complexity assessment,” JCTVC-M0455, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Apr. 2013, Incheon, KR.

[3] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.