| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0082 |
| --- | --- |

| Title: | 3D-CE2: Residual DC quantization in intra SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

DLT is used to reduce the magnitude of the residual DC for the general modes in intra SDC. DLT signals  an index instead of the DC value. To reconstruct the residual DC in a decoder side, DC of the predicted pixels should be first calculated and then re-mapping from the DLT index to the value is performed. In order to simplify these operation, the proposed method quantizes the residual DC with one shift operation to reduce the magnitude of the residual DC, instead of using DLT. The results shows that the proposed method achieves the coding gain of -0.1% and removes the DC calculation and re-mapping processes.

# Proposed Method

The residual DC of the general modes in intra SDC is encoded with DLT indexes, as follow:

residual = DLT[original DC] – DLT[predicted DC]

Since the magnitude of the intra residual DC is relatively high, the DLT-based coding can reduce its size and results in the depth bit rate saving. However, To reconstruct the residual DC in a decoder side, DC of the predicted pixels should be first calculated and then re-mapping from the DLT index to the value is performed, which is little bit complicated.

In order to improve the coding performance at the same time simplify the process above, the proposed method performs the quantization for the residual DC. It quantizes the DC difference between the original and predicted pixels with a simple shift operation as follows.

residual = (original DC – predicted DC) >> residualQp

The residual quantization value (residualQp) is set to the maximum value between a default quantization value signalled in VPS (defaultQp) and a value derived from the DLT information (derivedQp).

residualQp = Max(defaultQp, derivedQp)

derivedQp is calculated as follow:

derivedQp=DLT[numDepth−1]−DLT[0]numDepth+0.5

where numDepth is the number of valid depth pixel values. Usually, DLT with an index equal to 0 stores the mimum depth value, whereas DLT with the index equal to numDepth-1 has the maximum depth value. Hence, if the range of the valid depth values and the number of them are given, we can know how much can be quantized. When defaultQP is equal to 0, as in the equations, the residual quantization parameter depends on the DLT information only.

At a decoder side, a reverse process is performed with the shift operation as follows.

residual <<= residualQp

# Test Results

Test results of the proposed method are summarized in Tables 1 and 8.

Tests 1 and 2 are the results when DLT is applied to all the sequence by removing a threshold 128.

Table 1. Performance of the DLT extension in CTC

Table 2. Performance of the DLT extension in all intra condition

Tests 3 and 4 are the results when the residual quantization is implemented at an encoder only by using the existing DLT function (DLT quantization). When DLT is not constructed by the threshold, it stores every even depth pixel value as an index increases from 0 to 127.

Table 3. Performance of the DLT quantization in CTC

Table 4. Performance of the DLT quantization in all intra condition

Tests 5 and 6 are the results when the proposed residual quantization with defaultQp equal to 0 is applied.

Table 5. Performance of the proposed method in CTC (defaultQp=0)

Table 6. Performance of the proposed method in all intra condition (defaultQp=0)

Tests 7 and 8 are the results when the proposed residual quantization with defaultQp equal to 1 is applied.

Table 7. Performance of the proposed method in CTC (defaultQp=1)

Table 8. Performance of the proposed method in all intra condition (defaultQp=1)

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can improve the coding performance and simplify the DC calculation and re-mapping processes. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).