| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0083 |
| --- | --- |

| Title: | 3D-CE2 related: Allowing zero DC candidate in inter SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

At the previous meeting, multiple DC candidates were adopted to improve the performance of the inter SDC (G0101). However, the zero DC candidate was not adopted due to the additional branch in merge mode in WD (G0058). To figure out the problem and for the design generality (that covers the zero DC candidate as well), the proposed method allows the zero candidate. It can be represented by replacing or deleting one condition in WD. Results demonstrate that the proposed method does not affect the coding performance a lot.

# Proposed Method

In the current 3D-HTM, to find more the accurate DC for all the residual, inter SDC checks five different candidates, which consist of DC, DC-1, DC+1, DC-2, and DC+2, and decides the best one through RDO. However, when the DC candidate is equal to 0, it is not performed. For the design generality (that covers the zero DC candidate as well), the proposed method allows the zero DC candidate in AMVP only.

Fig. 1. Multiple DC candidates

Option 1> Zero DC is included in the multiple DC candidates and a flag indicating whether the residual is equal to 0 or not is always signaled.

Option 2> Zero DC is included in the multiple DC candidates and a flag indicating whether the residual is equal to 0 or not is signaled. The flag is not transmitted only when the merge flag is equal to 1 because zero DC in merge mode means skip mode.

# Proposed Text

Option 1>

if( cuDepthDcPresentFlag ) {

pbOffset = ( PartMode = = PART_NxN && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ? ( nCbS / 2 ) : nCbS

for( j = 0; j < nCbS; j = j + pbOffset )

for( k = 0; k < nCbS; k = k + pbOffset )

if( DmmFlag[ x0 + k ][ y0 + j ] ¦ ¦ sdc_flag[ x0 ][ y0 ] ) {

depth_dc_flag[ x0 + k ][ y0 + j ]

dcNumSeg = DmmFlag[ x0 + k ][ y0 + j ] ? 2 : 1

if( depth_dc_flag[ x0 + k ][ y0 + j ] )

for( i = 0; i < dcNumSeg; i ++ ) {

depth_dc_abs[ x0 + k ][ y0 + j ][ i ]

if ( depth_dc_abs[ x0 + k ][ y0 + j ][ i ] > 0 )

depth_dc_sign_flag[ x0 + k ][ y0 + j ][ i ]

}

}

}

Option 2>

if( cuDepthDcPresentFlag ) {

pbOffset = ( PartMode = = PART_NxN && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ? ( nCbS / 2 ) : nCbS

for( j = 0; j < nCbS; j = j + pbOffset )

for( k = 0; k < nCbS; k = k + pbOffset )

if( DmmFlag[ x0 + k ][ y0 + j ] ¦ ¦ sdc_flag[ x0 ][ y0 ] ) {

if(!merge_flag[x0][y0] ) {

depth_dc_flag[ x0 + k ][ y0 + j ]

dcNumSeg = DmmFlag[ x0 + k ][ y0 + j ] ? 2 : 1

} else

dcNumSeg = 1

if( depth_dc_flag[ x0 + k ][ y0 + j ] )

for( i = 0; i < dcNumSeg; i ++ ) {

depth_dc_abs[ x0 + k ][ y0 + j ][ i ]

if ( depth_dc_abs[ x0 + k ][ y0 + j ][ i ] > 0 )

depth_dc_sign_flag[ x0 + k ][ y0 + j ][ i ]

}

}

}

# Test Results

A test result of the proposed method is summarized in Tables 1 (Option 1) and 2 (Option 2). The result demonstrates that the proposed method does not affect the coding performance a lot.

Table 1. Performance of the proposed method (Option 1)

Table 2. Performance of the proposed method (Option 2)

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can cover the zero DC candidate in inter SDC. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).