| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0084 |
| --- | --- |

| Title: | 3D-CE2 related: Fast SDC DC offset decision |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

At the previous meeting, multiple DC candidates were adopted to improve the performance of inter and intra SDC. At an encoder, SDC checks five different candidates, which consist of DC, DC-1, DC+1, DC-2, and DC+2, and decides the best one through exhaustive comparison. In order to reduce the encoding complexity, the proposed method introduces a fast SDC DC offset decision method. Results demonstrate that the proposed method save the encoding time without coding loss.

# Proposed Method

In the current 3D-HTM, to find more the accurate DC for all the residual, inter and intra SDC checks five different candidates, which include DC, DC-1, DC+1, DC-2, and DC+2, and decides the best one through the comparison of their RD costs. It results in the significant complexity increase.

Fig. 1. Multiple DC candidates

This contribution proposes to terminate the DC decision process in several conditions:

1. When RD cost of DC is less than both RD costs of DC-1 and DC+1 scaled by 0.9, DC-2 and DC+2 are not tested.

2. When RD cost of DC is less than that of DC-1 as well as RD cost of DC+1 is less than that of DC, DC-2 is not tested.

3. When RD cost of DC is less than that of DC+1 as well as RD cost of DC-1 is less than that of DC, DC+2 is not tested.

# Test Results

Test results of the proposed method are summarized in Tables 1 and 2. The results demonstrate that the proposed method does not affect the coding performance a lot.

Table 1. Performance of the proposed method in CTC

Table 2. Performance of the proposed method in all intra condition

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can reduce the encoding complexity without the coding loss. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).