| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0085 |
| --- | --- |

| Title: | 3D-CE2 related: Separate enabling flag for intra coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Intra coding tools such as intra SDC and DMM (DMM1 and DMM4) are enabled and disabled by a single flag ‘vps_depth_modes_flag’, while inter coding tools are individually selectable by their corresponding flags. Even though intra SDC is on and DMM is off, a DMM flag must be signalled in each CU, which is completely redundant. In order to solve the design inconsistency and the redundant syntax signaling, this contribution proposes to define a separate flag for DMM and intra SDC. In addition, we suggest changing the names of DMM1 and DMM4 into DMM and IVP (Intra-view prediction), respectively.

# Separate Enabling Flag for Intra SDC and DMM

This contribution proposes separate enabling flags for intra coding tools such as intra SDC, DMM1, and DMM4, based on several problems in the current design. Identified problems include:

A. Design inconsistency between intra and inter coding tool flags

- Intra SDC and DMM have been developed to improve the depth coding efficiency. Intra SDC is enabled and disabled by vps_depth_modes_flag, which also controls DMM. In the case of inter coding tools such as MPI, VSP, IVMV, ARP, IC, and DBBP, they are individually selectable by their corresponding flags in VPS.

B. Redundant syntax signalling

- Even though Intra SDC is on and DMM is off in the config., a DMM flag must be signalled at each CU, which is completely redundant.

In addition, DMM1 and DMM4 are the coding tools of the different concept, so this contribution suggests changing the names of DMM1 and DMM4 into DMM and IVP, respectively.

# Test Results

A test result of the proposed method is summarized in Table 1. Tables 1 shows that the separate enabling flag affects no coding performance.

Table 1. Performance of separate enabling flag

# Conclusion

The proposed method signals the separate enabling flags in VPS for the intra coding tools such as intra SDC, DMM1, and DMM4. By integrating the proposed method on the current 3D-HTM, it can improve the current design. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).