| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0086 |
| --- | --- |

| Title: | Simplification of shift DV candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In 3D-HEVC, shift DV is used in merge mode. In order to find the shift DV, all the neighboring blocks are searched whether they have DV or not. After the DV searching process, it is generated by adding four to a horizontal component of the existing DV. However, during several meetings it was reported that the shift DV provides the small coding gain. Therefore, this contribution proposes to remove and simplify the shift DV as the first and second options, respectively. The third option removes a pruning process of DV on top of the second option. Based on WD, both the options reduce 30 condition checks in the worst case by removing a subclause H.8.5.3.2.15 (DV searching process). The experiment shows the negligible coding loss in the first option and no coding loss in the second option.

# Proposed Method

Currently, there are two additional shift candidates. One is the shift IVMV and the other is the shift DV. The shift IVMV has small gain (about -0.1%), but the shift DV have very minor gain (less than -0.1%). Hence, this contribution proposes to remove the shift DV (Option1) or simplify the shift DV by removing the DV searching process. The DV searching process is conducted by searching the neighboring blocks whether they have DV or not and requires 30 conditioning checks in the worst case.

Option1 simply removes the shift DV.

Option2 employs the DV and shift DV candidates only when the horizontal component of DoNBDV is not equal to zero. For example, the DV candidate is set to DoNBDV when its horizontal component is not equal to zero. If the DV candidate is available, the shift DV is generated by adding four to the horizontal DV component without the DV searching process.

Option3 removes the pruning process of DV on top of the option2.

# Test Results

Test results of the proposed method are summarized in Tables 1 to 3.

Tables 1 to 3 represent the coding performances of the options 1 to 3, respectively. As illustrated in the tables, almost no coding loss is observed (less than 0.1%).

Table 4 shows the benefit of the proposed method in terms of the simplification.

Table 1. Performance of the option1

Table 2. Performance of the option2

Table 3. Performance of the option3

Table 4. Analysis in terms of simplification

|  | Current | Option1 | Option2 | Option3 |
| --- | --- | --- | --- | --- |
| # of condition checks in the DV searching process | 30 | 0 | 1 | 1 |
| # of the DV pruning pocess | 2 | 2 | 2 | 0 |

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can simplify the process of the 3D Merge mode list construction. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).