| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0087 |
| --- | --- |

| Title: | Simplification of 3D Merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim Yichen Zhang and Lu Yu | Email: | jinyoung79.lee@samsung.com felixzyc@gmail.com yul@zju.edu.cn |
| Source: | Samsung Electronics Co. Ltd. and Zhejiang University |  |  |

_____________________________

# Abstract

At Geneva meeting, the simple Merge candidate list construction was adopted (F0093). It introduced a simple and low-complexity way to implement the Merge list construction module for 3D-HEVC by re-using the existing HEVC Merge list construction. As follow-up of H0097, this contribution proposes to remove pruning processes, VSP inheritance, availability checking processes of HEVC candidates such as candidate A0, A1, B0, B1, and B2 in the 3D Merge list construction. As a result, it can minimize the interaction between the HEVC candidates and 3D candidates. The result demonstrates that the proposed method has small coding loss with about 0.1%. In terms of simplification, the availability of the candidate A0, A1, B0, B1, and B2 is not needed to be checked in the Merge list construction, the number of pruning processes is reduced from 8 to 0, and the VSP flag storage is not required. In addition, this contribution proposes several alternative options for simplification.

# Proposed Method

This contribution proposed four different options to simplify the Merge list construction.

(Options 1 and 2) In the Merge list construction, five and three pruning processes are performed in the texture and depth coding, respectively, as shown in Table 1. It is observed that some pruning processes have no coding gain at all. Based on the evaluation, the proposed method performs pruning processes only between IVMV-A1 in texture coding and MPI-A1 in depth coding in option 1, and no pruning in option 2, respectively.

Table 1. Proposed pruning process in opions 1 and 2

| Texture | Depth | Option 1 | Option 2 |
| --- | --- | --- | --- |
| N/A | MPI-IVMV | Removed | Removed |
| IVMV-A1 | MPI-A1 | Allowed | Removed |
| IVMV-B1 | MPI-B1 | Removed | Removed |
| A1-DV | N/A | Removed | Removed |
| B1-DV | N/A | Removed | Removed |
| IVMV-Shift IVMV | N/A | Removed | Removed |

(Options 3 and 4) In the Merge list construction, it is required to know what candidates are in the HEVC list and where the candidates are placed in the list. However, in the proposed method, it is not required to know the information of the already constructed HEVC list at all, which means that it can minimize the interaction between the HEVC candidates and 3D candidates.

First of all, the proposed method does not perform pruning processes, as it is in option 2

Second, the proposed method does not require checking the availability of A0, A1, B0, B1, and B2 in the 3D Merge list construction. For example, the HEVC Merge list is already constructed and five candidates are decided in the list. In the texture coding, firstly, IVMV is added in the 3D Merge list. In depth coding, MPI is placed ahead of DDD and IVMV. After the insertion of IVMV, the candidates firstly and secondly placed in the HEVC list are added. Whether A1 and B1 exist in the list or not is not necessary to be known here. Next, DV, Shift IVMV, and Shift DV, are added in order. Finally, the remaining ones in the HEVC list are added. Tables 2 and 3 illustrate the proposed 3D Merge list construction in the texture and depth coding, respectively.

Table 2. Proposed 3D Merge list construction (Texture)

| Current | Proposed |
| --- | --- |
| IVMV | IVMV |
| A1 | First candidate in the HEVC list |
| B1 | Second candidate in the HEVC list |
| B0 | DV |
| DV | Shift IVMV |
| VSP | Shift DV |
| A0 | Third candidate in the HEVC list |
| B2 | Forth candidate in the HEVC list |
| Shift IVMV | Fifth candidate in the HEVC list |
| Shift DV |  |
| Temporal |  |

Table 3. Proposed 3D Merge list construction (Depth)

| Current | Proposed |
| --- | --- |
| MPI | MPI |
| DDD | DDD |
| IVMV | IVMV |
| A1 | First candidate in the HEVC list |
| B1 | Second candidate in the HEVC list |
| B0 | Third candidate in the HEVC list |
| A0 | Forth candidate in the HEVC list |
| B2 | Fifth candidate in the HEVC list |
| Temporal |  |

Finally, in option 3, the proposed method removes VSP flags which indicate whether the neighboring blocks are coded by VSP mode or not, except for A1, only when neighboring block A1 is VSP-coded, a default VSP candidate is inserted to the fourth position in the Merge list. In option 4, the proposed method removes all the VSP flags. The candidate corresponding to a VSP coded neighboring block is treated as a DCP candidate. As a result, the VSP flag storage can be totally removed in option 4. Instead, the VSP candidate is inserted to the fourth position in the Merge list, i.e., with merge index equaling to 3, the coding mode is always VSP (when IC and ARP is not applied to current PU).

Consequently, option 3 allows the VSP inheritance of A1 and fixed VSP. Option 4 employs the fixed VSP only.

In summary,

Option 1 : Only A1-IVMV/MPI pruning

Option 2 : No pruning

Option 3 : No pruning + Only A1 VSP inheritance + No availability checking of the neighboring blocks

Option 4 : No pruning + No VSP inheritance + No availability checking of the neighboring blocks

# Test Results

Test results of the proposed method are summarized in Tables 4 to 7. Even though the proposed method simplifies the Merge list construction, only small coding loss is observed (about 0.1%). Table 8 shows the benefit of the proposed method in terms of simplification.

Table 4. Performance of the proposed method (Option 1)

Table 5. Performance of the proposed method (Option 2)

Table 6. Performance of the proposed method (Option 3)

Table 7. Performance of the proposed method (Option 4)

Table 8. Analysis in terms of simplification

|  | Availability Check | Pruning | VSP flag |
| --- | --- | --- | --- |
| Current | A0, A1, B0, B1, and B2 | 8 | A0, A1, B0, B1, and B2 |
| Option1 | A0, A1, B0, B1, and B2 | 2 | A0, A1, B0, B1, and B2 |
| Option2 | A0, A1, B0, B1, and B2 | 0 | A0, A1, B0, B1, and B2 |
| Option3 | A1 | 0 | A1 |
| Option4 | N/A | 0 | N/A |

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can simplify the process of the 3D Merge mode list construction. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).