| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0088 |
| --- | --- |

| Title: | Simplification of filtering in DBBP and its extension to PU boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Mikhail Mishurovskiy, Yongjin Cho, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

In DBBP, an arbitrarily shaped block partitioning for a collocated texture block is derived from a binary segmentation mask computed by its corresponding depth block. Two partitions are motion-compensated and then merged based on the mask. Finally, both luminance and chrominance components of boundary pixels in the merged block is filtered. In order to simplify the filtering process, this contribution proposes to remove the filtering in the chrominance. Also, the contribution proposes to extend the filtering into PU boundaries in general partitioned modes. The result demonstrates that the proposed method obtains the gain of about -0.3% in dependent views and about -0.1% in overall performance.

# Proposed Method

Method 1 – Simplification of filtering in DBBP

DBBP partitions the object and background regions by the segmentation binary mask. Then, object- and background-focus motion compensations are performed for one block. Each of the motion-compensated blocks consists of the object and background. These two blocks are merged by extracting the object and background from the block, based on the mask. Finally, the filtering is performed on both luminance and chrominance components of boundary pixels p and q in the merged block, as displayed in Fig. 1. Filtered pixels p’ and q’ can be generated as follows:

p’ = ((p-1) + (p)<<1 + (q))>>2,  q’= ((p) + (q)<<1 + (q-1))>>2

Fig. 1. Filtering in DBBP

To simplify the filtering process, the proposed method removes the chrominance filtering. For example, the luminance component of the pixel p is only filtered, if p-1 and q belong to the different regions based on the segmentation mask. In the case of the pixel q, if p and q-1 is placed in the different regions, the filtering is performed on the luminance of q.

Method 2 – Extension of the filtering to PU boundary

The proposed method extends the filtering process into PU boundaries in the general partitioned modes such as 2NxN, Nx2N, NxN, and other AMP modes. Fig. 2 illustrates examples of 2NxN and Nx2N for the filtering. The filtered pixels p’ and q’ can be generated in the same manner as the filtering in DBBP

Fig. 2. Extension of the filtering

# Test Results

Test results of the proposed method are summarized in Tables 1 to 3.

Table 1. Performance of the proposed simplification

Table 2. Performance of the proposed extension

Table 3. Performance of the combination of the simplification and extension

# Conclusion

By integrating the proposed method on the current 3D-HTM, it can improve the coding performance. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).