| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0089 |
| --- | --- |

| Title: | 3D-CE1 related: Diagonal prediction using ARP |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Advanced residual prediction (ARP) is used to efficiently code a residual between original and predicted pixels. Both temporal and inter-view ARP are supported and three weighting factors such as 0, 1, and 0.5 are considered. However, it is not clear why the 0.5 weighting factor is used. This contribution proposes to support diagonal prediction instead of the 0.5 weighting factor. The result illustrates that, even though the proposed method achieves the minor coding gain (about -0.1% in a dependent view), it reduces the memory bandwidth (from 172% to 67% in the 8x2 memory pattern) by removing the additional block accesses when the weighting factor is equal to 0.5.

# Proposed Method

Fig. 1 represents a structure of the temporal and inter-view ARP. In ARP, a predicted block is modified according to three weighting factors such as 0, 1, and 0.5, as shown in Table 1. The best predicted block is determined through RDO. Final residual is generated by subtracting the predicted block derived based on the weighting factor from an original one (Curr).

(a) Temporal ARP			(b) Inter-view ARP

Fig. 1. ARP (F0123).

Table 1. Predicted block according to weighting factors

| Index | Temporal ARP | Inter-view ARP |
| --- | --- | --- |
| 0 | CurrRef | Base |
| 1 | CurrRef+Base-BaseRef | Base+CurrRef-BaseRef |
| 2(0.5) | CurrRef+(Base-BaseRef)>>1 | Base+(CurrRef-BaseRef)>>1 |

The proposed method performs the diagonal prediction using the existing ARP functions. In the temporal ARP, instead of the block accesses of CurrRef, Base, and BaseRef, only BaseRef is fetched. In order to derive BaseRef, the same motion derivation as in ARP is performed. In the inter-view ARP, the proposed method derives BaseRef in the same manner as the temporal ARP.

Table 2. Proposed predicted block

| Index | Temporal ARP | Inter-view ARP |
| --- | --- | --- |
| 0 | CurrRef | Base |
| 1 | CurrRef+Base-BaseRef | Base+CurrRef-BaseRef |
| 2 | BaseRef | BaseRef |

# Test Results

Test results of the proposed method are summarized in Tables 1 and 2. Table 1 indicates the performance in terms of the coding gain. Table 2 illustrates the performance in terms of the memory bandwidth when the weighting factor is equal to 0.5.

Table 1. Coding performance of the proposed method

Table 2. Analysis of memory bandwidth

| Memory pattern | Original | Proposed |
| --- | --- | --- |
| 1x1 | 132% | 50% |
| 4x2 | 148% | 58% |
| 4x4 | 139% | 54% |
| 8x2 | 172% | 67% |

# Conclusion

By integrating the proposed method on the current 3D-HTM, it reduces the average memory bandwidth in ARP. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).