| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0090 |
| --- | --- |

| Title: | 3D-HEVC HLS Clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Min Woo Park Jin Young Lee | Tel: Email: | {yongjin9.cho, m.w.park, jinyoung79.lee} @samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes some high level clean-ups in the current 3D-HEVC draft as the following:

- Modification of vps_extension2_flag semantics such that the flag shall be zero when the number of views is equal to 1

- Modification of for-loops in vps_extension2 syntax such that they make more sense than those in the current draft.

Changes to the current draft are marked by yellow highlighted strikethroughs and red font.

# Proposal

- Semantics of vps_extension2_flag

Current semantics:

vps_extension2_flag equal to 0 specifies that no vps_extension2( ) syntax structure is present in the VPS RBSP syntax structure. vps_extension_flag equal to 1 specifies that the vps_extension2( ) syntax structure is present in the VPS RBSP syntax structure.

The variable MvHevcCompatibilityFlag is set equal to !vps_extension2_flag.

Problem statement:

There is no constraint on the value of the flag, which potentially result in signaling of unnecessary syntax elements. Because vps_extension2 () syntax structure delivers parameters related only to 3D-HEVC, when the number of views is equal to 1, none of the parameters needs to be signaled. Hence, the flag has to be 0 when the number of views is equal to 1

Proposed modification:

vps_extension2_flag equal to 0 specifies that no vps_extension2( ) syntax structure is present in the VPS RBSP syntax structure. vps_extension_flag equal to 1 specifies that the vps_extension2( ) syntax structure is present in the VPS RBSP syntax structure. The value of vps_extension2_flag shall be 0 when the value of NumViews is equal to 1.

The variable MvHevcCompatibilityFlag is set equal to !vps_extension2_flag.

- vps_extension2_syntax

- The first for-loop

Problem statement:

In the current working draft, the loop starts from 0. When i is equal to 0, layerId is assigned the value of layer_id_in_nuh[0], which is always 0. Hence, the condition in the loop that checks whether the layerId is not equal to 0 is false – which disables the signalling of the syntax elements under the condition – only when i is equal to 0. So, having the loop starts from 1, and removing the condition will provide identical signaling without any condition checks.

Proposed syntax modification: let the loop start from 1, and remove the condition

| for( i = 01; i <= vps_max_layers_minus1; i++ ) { |  |
| --- | --- |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId != 0 ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| log2_sub_pb_size_minus3[ layerId ] | ue(v) |
| …… |  |
| } |  |
| } |  |

- The second for-loop

Problem statement:

According to the current working draft, when NumViews is equal to 1, cp_present_flag[0] and cp_in_slice_segment_header_flag[0] are for no reason. That is, even though the values of the flags are 1 and 0, respectively, following camera parameters will not be sent because the loop indexed by ‘j’ loop from 0 to 0 when i is equal to 0. Since camera parameters signals relationship between views (or cameras), it is illogical to consider camera parameter for the first view (or camera).

Proposed syntax modification: make the loop start from 1 instead of 0

| for( i = 01; i < NumViews; i++ ) { |  |
| --- | --- |
| cp_present_flag[ i ] | u(1) |
| if( cp_present_flag[ i ] ) { |  |
| cp_in_slice_segment_header_flag[ i ] | u(1) |
| if( !cp_in_slice_segment_header_flag[ i ] ) |  |
| for( j = 0; j < i; j++ ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).