Replace subclauses 7.4.2.4.4 with the following (with differences indicated in turquois):

7.4.2.4.4	Order of NAL units and coded pictures and their association to access units

Note: The changes to this subclause are in relative to the spec text in JCTVC-R0236 / JCT3V-I0049.

This subclause specifies the order of NAL units and coded pictures and their association to access units for CVSs that conform to one or more of the profiles specified in Annex A and that are decoded using the decoding process specified in clauses 2 through 10.

NOTE 1 – The association of NAL units to access units may differ depending on which profiles the CVSs conform to or which decoding process is used for decoding the bitstream.

An access unit consists of one coded picture with nuh_layer_id equal to 0, zero or more VCL NAL units with nuh_layer_id greater than 0, and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in subclause 7.4.2.4.5.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

Let firstBlPicNalUnit be the first VCL NAL unit of a coded picture with nuh_layer_id equal to 0. The first of any of the following NAL units preceding firstBlPicNalUnit and succeeding the lastVCL NAL unit preceding firstBlPicNalUnit, if any, specifies the start of a new access unit:

NOTE 2 – The VCL NAL unit preceding the firstBlPicNalUnit in decoding order may have nuh_layer_id greater than 0.

–	access unit delimiter NAL unit with nuh_layer_id equal to 0 (when present),

–	VPS NAL unit with nuh_layer_id equal to 0 (when present),

–	SPS NAL unit with nuh_layer_id equal to 0 (when present),

–	PPS NAL unit with nuh_layer_id equal to 0 (when present),

–	Prefix SEI NAL unit with nuh_layer_id equal to 0 (when present),

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 and with nuh_layer_id equal to 0 (when present),

–	NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 and with nuh_layer_id equal to 0 (when present).

NOTE 3 – The the first NAL unit preceding firstBlPicNalUnit and succeeding the last VCL NAL unit preceding firstBlPicNalUni, if any, can only be one of the above-listed NAL units.

When there is none of the above NAL units preceding firstBlPicNalUnit and succeeding the last VCL NAL unit preceding firstBlPicNalUnit, if any, firstBlPicNalUnit starts a new access unit.

The order of the coded pictures and non-VCL NAL units within an access unit shall obey the following constraints:

–	When an access unit delimiter NAL unit with nuh_layer_id equal to 0 is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit with nuh_layer_id equal to 0 in any access unit.

–	When any VPS NAL units, SPS NAL units, PPS NAL units, prefix SEI NAL units, NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44, or NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 are present, they shall not follow the last VCL NAL unit of the access unit.

–	NAL units having nal_unit_type equal to FD_NUT or SUFFIX_SEI_NUT, or in the range of RSV_NVCL45..RSV_NVCL47 or UNSPEC56..UNSPEC63 shall not precede the first VCL NAL unit of the access unit.

–	When an end of sequence NAL unit is present, it shall be the last NAL unit in the access unit other than an end of bitstream NAL unit (when present).

–	When an end of bitstream NAL unit is present, it shall be the last NAL unit in the access unit.

- NOTE 4 – Decoders conforming to profiles specified in Annex Error: Reference source not found do not use NAL units with nuh_layer_id greater than 0, e.g. access unit delimiter NAL units with nuh_layer_id greater than 0, for access unit boundary detection, except for identification of a NAL unit as a VCL or non-VCL NAL unit. Consequently, HRD parameters carried in non-nested buffering period, picture timing, and decoding unit information SEI messages apply to access units based on such access unit boundary detection.

The structure of access units not containing any NAL units with nal_unit_type equal to FD_NUT, VPS_NUT, SPS_NUT, PPS_NUT, RSV_VCL_N10, RSV_VCL_R11, RSV_VCL_N12, RSV_VCL_R13, RSV_VCL_N14, or RSV_VCL_R15, RSV_IRAP_VCL22, or RSV_IRAP_VCL23, or in the range of RSV_VCL24..RSV_VCL31, RSV_NVCL41..RSV_NVCL47, or UNSPEC48..UNSPEC63 is shown in Figure 8-1.

Figure 8-1 – Structure of an access unit not containing any NAL units with nal_unit_type equal to FD_NUT, SUFFIX_SEI_NUT, VPS_NUT, SPS_NUT, PPS_NUT, RSV_VCL_N10, RSV_VCL_R11, RSV_VCL_N12, RSV_VCL_R13, RSV_VCL_N14, RSV_VCL_R15, RSV_IRAP_VCL22, or RSV_IRAP_VCL23, or in the range of RSV_VCL24..RSV_VCL31, RSV_NVCL41..RSV_NVCL47, or UNSPEC48..UNSPEC63

Replace subclauses F.7.4.2.4.4 with the following:

F.7.4.2.4.4	Order of NAL units and coded pictures and association to access units

This clause specifies the order of NAL units and coded pictures and their association to access unit for CVSs that contain NAL units with nuh_layer_id greater than 0 that are decoded using the decoding processes specified in Annex F, Annex G and Annex H.

An access unit consists of one or more coded pictures, each with a distinct value of nuh_layer_id, and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in subclause 7.4.2.4.5.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

A VCL NAL unit is the first VCL NAL unit of an access unit, when all of the following conditions are true:

–	first_slice_segment_in_pic_flag is equal to 1.

–	At least one of the following conditions is true:

–	The previous picture in decoding order belongs to a different POC resetting period than the picture containing the VCL NAL unit.

–	PicOrderCntVal derived for the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

NOTE 1 – For any two consectuive access units auA and auB that belong to the same POC resetting period, the PicOrderCntVal of pictures in auA cannot be the same as the PicOrderCntVal of pictures in auB.

NOTE 2 – Additionally, more conditions, e.g. the following conditions, could, but need not be used:

–	The nuh_layer_id value of the VCL NAL unit is equal to 0.

–	vps_poc_lsb_aligned_flag is equal to 1 and the slice_pic_order_cnt_lsb value of the VCL NAL unit differs from the slice_pic_order_cnt_lsb value of the previous VCL NAL unit in decoding order.

The first of any of the following NAL units preceding the first VLC NAL unit firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, specifies the start of a new access unit:

–	Access unit delimiter NAL unit (when present).

–	VPS NAL unit (when present)

–	SPS NAL unit (when present)

–	PPS NAL unit (when present)

–	Prefix SEI NAL unit (when present)

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 (when present)

–	NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 (when present)

When there is none of the above NAL units preceding firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, firstVclNalUnitInAu starts a new access unit.

The order of the coded pictures and non-VCL NAL units within an access unit shall obey the following constraints:

–	When an access unit delimiter NAL unit is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit in any access unit.

–	When any VPS NAL units, SPS NAL units, PPS NAL units, prefix SEI NAL units, NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44, or NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 are present, they shall not follow the last VCL NAL unit of the access unit	.

–	NAL units having nal_unit_type equal to FD_NUT or SUFFIX_SEI_NUT, or in the range of RSV_NVCL45..RSV_NVCL47 or UNSPEC56..UNSPEC63 shall not precede the first VCL NAL unit of the access unit.

–	When an end of sequence NAL unit is present, it shall be the last NAL unit in the access unit other than an end of bitstream NAL unit (when present).

–	When an end of bitstream NAL unit is present, it shall be the last NAL unit in the access unit.

Replace subclauses F.7.4.3.5 with the following:

F.7.4.3.5	Access unit delimiter RBSP semantics

The specifications in subclause 7.4.3.5 apply, with following additions and modifications.

The access unit delimiter may be used to indicate the type of slices present in the coded pictures in the access unit containing the access unit delimiter and to simplify the detection of the boundary between access units. There is no normative decoding process associated with the access unit delimiter.

pic_type indicates that the slice_type values for all slices of the coded pictures in the access unit containing the access unit delimiter are members of the set listed in Error: Reference source not found for the given value of pic_type. The value of pic_type shall be equal to 0, 1 or 2 in bitstreams conforming to this version of this Specification. Other values of pic_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved values of pic_type.

Modify subclause D.2.8 as follows:

| recovery_point( payloadSize ) { | Descriptor |
| --- | --- |
| recovery_poc_cnt | se(v) |
| exact_match_flag | u(1) |
| broken_link_flag | u(1) |
| poc_msb_sei_val | ue(v) |
| } |  |

Modify subclause D.3.8 as follows:

The recovery point SEI message assists a decoder in determining when the decoding process will produce acceptable pictures in the current layer for display after the decoder initiates random access or layer up-switching, or after the encoder indicates a broken link.

When all decoded pictures in the access units that precede the current access unit in decoding order are removed from the bitstream, the recovery point picture (defined below) and all the subsequent pictures in output order in the current layer can be correctly or approximately correctly decoded, the current picture is referred to as a layer random-accessing picture. When all the pictures that belong to the reference layers of the current layer and may be used for reference by the current picture or subsequent pictures in decoding order are correctly decoded, and the recovery point picture and all the subsequent pictures in output order in the current layer can be correctly or approximately correctly decoded when no picture prior to the current picture in decoding order in the current layer are present in the bitstream, the current picture is referred to as a layer up-switching picture.

When the recovery point SEI message applies to the current layer and all the reference layers of the current layer, the current picture is indicated as a layer random-accessing picture. When the recovery point SEI message applies to the current layer but not all the reference layers of the current layer, the current picture is indicated as a layer up-switching picture.

Decoded pictures in the current layer produced by random access or layer up-switching at or before the current access unit need not be correct in content until the indicated recovery point, and the operation of the decoding process for pictures in the current layer starting at the current picture may contain references to pictures unavailable in the decoded picture buffer.

In addition, by use of the broken_link_flag, the recovery point SEI message can indicate to the decoder the location of some pictures in the current layer in the bitstream that can result in serious visual artefacts when displayed, even when the decoding process was begun at the location of a previous IRAP access unit in decoding order that contain IRAP pictures in all layers.

NOTE 1 – The broken_link_flag can be used by encoders to indicate the location of a point after which the decoding process for the decoding of some pictures in the current layer may cause references to pictures that, though available for use in the decoding process, are not the pictures that were used for reference when the bitstream was originally encoded (e.g. due to a splicing operation performed during the generation of the bitstream).

When the current picture is a layer random access-accessing picture and random access is performed to start decoding from the current access unit, the decoder operates as if the current access unit was the first access unit in the bitstream in decoding order, and the following applies:

–	If poc_msb_sei_val is not present, the variable PrevPicOrderCnt[ nuh_layer_id ] used in derivation of PicOrderCntVal for each picture in the access unit is set equal to 0.

–	Otherwise (poc_msb_sei_val is present), the PicOrderCnt of the current picture is derived as if poc_msb_val was present and equal to poc_msb_sei_val, and the PicOrderCnt of each of the other pictures in the current access unit is also is derived as if poc_msb_val was present and equal to poc_msb_sei_val.

When the current picture is either a layer random-accessing picture or a layer up-switching picture and layer up-switching is performed to start decoding of the current layer from the current access (while decoding of the reference layers of the current layer have started eariler and pictures of those layers in the current access unit are correctly decoded), the decoder operates as if the current pictrue was the first picture of the current layer in the bitstream in decoding order, and the PicOrderCntVal of the current picture is set equal to the PicOrderCntVal of any other decoded picture in the current access unit.

NOTE 2 – When HRD information is present in the bitstream, a buffering period SEI message should be associated with the access unit associated with the recovery point SEI message in order to establish initialization of the HRD buffer model after a random access.

Any SPS or PPS RBSP that is referred to by a picture of the access unit containing a recovery point SEI message or by any picture in a subsequent access unit in decoding order shall be available to the decoding process prior to its activation, regardless of whether or not the decoding process is started at the beginning of the bitstream or with the access unit, in decoding order, that contains the recovery point SEI message.

recovery_poc_cnt specifies the recovery point of decoded pictures in the current layer in output order. If there is a picture picB in the current layer that follows the current picture picA but precedes an access unit containing an IRAP picture in the current layer in decoding order and PicOrderCnt( picB ) is equal to PicOrderCnt( picA ) plus the value of recovery_poc_cnt, where PicOrderCnt( picA ) and PicOrderCnt( picB ) are the PicOrderCntVal values of picA and picB, respectively, immediately after the invocation of the decoding process for picture order count for picB, the picture picB is referred to as the recovery point picture. Otherwise, the first picture picC in the current layer in output order for which PicOrderCnt( picC ) is greater than PicOrderCnt( picA ) plus the value of recovery_poc_cnt is referred to as the recovery point picture, where PicOrderCnt( picA ) and PicOrderCnt( picC ) are the PicOrderCntVal values of picA and picC, respectively, immediately after the invocation of the decoding process for picture order count for picC. The recovery point picture shall not precede the current picture in decoding order. All decoded pictures in the current layer in output order are indicated to be correct or approximately correct in content starting at the output order position of the recovery point picture. The value of recovery_poc_cnt shall be in the range of −MaxPicOrderCntLsb / 2 to MaxPicOrderCntLsb / 2 − 1, inclusive.

exact_match_flag indicates whether decoded pictures in the current layer at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit containing the recovery point SEI message will be an exact match to the pictures in the current layer that would be produced by starting the decoding process at the location of a previous access unit where the picture of the layer in the current layer and the pictures of all the direct and indirect reference layers are IRAP pictures, if any, in the bitstream. The value 0 indicates that the match may not be exact and the value 1 indicates that the match will be exact. When exact_match_flag is equal to 1, it is a requirement of bitstream conformance that the decoded pictures in the current layer at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit containing the recovery point SEI message shall be an exact match to the pictures in the current layer that would be produced by starting the decoding process at the location of a previous access unit where the picture of the layer in the current layer and the pictures of all the direct and indirect reference layers are IRAP pictures, if any, in the bitstream.

NOTE 3 – When performing random access, decoders should infer all references to unavailable pictures as references to pictures containing only intra coding blocks and having sample values given by Y equal to ( 1  <<  ( BitDepthY − 1 ) ), Cb and Cr both equal to ( 1  <<  ( BitDepthC − 1 ) ) (mid-level grey), regardless of the value of exact_match_flag.

When exact_match_flag is equal to 0, the quality of the approximation at the recovery point is chosen by the encoding process and is not specified in this Specification.

broken_link_flag indicates the presence or absence of a broken link in the layer in the current layer at the location of the recovery point SEI message and is assigned further semantics as follows:

–	If broken_link_flag is equal to 1, pictures in the current layer produced by starting the decoding process at the location of a previous access unit where the picture of the layer in the current layer and the pictures of all the direct and indirect reference layers are IRAP pictures may contain undesirable visual artefacts to the extent that decoded pictures in the current layer at and subsequent to the access unit containing the recovery point SEI message in decoding order should not be displayed until the specified recovery point in output order.

–	Otherwise (broken_link_flag is equal to 0), no indication is given regarding any potential presence of visual artefacts.

When the current picture is a BLA picture, the value of broken_link_flag shall be equal to 1.

Regardless of the value of the broken_link_flag, pictures in the current layer subsequent to the specified recovery point in output order are specified to be correct or approximately correct in content.

poc_msb_sei_val indicates the value of the most significant bits of the picture order count value of pictures in the current access unit. The value of poc_msb_sei_val shall be in the range of 0 to 232 − log2_max_pic_order_cnt_lsb_minus4 − 4, inclusive. When not present, the value of poc_msb_sei_val is inferred to be equal to 0.

Modify subclause D.2.22 as follows:

| temporal_sub_layer_zero_index( payloadSize ) { | Descriptor |
| --- | --- |
| temporal_sub_layer_zero_idx | u(8) |
| irap_pic_id | u(8) |
| num_prev_discardable_tsl0_pics | ue(v) |
| } |  |

Modify subclause D.3.22 as follows:

The temporal sub-layer zero index SEI message provides information that can assist the decoder for detection of missing coded pictures that have TemporalId and discardable_flag both equal to 0 and are not RASL pictures, RADL pictures, or sub-layer non-reference (SLNR) pictures.

When a temporal sub-layer zero index SEI message is present in the current access unit and applies to the current layer and the current picture is not an IRAP picture, a temporal sub-layer zero index SEI message that applies to the current layer shall also be present in the preceding access unit in decoding order with TemporalId equal to 0 and containing a picture that is not a RASL picture, RADL picture, or SLNR picture in the current layer.

Let ReferencedTsl0Pic denote a picture that has TemporalId equal to 0 and is not a RASL picture, RADL picture, or SLNR picture.

temporal_sub_layer_zero_idx indicates a temporal sub-layer zero index as follows:

–	If the current picture is a ReferencedTsl0Pic, temporal_sub_layer_zero_idx indicates the temporal sub-layer zero index for the current picture.

–	Otherwise, temporal_sub_layer_zero_idx indicates the temporal sub-layer zero index of the preceding picture in the current layer in decoding order that is a ReferencedTsl0Pic.

When the bitstream contains a preceding access unit in decoding order that contained a ReferencedTsl0Pic, and that preceding access unit contains a temporal sub-layer zero index SEI message that applies to the current layer, the variable prevTsl0Idx is set equal to the value of temporal_sub_layer_zero_idx of that SEI message.

The following constraints apply to the value of temporal_sub_layer_zero_idx:

–	If the current picture is an IRAP picture, temporal_sub_layer_zero_idx shall be equal to 0.

–	Otherwise, the following applies:

–	If the current picture is a ReferencedTsl0Pic, temporal_sub_layer_zero_idx shall be equal to ( prevTsl0Idx + 1 ) % 256.

–	Otherwise, temporal_sub_layer_zero_idx shall be equal to prevTsl0Idx.

irap_pic_id is an IRAP picture identifier for the current layer. When the current picture is not the first picture in the current layer in the bitstream in decoding order and the preceding IRAP picture in the current layer in decoding order has an associated temporal sub-layer zero index SEI message, the following constraints apply to the value of irap_pic_id:

–	If the current access unitpicture is an IRAP picture, irap_pic_id shall differ in value from the value of irap_pic_id of the temporal sub-layer zero index SEI message associated with the preceding IRAP picture in the current layer in decoding order.

NOTE – It is suggested for the value of irap_pic_id to be set to a random value (subject to the constraints specified herein), to minimize the likelihood of duplicate values appearing in a layer in the bitstream due to picture losses or splicing operations.

–	Otherwise, irap_pic_id shall be equal to the value of irap_pic_id of the temporal sub-layer zero index SEI message associated with the preceding IRAP picture in the current layer in decoding order.

When the current picture is a ReferencedTsl0Pic that is not an IRAP picture or the current picture is not ReferencedTsl0Pic, the variable latestReferencedTsl0Pic is defined for the current picture as follows:

–	If the current picture is a ReferencedTsl0Pic and not an IRAP picture, the latestReferencedTsl0Pic is the current picture.

–	Otherwise (the current picture is not a ReferencedTsl0Pic), the lastestReferencedTsl0Pic is the most recent picture in decoding order in the current layer that is a ReferencedTsl0Pic and not an IRAP picture and that precedes the current picture in decoding order.

The variable latestIrapPic is defined for the current picture as follows:

–	If the current picture is an IRAP picture, the latestIrapPic is the current picture.

–	Otherwise, the latestIrapPic is the most recent picture in decoding order in the current layer that is an IRAP picture and that precedes the current picture in decoding order.

num_prev_discardable_tsl0_pics specifies, when the current picture is a ReferencedTsl0Pic that is not an IRAP picture or the current picture is not ReferencedTsl0Pic, the number of ReferencedTsl0Pic pictures for which all of the following conditions are true:

–	The pictures are consecutive in decoding order.

–	The pictures are discardable pictures (i.e. pictures with discardable_flag equal to 1).

–	The pictures precede the latestReferencedTsl0Pic in decoding order.

–	Either one of the pictures is the latestIrapPic and others succeed the latestIrapPic in decoding order, or the pictures all succeed the latestIrapPic in decoding order.

The value of num_prev_discardable_tsl0_pics shall be equal to 0 when either the current picture is an IRAP picture or the current picture is not a ReferencedTsl0Pic and the most recent ReferencedTsl0Pic in the current layer that precedes the current picture in decoding order is an IRAP picture.

The value of num_prev_discardable_tsl0_pics shall be in the range of 0 to temporal_sub_layer_zero_idx, inclusive.