| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0280v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0092v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Bitstream conformance restrictions in subclause C.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

There are a few editor's notes in subclause C.4 (Bitstream conformance) in the latest MV-HEVC/SHVC draft texts. This document proposes changes that are asserted to address some of those editor's notes as well as some other ambiguities related to the bitstream conformance constraints. The issues and the proposed changes are both summarized, and the text changes are provided in an attachment (based on the AHG10 output text in JCTVC-R0010v3 / JCT3V-I0010v3). The text changes also include some other changes that are asserted as editorial changes.

In the version v2 of this document, the bitstream conformance constraint related to the CPB output time of decoding units is updated. Other minor editorial improvements are also made.

# Summary of the issues

- Some of the specified bitstream conformance parameters apply to an access unit. In a bitstream-partition-specific CPB model, an access unit may be split across multiple bitstream partitions and checking the CPB-related parameters for the entire access unit is not possible.

- A decoding unit is defined as either the entire access unit, or a set of consecutive VCL NAL units in the access unit and their associated non-VCL NAL units. Consider a partitioning scheme where two adjacent layers (e.g. layers with nuh_layer_id equal to 0 and 1) are in different bitstream partitions. Defining a decoding unit that contains VCL NAL units from different bitstream partitions would make it unclear as to how the delivery schedule should be defined for any bitstream partition that contains VCL NAL units in the decoding unit.

- The value of the number of decoding units in the picture timing SEI message is constrained to be in the range of 0 to PicSizeCtbsInY − 1, inclusive. When an access unit or NAL units of an access unit within a bitstream partition contain NAL units of more than one picture, the specified range is insufficient when each CTB is coded as a decoding unit.

- The variables DpbOutputTime[ ] and other related variables are derived for each picture. When there are multiple layers in a bitstream, where at least one access unit could contain more than one picture, deriving DpbOutputTime[ ] for each picture and restricting the difference of DpbOutputTime[ ] of successive pictures in output order to be greater than a lower bound could prevent access units to contain more than one picture.

- The value of decoding unit (DU) CPB removal time of a decoding unit duX is restricted to be greater than the DU CPB removal time for all decoding units that precede duX in decoding order. However this restriction is currently specified to be only applied for constant bit rate CPB model. Absence of such a restriction for the variable bit rate CPB model could lead to pictures being decoded out of order and reference pictures being unavailable for decoding of some pictures. Moreover, the restriction is currently specified not to be applied for DUs within the 0-th bitstream partition. However, when multiple layers are included in the 0-th bitstream partition, absence of such a restriction could also lead to pictures being decoded out of order and reference pictures being unavailable for decoding of some pictures.

- The difference between the maximum and the minimum POC value of pictures included in the following list:

    - the current picture

    - the previous picture in decoding order that has TemporalId equal to 0 and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture

    - the short-term reference pictures in the RPS of the current picture

    - the pictures that have PicOutputFlag equal to 1, precede the current picture in decoding order, and succeed the current picture in output order

is constrained to be less than MaxPicOrderCntLsb / 2, when the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1. When an access unit auA contains a picture with nuh_layer_id equal to 0 and NoClrasOutputFlag equal to 1, the enhancement layer pictures in the access unit could contain pictures that are not IRAP pictures. This constraint should not be applied to pictures in auA, because when NoClrasOutputFlag equal to 1, all the layers in the bitstream are reinitialized and there may not be any relation between the POC values of pictures that precede auA in decoding order and pictures that succeed auA, inclusive, in decoding order.

- For two pictures m and n within a CVS, when DpbOutputTime[ m ] is greater than DpbOutputTime[ n ], it is constrained that PicOrderCntVal of picture m shall be greater than PicOrderCntVal of picture n. When the PicOrderCntVal of the picture m or picture n or a picture in between is reset, this constraint is ambiguous as to what value of PicOrderCntVal of the pictures is to be used. Moreover, some restrictions of PicOrderCntVal should be applicable even across coded video sequences because POC chain can continue in enhancement layers even across a CVS boundary.

# Summary of the proposed changes

- For the bitstream-partition-specific CPB model, the bitstream conformance parameters are specified to be applicable to partition units, where a partition unit comprises of VCL NAL units of an access unit that belong to the layers contained in the partition and their associated non-VCL NAL units.

- A decoding unit is defined as either an access unit or a subset of an access unit otherwise, consisting of one or more VCL NAL units in an access unit within a bitstream partition and the associated non-VCL NAL units.

- The value of num_decoding_units_minus1 is allowed to be in the range of 0 to totSizeInCtbs − 1, inclusive, where totSizeInCtbsY is the number of CTBs in a partition unit when the picture timing SEI message is associated with the partition unit and the number of CTBs in an access unit when the picture timing SEI message is associated with the access unit.

- The variable DpbOutputTime[ ] and DpbOutputInterval[ ] are derived for each access unit and the restrictions on the difference of the DpbOutputTime is applied to pictures in different access units.

- The value of DU CPB removal time of a decoding unit duX is restricted to be greater than the DU CPB removal time for all decoding units that precede duX in decoding order also for the variable bit rate CPB model, and the restriction also applies to DUs within the 0-th bitstream partition.

- The difference between the maximum and the minimum POC values of pictures in the following list:

    - the current picture

    - the previous picture in decoding order that has TemporalId equal to 0 and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture

    - the short-term reference pictures in the RPS of the current picture

    - the pictures that have PicOutputFlag equal to 1, precede the current picture in decoding order and succeed the current picture in output order

is constrained to be less than MaxPicOrderCntLsb / 2 when the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 or is not the first picture of the current layer with nuh_layer_id greater than 0 that follows an IRAP picture that has nuh_layer_id equal to 0 and has NoClrasOutputFlag equal to 1.

- The constraint on DpbOutputTime and PicOrderCntVal is clarified.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).