| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0093 |
| --- | --- |

| Title: | Depth coding compatible with arbitrary bit-depth in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the disparity vector between two views used in inter-view motion prediction (IVMP) for depth coding is converted from a fixed depth value number 128. When the depth component is coded with a bit-depth value other than 8, 128 is no longer the average depth value. In this contribution, it is proposed to make depth coding compatible with arbitrary bit-depth by replacing the number 128 with 1<<(BitDepthY –1 ).

# Introduction

Inter-view motion prediction (IVMP) is applied for depth coding as well as for texture coding [1]. For texture coding, neighboring block disparity vector (NBDV) method is adopted to derive the disparity vector (DV) between two views. For depth coding, however, a much more simplified method is applied. The DV is directly converted from a fixed depth value number 128, which represents a median value for the depth component.

Nevertheless, 128 is the median value only when the depth component is coded with 8 bit-depth, indicating the value range [0, 255]. When the depth component is coded with other bit-depth valued, such as 10 or 12, 128 is meaningless and is no longer an appropriate estimation.

# Proposed method

In this proposed modification, in order to support arbitrary bit-depth, the DV used in IVMP for depth coding is converted from the depth value 1<<(BitDepthY –1 ) instead of 128.

# Conclusion

In this contribution, 1<<(BitDepthY –1 ) instead of 128 is used to derive the disparity vector used by IVMP in depth coding. So that depth coding can be compatible with arbitrary bit-depth.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H1001, Apr. 2014.