| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCT3V-I0094 |
| --- | --- |

| Title: | DBBP simplification based on sub-block level processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1MediaTek Beijing: Building 1-B, No. 6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, depth-based block partitioning (DBBP) is designed to generate a segment mask, combine two predicted blocks and internally filter the combined block all at pixel-level. In this contribution, it is proposed to calculate the mask values at 2x2 sub-block level to reduce complexity of DBBP. Experimental results reportedly show such a simplification brings no coding loss.

# Introduction

- In the current 3D-HEVC [1], with the derived virtual depth block and modified partition as input, DBBP conducts the motion compensation process within 4 steps (Fig. 1) as follows:

- Mean value calculation: A mean value of all the pixels of the virtual depth block is calculated;

- Mask generation: A segment mask is generated based on the virtual depth block by pixel-by–pixel comparison between the depth block and the calculated mean value;

- Prediction combination: Two 2Nx2N prediction blocks with their own decoded motion vectors are combined pixel by pixel depending on the segment mask;

- Post filtering: The pixel boundaries in the combined prediction block are smoothed by filtering horizontally and vertically. The pixel boundaries are positions where adjacent values in the segment mask are different.

- There are several complexity concerns in the current DBBP design. First, the process requires pixel-level mean-value calculation and pixel-by-pixel comparison for mask generation. As a result, the combination of the two 2Nx2N predicted block also must be operated pixel by pixel. In addition, the post filtering process must compare three mask values in each horizontal or vertical filtering process (comparing left, current and right mask for horizontal; above, current and below values for vertical). The worst case of mask value distribution is as Fig. 2 shows. The decoder should access every pixel in the mask matrix, and every predicted pixel would be filtered two times for both horizontal and vertical directions.

Fig. 1. Mask based prediction in the current DBBP procedure

Fig. 2. The mask value distributions which produces the worst case of the current DBBP motion compensation

# Proposed method

In this invention, it is proposed to perform the DBBP process of mask generation, block combination and post filtering sub-block by sub-block, as shown in Fig. 3. Supposing the derived depth block be D, the block size be 2Nx2N, the calculation of the mean value a and the mask matrix M is modified as follows:

In addition, the post filtering is modified by the following changes in text specification:

for ( y = 0; y < nCbSX; y++ )
		for( x = 0; x < nCbSX; x++ ) {
			tFlag = segMask[ n * x ][ Max( 0, n * ( y − 1 ) ) ]
			lFlag = segMask[ Max( 0, (n * ( x − 1 ) ) ][ n * y ]
			bFlag = segMask[ n * x ][ Min( n * ( y + 1 ), nCbSL − 1 ) ]
			rFlag = segMask[ Min( n * ( x + 1 ), nCbSL − 1 ) ][ n*y ]
			cFlag = segMask[ n * x ][ n * y ]
			filt = p[ x ][ y ]
			if( ( lFlag  | |  cFlag  | |  rFlag )  &&  ( !lFlag  | |  !cFlag  | |  !rFlag ) )

if( lFlag != rFlag)
				filt = ( p[ Max( 0, x − 1 ) ][ y ] + ( filt  <<  1 ) + p[ Min( x + 1, nCbSX − 1 ) ][ y ] )  >>  2
			if( ( tFlag  | |  cFlag  | |  bFlag )  &&  ( !tFlag  | |  !cFlag  | |  !bFlag) )

if( tFlag != bFlag)
				filt = ( p[ x ][ Max( 0, y − 1 ) ] + ( filt  <<  1 ) + p[ x ][ Min( y + 1, nCbSX − 1 ) ] )  >>  2
			predSamples[ x ][ y ] = filt
		}

With the proposed modification, the complexity of DBBP can be reduced significantly. Table 1 lists the number of operations in the mask generation, prediction combination and post filtering processes of the original and modified DBBP. In Table 1, Step 1 demonstrates the mean value calculation for mask generation, step 2 is the pixel-by-pixel comparison for mask generation, step 3 is block combination and step 4 means the post filtering. As a conclusion, the number of operations for each pixel is reduced from 12 to 5.96 for 8x8-CU DBBP cases. In opposite to Fig. 2, the worst case of mask value distribution is as Fig. 4 shows. Only 1/4 the pixels in the mask matrix should be accessed and not every predicted pixel would be filtered two times for horizontal and vertical direction, such as the pixels on the four borders.

Table 1. Operation comparison between the original and proposed methods

| OP. in DBBP w/o partition decision | ADD |  | COMPARISON |  |  | SUM | SUM Calc per pixel | Least Additional Memory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STEP | 1 | 4 | 2 | 3 | 4 | 1~4 | 1~4 | 1~4 |
| ORG | 2Nx2N | 1.5x2x2Nx2Nx2 | 2Nx2N | 2Nx2N | 1.5x2x2Nx2Nx3 | 72NxN | 12 | 2Nx2N |
| PROPOSED | NxN | 1.5x2x (2Nx2Nx2-4N) | NxN | NxN | 1.5x2x (2Nx2N-4N+4)x1 | 39NxN-16N+12 | N=4: 5.96 N=8: 6.20 N=16: 6.34 N=32: 6.41 | NxN |

Fig. 3. 2x2 sub-block based prediciton in the suggested DBBP procedure

Fig. 4. The mask value distributions which produces the worst case of the modified DBBP motion compensation

- Experimental results

The proposed method is implemented on HTM11.0. The experiments are conducted under the common test condition [3]. As the results shown in Table 2, this proposed simplification brings no BD-rate loss.

Table 2.  The BD-rate results of the proposed simplification

- WD change

- I.8.4.4.4 Derivation process for a texture predicted contour pattern

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nTbS specifying the transform block size.

Output of this process is:

- binary partition pattern wedgePattern[ x ][ y ], with x, y =0..nTbS − 1.

The variable recTextPic is set equal to the array of the reconstructed luma picture samples of TexturePic.

For x = 0..nTbS − 1 and y = 0..nTbS − 1 the following applies:

  - refSamples[ x ][ y ] = recTextPic[ xTb + x ][ yTb + y ]

The derivation process for an inter-layer predicted contour pattern as specified in subclause  is invoked with the variable sampInt equal to 2, the block size nTbS and the a reference sample array refSamples[ x ][ y ] as inputs, and the output is the binary partition pattern wedgePattern[ x ][ y ].

- I.8.5.3.3.9.1 Derivation process for contour boundary filtered samples

Inputs to this process are:

- a variable nCbSL specifying the size of the current luma coding block,

- a variable nCbSX specifying the size of the current coding block,

- an (nCbSL)x(nCbSL) array segMask

- an (nCbSX)x(nCbSX) array predSamples prediction samples

Outputs to this process are:

- an modified (nCbS)x(nCbS) array predSamples of luma prediction samples

The (nCbSX)x(nCbSX) array p is set equal to predSamples and the variable n is set equal to ( nCbSL / nCbSX ).

The values of predSamples are derived as specified in the following:

for ( y = 0; y < nCbSX; y++ )
		for( x = 0; x < nCbSX; x++ ) {
			tFlag = segMask[ n * x ][ Max( 0, n * ( y − 1 ) ) ]
			lFlag = segMask[ Max( 0, (n * ( x − 1 ) ) ][ n * y ]
			bFlag = segMask[ n * x ][ Min( n * ( y + 1 ), nCbSL − 1 ) ]
			rFlag = segMask[ Min( n * ( x + 1 ), nCbSL − 1 ) ][ n*y ] 
			cFlag = segMask[ n * x ][ n * y ]
			filt = p[ x ][ y ]
			if( ( lFlag  | |  cFlag  | |  rFlag )  &&  ( !lFlag  | |  !cFlag  | |  !rFlag ) )

if( lFlag != rFlag)
				filt = ( p[ Max( 0, x − 1 ) ][ y ] + ( filt  <<  1 ) + p[ Min( x + 1, nCbSX − 1 ) ][ y ] )  >>  2
			if( ( tFlag  | |  cFlag  | |  bFlag )  &&  ( !tFlag  | |  !cFlag  | |  !bFlag) )

if( tFlag != bFlag)
				filt = ( p[ x ][ Max( 0, y − 1 ) ] + ( filt  <<  1 ) + p[ x ][ Min( y + 1, nCbSX − 1 ) ] )  >>  2
			predSamples[ x ][ y ] = filt
		}

- I.8.8.1 Derivation process for an inter-layer predicted contour pattern

Inputs to this process are:

- a variable sampInt specifying the sampling interval,

- a variable nTbS specifying the block size,

- a reference sample array refSamples[ x ][ y ], with x, y =0..nTbS − 1.

Output of this process is:

- the binary partition pattern contourPattern[ x ][ y ], with x, y =0..nTbS − 1.

The values of the binary partition pattern contourPattern[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable threshVal specifying a threshold for the segmentation of refSamples is derived as specified in the following:

    - The variable sumRefVals is set equal to 0.

    - For x = 0..nTbS − 1 and y = 0..nTbS − 1, when ( x % sampInt ) and ( y % sampInt ) are both equal to 0, the following applies:

        - sumRefVals  +=  refSamples[ x ][ y ]		(Error: Reference source not found-1)

    - The variable threshVal is set equal to ( sumRefVals  >>  ( 2 * log2( nTbS / sampInt ) ) )

  - The variable contourPattern[ x ][ y ] with x, y =0..nTbS − 1 specifying a binary partition pattern is derived as specified in the following:

    - For x = 0..nTbS − 1 and y = 0..nTbS − 1, the following applies:

        - contourPattern[ x ][ y ] = ( refSamples[ ((x >>1)<<1) ][ ((y >>1)<<1) ] > threshVal )	(Error: Reference source not found-2)

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H1001-v2, April, 2014.

- HTM-11.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.