| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0095 |
| --- | --- |

| Title: | 3D-CE2: Single depth intra mode for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of CE2 in which the single depth mode is proposed to code the smooth area within a depth map. A CU-level flag is signaled to indicate whether a CU is coded as single depth mode. When a CU is coded as single depth mode, it is reconstructed by filling this CU with one single depth value. Additionally, an index is transmitted to select one out of several available depth sample candidates derived from the spatial neighboring pixels. Compared to the anchor HTM-11.0, the proposed method reportedly shows overall 0.3% BD-rate savings under common test conditions.

# Introduction

It is found that depth map data show several signal characteristics different from natural video data. The most intuitive one is that depth map contains a lot of smooth area with similar sample values. For most of the cases, the samples within the smooth area share a same depth value.

In Valencia meeting, a coding mode termed as “Single Depth Mode” was proposed [1] to efficiently code the smooth area within a depth picture. The concept of single depth mode is to simply reconstruct the current coding unit (CU) as a smooth area with a single depth sample value. With the help of this new coding mode, the smooth area within a depth map could be coded more efficiently through more concise syntax. To further study the technical parts and evaluate its performance, the single color mode is tested in CE2.

# Proposed method

In the proposed scheme, a flag is signaled in the slice header to indicate the enabling of the single depth mode in current slice. When the slice-level flag is enabled, one CU-level flag is further signaled to indicate the enabling of the single depth mode for each coding CU.

To reconstruct a CU coded as single depth mode, a sample candidate list is first constructed by inserting the sample candidates derived from the neighboring samples of current CU in a predefined order into the candidate list. We have further simplified the method proposed in [1] by reducing the size of the candidate list from 5 to 2 and replacing the corner spatial candidates with the candidates located at the center of the PU boundary. As shown in Figure 1 (each circle represents a sample), the spatial neighboring samples are those five reconstructed samples around the current CU and are inserted into the candidate list in the order: (An/2, Bn/2, B0, A0 and B-1). In the proposed scheme, the size of the sample candidate list is fixed to 2. If empty entry still exists in the candidate list after the pruning and the derivation of candidates from the spatial neighboring samples, offset candidates which are derived by adding offsets to first available sample candidate will be used to fill in those empty entries. After the sample candidate list is constructed, a candidate index is signaled to indicate which sample candidate is selected to fill the current CU. It is noted that, in the proposed scheme, no residual coding is performed on the single depth mode.

Figure 1. The spatial sample candidates used to construct the sample candidate list.

# Simulation results

Experiments are conducted based on HTM-11.0 [2]. All the results are simulated according to the common test conditions (CTC) defined in [3]. The anchor for all the following tests is HTM-11.0.

## Performance of the single depth mode

As shown in Table 1, the proposed single depth mode brings about 0.3% overall coding gain under CTC. Since the reconstruction process of the proposed single depth mode is simple, the decoder could quickly decode and reconstruct a CU coded as single depth mode. As can be seen in Table 1, the decoding time is reduced by 2% compared to the anchor.

Table 1.  The results of the proposed single depth mode under CTC

To further study the technical parts and evaluate the performance of single depth mode, three additional experiments are also conducted and are elaborated as below.

## Performance of modified intra SDC to imitate single depth mode

To see the performance of modifying intra SDC encoding to imitate single depth mode, we have modified the HTM-11.0 encoder to perform additional RDO tests for intra SDC with the DC values equal to the five spatial candidates used in single depth mode. However, as shown in Table 2, no coding gain could be achieved and the encoding time is even increased by about 5%.

Table 2.  The results of modifying intra SDC encoding to imitate single depth mode under CTC

## Performance of including DDD into the candidate list for single depth mode

Table 3 shows the results of including disparity derived depth (DDD) into the candidate list for single depth mode. Compared with the coding performance of single depth mode shown in Table 1, adding DDD as a new candidate for single depth mode could not provide additional coding gain

Table 3.  The results of including DDD into the candidate list for single depth mode under CTC

## Performance of supporting delta DC coding for single depth mode

To see the performance of transmitting residual for single depth mode, a simulation is conducted by adding a delta DC value for each CU coded as single depth mode. Table 4 shows the results of single depth mode with residual coding. Compared to the proposed single depth mode without residual coding (Table 1), no coding gain is observed by adding residual coding to the single depth mode.

Table 4.  The results of transmitting delta DC values for single depth mode under CTC

# Conclusions

In this contribution, an intra coding mode termed as single depth mode is proposed to efficiently code the smooth area within a picture. The simulation results reportedly show that the proposed method introduces 0.3% coding gain under CTC.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “Single depth intra mode for 3D-HEVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H0087, July, 2014.

- HTM-11, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.