| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0097 |
| --- | --- |

| Title: | Removal of the decoder-side DBBP partition derivation process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin1, Yi-Wen Chen1, Xianguo Zhang2, Kai Zhang2, Yu-Wen Huang1, Shawmin Lei1 1MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, xianguo.zhang, kai.zhang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

To signal the depth-based block partitioning (DBBP) mode, a dbbp_flag is signaled at CU level after a partition mode (part_mode) is transmitted. When the DBBP is enabled, the transmitted partition mode will be further replaced by a modified partition mode which is implicitly derived from the corresponding depth block. Since the partition mode is always transmitted to the decoder to indicate the PU partition of current CU even when the DBBP mode is enabled, the partition derivation process for DBBP on the decoder-side in not necessary and could be totally removed. It is proposed to let encoder directly transmit the DBBP partition mode to the decoder through the part_mode syntax and remove the decoder-side DBBP partition derivation process. The experiment results reportedly show no coding loss.

# Introduction

In 3D-HEVC [1], when the depth-based block partitioning (DBBP) mode is applied on a texture coding block, a DBBP partition mode which is derived from the corresponding depth block is used to derive the MVP for the reconstruction of motion information. After the motion information is reconstructed, an arbitrary shaped segmentation mask is derived from a corresponding depth block. The motion compensation is then performed using two 2Nx2N motion compensated blocks and merged based on the depth-based segmentation mask. At the final stage, the motion information for a DBBP CU is stored with the derived DBBP partition mode.

Table 1 shows the syntax design for the DBBP mode. A dbbp_flag is signaled at CU level after the part_mode syntax to indicate whether current CU applies DBBP prediction or not. When the DBBP is enabled, the transmitted partition mode will be further replaced by the aforementioned DBBP partition mode which is implicitly derived from the corresponding depth block as described in the next section.

# Derivation process of a modified partition mode for DBBP

In DBBP, a DBBP partition mode is derived by mapping the depth-derived segmentation mask into one of 2NxN and Nx2N partitions. In the process of partition mode modification, DBBP firstly calculates the mean value of the left-up corner samples of all the 4x4 sub-blocks in the derived virtual depth block. Secondly, a depth predicted contour pattern is generated using the calculated mean value. By comparing the contour pattern and the negation of the contour pattern with the two-segment rectangular partitioning modes, a best matched block partition mode is selected for motion vector storage and MVP derivation. The derivation process of the DBBP partition mode for DBBP is summarized in Fig. 1.

Table 1.  The syntax design for the DBBP flag

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |

Figure 1.  Derivation process of the DBBP partition mode for DBBP

# Proposed methods

Since the the part_mode syntax is always transmitted to the decoder to indicate the PU partition of current CU even when the DBBP mode is enabled. It is not necessary for the decoder to derive the partition mode again from the corresponding depth block. Therefore, it is proposed to let encoder directly transmit the DBBP partition mode to the decoder. The derivation process of the DBBP partition mode on the decoder side could be totally removed.

# Experimental results

The proposed simplification is conducted based on HTM-11.0 [2] under the common test conditions (CTC) [3]. As shown in Table 2, the experiments results (Test1) show that removal of the decoder-side partition mode derivation process in DBBP bring no coding loss, while the derivation process of the DBBP partition mode on the decoder-side could be totally removed. In an additional test (Test2), the dbbp_flag is signaled only when the transmitted partition mode equal to 2NxN or Nx2N. The experimental results of Test2 are shown in Table 3.

Table 2.  The results of Test1

Table 3.  The results of Test2

# Conclusion

In this proposal, we propose to remove the decoder-side partition derivation process in DBBP and let encoder directly send the partition mode through the part_mode syntax. The experimental results show no coding loss (slight coding gain) while the derivation process of the DBBP partition mode on the decoder-side is removed.

# Specification Text Changes

The added part is highlighted in Yellow color and the deleted part in Red color.

- I.7.4.9.5	  Coding unit semantics

- dbbp_flag[ x0 ][ y0 ] equal to 1 specifies that depth based block partition is used for the current coding unit. dbbp_flag[ x0 ][ y0 ] equal to 0 specifies that depth based block partition is not used for the current coding unit. When not present, the value of dbbp_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When dbbp_flag[ x0 ][ y0 ] is equal to 1, PartMode is set equal to PART_2NxN.

- I.8.5	Decoding process for coding units coded in inter prediction mode

- I.8.5.1		General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause 8.6.1 is invoked with the luma location ( xCb, yCb ) as input.

The variable nCbSL is set equal to 1  <<  log2CbSize and the variable nCbSC is set equal to 1  <<  ( log2CbSize − 1 ).

The decoding process for coding units coded in inter prediction mode consists of following ordered steps:

  - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, following applies:

    - If DepthFlag is equal to 0 the derivation process for disparity vectors as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as inputs.

    - Otherwise (DepthFlag is equal to 1), the derivation process for disparity vectors for depth layers as specified in subclause Error: Reference source not found is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as inputs.

  - When dbbp_flag is equal to 1, the derivation process for a modified partitioning mode as specified in subclause  is invoked with the luma locations ( xCb, yCb ) and the coding block size nCbSL as the inputs.

  - The inter prediction process as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the outputs are three arrays predSamplesL, predSamplesCb, and predSamplesCr.

  - When dbbp_flag is equal to 1, the arrays predSamplesL, predSamplesCb, and predSamplesCr are set equal to PredSamplesDbbpL, PredSamplesDbbpCb, and PredSamplesDbbpCr, respectively. [ Ed. (GT): See note in section Error: Reference source not found].

  - The decoding process for the residual signal of coding units coded in inter prediction mode specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the outputs are three arrays resSamplesL, resSamplesCb, and resSamplesCr.

  - The reconstructed samples of the current coding unit are derived as follows:

    - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the luma coding block location ( xCb, yCb ), the variable nCurrS set equal to nCbSL, the variable cIdx set equal to 0, the (nCbSL)x(nCbSL) array predSamples set equal to predSamplesL, and the (nCbSL)x(nCbSL) array resSamples set equal to resSamplesL as inputs.

    - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the chroma coding block location ( xCb / 2, yCb / 2 ), the variable nCurrS set equal to nCbSC, the variable cIdx set equal to 1, the (nCbSC)x(nCbSC) array predSamples set equal to predSamplesCb, and the (nCbSC)x(nCbSC) array resSamples set equal to resSamplesCb as inputs.

    - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the chroma coding block location ( xCb / 2, yCb / 2 ), the variable nCurrS set equal to nCbSC, the variable cIdx set equal to 2, the (nCbSC)x(nCbSC) array predSamples set equal to predSamplesCr, and the (nCbSC)x(nCbSC) array resSamples set equal to resSamplesCr as inputs.

- I.8.5.7		Derivation process for a modified partitioning mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block.

The derivation process for a depth predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sampling interval sampInt equal to 4, the sample location ( xTb, yTb ) equal to ( xCb, yCb ), and the block size nTbS equal to nCbS as inputs, and the output is a binary partition pattern wedgePattern[ x ][ y ].

For p in the range of 0 to 1, inclusive and i in the range of 0 to 1, inclusive, partSum[ p ][ i ] is set equal to 0.

for( y = 0; y < nCbS ; y += 4 )
		for( x = 0; x < nCbS ; x += 4 ) { 
			segFlag = wedgePattern[ x ][ y ]
			partSum[ 0 ][ ( x < ( nCbS  >>  1 ) ) ? segFlag : !segFlag ]++
			partSum[ 1 ][ ( y < ( nCbS  >>  1 ) ) ? segFlag : !segFlag  ]++	(Error: Reference source not found-1)
		}

The variable partFlag is derived as specified in the following:

partFlag = 0
	maxPartSum = 0
	for( p = 0; p < 2; p++ )
		for( i = 0; i < 2; i++ ) {
			if( partSum[ p ][ i ] > maxPartSum ) {	(Error: Reference source not found-2)
				maxPartSum = partSum[ p ][ i ]
				partFlag = p
			}
		}

The variables x0 and y0 are derived and the variable PartMode is modified depending on partFlag as specified in the following:

  - x0 = partFlag ? xCb : xCb + nCbS / 2

  - y0 = partFlag ? yCb + nCbS / 2 : yCb

  - PartMode = partFlag ? SIZE_2NxN : SIZE_Nx2N

The following applies:

  - MergeIdx[ x0 ][ y0 ] = merge_idx[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-3)

  - MergeFlag[ x0 ][ y0°] = merge_flag[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-4)

  - InterPredIdc[ x0 ][ y0°] = inter_pred_idc[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-5)

For X in the range of 0 to 1, inclusive, the following applies:

  - PuRefIdxLX[ x0 ][ y0°] = ref_idx_lX[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-6)

  - MvpLXFlag[ x0 ][ y0°] = mvp_lX_flag[ xCb ][ yCb + nCbS / 2 ]	(Error: Reference source not found-7)

  - MvdLX[ x0 ][ y0°] = MvdLX[ xCb ][ yCb + nCbS/2 ]		(Error: Reference source not found-8)

- I.7.4.9.5	  Coding unit semantics

- dbbp_flag[ x0 ][ y0 ] equal to 1 specifies that depth based block partition is used for the current coding unit. dbbp_flag[ x0 ][ y0 ] equal to 0 specifies that depth based block partition is not used for the current coding unit. When not present, the value of dbbp_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When dbbp_flag[ x0 ][ y0 ] is equal to 1, PartMode is set equal to PART_2NxN.PartMode shall be equal to PART_2NxN or PART_Nx2N.

Additional Changes for Test2

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && CuPredMode[ x0 ][ y0 ] != MODE_INTRA && PartMode == PART_2NxN && PartMode == PART_Nx2N) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 4,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H1001-v2, April, 2014.

- HTM-11.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.