| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0100r1 |
| --- | --- |

| Title: | 3D-HEVC HLS: General comments on 3D-HEVC high level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Min Woo Park Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The following changes of 3D-HEVC high level syntax are proposed.

- Illumination compensation syntax elements are not present when NumDirectRefLyaers[ i ] is equal to 0.

- The syntax element iv_mv_pred_flag[ layerId ] in vps_extension2 is present only when NumDirectRefLayers[ nuh_layer_id ] is greater than 0.

- MvHevcCompatibilityFlag is replaced by mv_hevc_compatibility_flag[i] in VPS_extension2.

- iv_mv_scaling_flag is present when iv_mv_pred_flag is equal to 1.

# Proposal

For 3D-HEVC, general high level syntax improvements are proposed.

1.1. 	Conditional signalling of illumination compensation related syntax elements

The syntax elements of illumination compensation in slice header, slice_ic_enable_flag and slice_ic_disable_merge_zero_idx_flag are not needed when the current layer has no inter-layer prediction, because the illumination compensation is an inter-layer prediction from the reference layer. Therefore, when NumDirectRefLayers[nuh_layer_id] is equal to 0, the slice_ic_enable_flag and slice_ic_disable_merge_zero_idx_flag are not present in slice header and slice_ic_enable_flag is inferred to be equal to 0.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if( nuh_layer_id > 0 && !DepthFlag && !MvHevcCompatibilityFlag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| slice_ic_enable_flag | u(1) |
| if( slice_ic_enable_flag ) |  |
| slice_ic_disable_merge_zero_idx_flag | u(1) |
| } |  |
| … |  |

1.2. 	Conditional signalling of iv_mv_pred_flag in vps_extension2

The syntax element iv_mv_pred_flag[ layerId ] in vps_extension2 is present only when NumDirectRefLayers[ layerId ] is greater than 0.

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId != 0 ) { |  |
| if(NumDirectRefLayers[ layerId ] > 0) |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| log2_sub_pb_size_minus3[ layerId ] | ue(v) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| depth_based_blk_part_flag[ layerId ] | u(1) |
| } else { |  |
| mpi_flag[ layerId ] | u(1) |
| vps_depth_modes_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| vps_inter_sdc_flag[ layerId ] | u(1) |
| } |  |
| } |  |
| } |  |
| cp_precision | ue(v) |

1.3. MvHevcCompatibilityFlag is replaced by mv_hevc_compatibility_flag[i] in VPS_extension2

MvHevcCompatiblityFlag indicates whether all layers are compatible with MV-HEVC and is inferred from the presence of vps_extension2. However, each layer can or cannot be compatible with MV-HEVC according to its coding mode. Even if all layers in the original bitstream are not perfectly compatible with MV-HEVC, layers in output bitstream of sub-bitstream extraction process can be compatible with MV-HEVC. Then, the sub-bitstream is compatible with MV-HEVC. Therefore, indicating the MV-HEVC compatibility of each layer is helpful for sub-bitstream extraction. Moreover, the value of MvHevcCompatibilityFlag is inferred from the presence of vps_extension2. However, note that the presence of vps_extension2 is not equivalent to the MV-HEVC compatibility.

In this proposal, mv_hevc_compatibility_flag[i] signalled in vps_extension2 replaces the MvHevcCompatibilityFlag. When mv_hevc_compatibility_flag[i] is equal to 0, the i-th layer is compatible with MV-HEVC.

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| mv_hevc_compatibility_flag[i] | u(1) |
| } |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId != 0 && mv_hevc_compatibility_flag[i]) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| log2_sub_pb_size_minus3[ layerId ] | ue(v) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| depth_based_blk_part_flag[ layerId ] | u(1) |
| } else { |  |
| mpi_flag[ layerId ] | u(1) |
| vps_depth_modes_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| vps_inter_sdc_flag[ layerId ] | u(1) |
| } |  |
| } |  |
| } |  |
| … | ue(v) |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| else if( nuh_layer_id > 0 && !DepthFlag && !MvHevcCompatibilityFlag !mv_hevc_compatibility_flag[nuh_layer_id] ) { |  |
| slice_ic_enable_flag | u(1) |
| if( slice_ic_enable_flag ) |  |
| slice_ic_disable_merge_zero_idx_flag | u(1) |
| } |  |
| … |  |
| if( nuh_layer_id > 0 && cp_in_slice_segment_header_flag[ ViewIdx ] && !mv_hevc_compatibility_flag[nuh_layer_id]) |  |
| for ( j = 0; j < ViewIdx; j++ ) { |  |
| cp_scale[ j ] | se(v) |
| cp_off[ j ] | se(v) |
| cp_inv_scale_plus_scale[ j ] | se(v) |
| cp_inv_off_plus_off[ j ] | se(v) |
| } |  |
| … |  |
| } |  |

1.4. Conditional signalling of iv_mv_scaling_flag per layer

When iv_mv_pred_flag is equal to 0, the interview motion prediction is not used. Thus, iv_mv_scaling_flag is not needed when iv_mv_pred_flag is equal to 0. Therefore, it is proposed to signal iv_mv_scaling_flag only when iv_mv_pred_flag is equal to 1. Moreover, to be aligned with iv-mv_pred_flag[i], signalling iv_mv_scaling-flag[i] per layer seems more reasonable.

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId != 0 ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| if(iv_mv_pred_flag[ layered ]) |  |
| iv_mv_scaling_flag[ layerId ] | u(1) |
| log2_sub_pb_size_minus3[ layerId ] | ue(v) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| depth_based_blk_part_flag[ layerId ] | u(1) |
| } else { |  |
| mpi_flag[ layerId ] | u(1) |
| vps_depth_modes_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| vps_inter_sdc_flag[ layerId ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| iv_mv_scaling_flag | u(1) |
| log2_mpi_sub_pb_size_minus3 | ue(v) |
| } |  |

iv_mv_scaling_flag[ layereId] equal to 1 specifies that motion vectors used for inter-view prediction in a layer with nuh_layer_id equal to layerId may be scaled based on ViewId[ layerId ] values. iv_mv_scaling_flag[ layereId]  equal to 0 specifies that motion vectors used for inter-view prediction in a layer with nuh_layer_id equal to layerId are not scaled based on ViewId[ layerId ] values. When not present, the value of iv_mv_scaling_flag[ layereId] is inferred to be equal to 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).