| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0101 |
| --- | --- |

| Title: | 3D-HEVC HLS: Dependency between texture and depthmap |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Jin Young Lee Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In order to explicitly indicate the dependencies between the texture pictures and the depthmap pictures referenced by depth-oriented texture coding tools, it is asserted that the direct_dependency_flags[i][j] value of the j-th layer with depthmap pictures which is referenced by the i-th layer with texture pictures shall be set equal to 1 and vice versa. Moreover, a condition checking of the VpsDepthFlag values is proposed in the reference picture list derivation, to prevent inserting the depthmpa picture into the reference picture list of the texture picture.

# Proposal

Many coding tools in 3D-HEVC have cross dependency between texture and depthmap pictures. Depth-oriented texture coding tools, such as depth oriented neighboring block disparity vector (DoNBDV), view synthesis prediction (VSP) and depth-based block partitioning (DBBP), reference the depthmap to code the corresponding texture picture. On the other hand, texture-oriented depth coding tools, which are motion parameter inheritance (MPI) and Quad-tree limitation & prediction coding (QTLPC), reference the corresponding texture pictures. The availabilities of those tools are indicated by flags in vps_extension2 for each layer for each tool. However, direct_dependency_flags in vps_extension do not indicate the dependencies between texture and depthmap, because depthmap pictures do not refer to the decoded texture pictures as references for inter-view disparity compensation and texture pictures do not refer to the decoded depthmap pictures as reference pictures.

This is a contradiction in the current 3D-HEVC spec. Even if a layer is dependent on another layer, it is not explicitly indicated by direct_dependency_flag. The problem caused by this implicit dependency is that the referenced depthmap pictures will not be stored in the decoded picture buffer (DPB) of the texture picture, because the DPB of each layer contains the decoded pictures which are explicitly indicated by direct_dependency_flag in VPS and inter-layer RPS in slice header. Furthermore, the timing to mark the referenced depthmap pictures as “unused for reference” is not clear, because the dependencies between the texture and depthmap pictures are not explicitly indicated by syntaxes.

Another problem is that an independent layer sub-tree consists of layers referring to only layers belonging to the same sub-tree internally, based on direct_dependency_flag. When independent layer sub-trees can be extracted into a sub-bitstream, the links between the texture pictures and the depthmap pictures can be broken. Thus, the presence of the corresponding texture or depthmap pictures cannot be guaranteed after sub-bitstream extraction process, when direct_dependency_flag in VPS does not indicate the layer dependency between those pictures.

By setting the value of direct_dependency_flag in VPS, the dependency between the texture and the corresponding depthmap can be indicated without spec. changes. And, the interlayer RPS in slice header can indicate the presence of the associated depthmap pictures for decoding the current texture picture. However, since the insertion of the decoded texture picture with direct_dependency_flag equal to 1 into the interlayer RPS of the depthmap pictures shall be prevented, the modification of the interlayer RPS process with a condition check of VpsDepthFlag values is proposed as the following specification text.

# Proposed specification text

- H.8.1.2  Decoding process for inter-layer reference picture set

Outputs of this process are updated lists of inter-layer pictures RefPicSetInterLayer0 and RefPicSetInterLayer1 and the variables NumActiveRefLayerPics0 and NumActiveRefLayerPics1.

The lists RefPicSetInterLayer0 and RefPicSetInterLayer1 are first emptied, NumActiveRefLayerPics0 and NumActiveRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	refPicSet0Flag = ( ViewId[ nuh_layer_id ]  <=  ViewId[ 0 ]   &&
								ViewId[ nuh_layer_id ]  <=  ViewId[ RefPicLayerId[ i ] ] )  | |
					( ViewId[ nuh_layer_id ]  >=  ViewId[ 0 ]  &&
								ViewId[ nuh_layer_id ]  >=  ViewId[ RefPicLayerId[ i ] ] ) ) 
	if(VpsDepthFlag[nuh_layer_id ] == VpsDepthFlag [ RefPicLayerId[ i ] ] ) {
   	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
										nuh_layer_id equal to RefPicLayerId[ i ] ) {
			if( refPicSet0Flag ) {
				RefPicSetInterLayer0[ NumActiveRefLayerPics0 ] = picX
				RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] is marked as "used for long-term reference"
			} else {
				RefPicSetInterLayer1[ NumActiveRefLayerPics1 ] = picX
				RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] is marked as "used for long-term reference"
			}
		} else {
			if( refPicSet0Flag )
				RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] = "no reference picture"
			else
				RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] = "no reference picture"
		}
   }
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer0 or RefPicSetInterLayer1.

There shall be no picture that has discardable_flag equal to 1 in RefPicSetInterLayer0 or RefPicSetInterLayer1.

If the current picture is a RADL picture, there shall be no entry in RefPicSetInterLayer0 or RefPicSetInterLayer1 that is a RASL picture.

NOTE – An access unit may contain both RASL and RADL pictures.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).