| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0102r1 |
| --- | --- |

| Title: | 3D-AVC defect correction: texture view dependencies missing from SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland Xuehui Huang University of Science and Technology of China, Hefei, P. R. China | Tel: Email: Email: | +358 10 4488000 firstname.lastname@nokia.com hxhy@mail.ustc.edu.cn |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that the signalling of inter-view dependencies between texture views is missing in 3D-AVC (i.e. SPSs for the Enhanced Multiview Depth High profile). In the proposed fix, the seq_parameter_set_3davc_extension( ) is appended to include the inter-view dependencies between texture views, when the texture and depth views are unpaired (i.e. for some views a texture view is present while a depth view is absent, or vice versa).

In revision 1, missing curly brackets were added to the proposed syntax.

# Problem Statement

The intent in the SPS design for 3D-AVC has been to share the same SPS between 3D-AVC-coded depth and texture views.

In a 3D-AVC bitstream with three texture and depth views that are all 3D-AVC-coded except the texture base view, the SPS activation could take place as follows:

- Texture base view: SPS with ID 0, High profile

- Texture non-base views and all depth views: SPS with ID 1, Enhanced Multiview Depth High profile (profile_idc 139)

In another example, the bitstream contains three texture and depth views, out of which the texture base view conforms to the High profile, two texture views conform to MVC and two depth views conform to MVC+D, while the remaining views conform to 3D-AVC. The SPS activation could take place as follows:

- Texture base view: SPS with ID 0, High profile

- Texture view 1: SPS with ID 1, Stereo High profile

- Texture view 2: SPS with ID 2, Enhanced Multiview Depth High profile (profile_idc 139)

- Depth base view and depth view 1, SPS with ID 3, Multiview Depth High profile (profile_idc 138)

- Depth view 2: SPS with ID 2, Enhanced Multiview Depth High profile (profile_idc 139)

The SPS syntax is specified as follows:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 134 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 138 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex Error: Reference source not found */ |  |  |
| } else if( profile_idc = = 139 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| seq_parameter_set_3davc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } |  |  |
| additional_extension2_flag | 0 | u(1) |
| if( additional_extension2_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension2_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

As can be observed from the syntax, the Multiview Depth High profile and the Enhanced Multiview Depth High profile utilize the same syntax structure, seq_parameter_set_mvcd_extension( ), which among other things specifies the inter-view prediction dependencies between depth views:

| seq_parameter_set_mvcd_extension( ) { | C | Descriptor |
| --- | --- | --- |
| num_views_minus1 | 0 | ue(v) |
| for( i = 0, NumDepthViews = 0; i <= num_views_minus1; i++ ) { |  |  |
| view_id[ i ] | 0 | ue(v) |
| depth_view_present_flag[ i ] | 0 | u(1) |
| DepthViewId[ NumDepthViews ] = view_id[ i ] |  |  |
| NumDepthViews += depth_view_present_flag[ i ] |  |  |
| texture_view_present_flag[ i ] | 0 | u(1) |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) |  |  |
| if( depth_view_present_flag[ i ] ) { |  |  |
| num_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l0[ i ]; j++ ) |  |  |
| anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l1[ i ]; j++ ) |  |  |
| anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) |  |  |
| if( depth_view_present_flag[ i ] ) { |  |  |
| num_non_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l0[ i ]; j++ ) |  |  |
| non_anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_non_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l1[ i ]; j++ ) |  |  |
| non_anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| … |  |  |

In MVC+D, the texture and depth views refer to a different SPS. The inter-view dependencies between texture views are indicated in the SPS where the profile_idc indicates an MVC profile and which is referred to by all texture views. However, the signalling of inter-view dependencies between texture views is missing in 3D-AVC (i.e. SPSs for the Enhanced Multiview Depth High profile).

# Proposal

In the proposed fix, the seq_parameter_set_3davc_extension( ) is appended to include the inter-view dependencies between texture views, when the texture and depth views are unpaired (i.e. for some views a texture view is present while a depth view is absent, or vice versa).

The specification is proposed to be amended as follows.

Add the following into subclause I.7.4.2.1.4, just before the semantics of applicable_op_depth_flag[ i ][ j ][ k ]. (Also add an equation number and update the other equation numbers in Annex I.)

The variable AllViewsPairedFlag is derived as follows:

AllViewsPairedFlag = 1
for( i = 1; i <= num_views_minus1; i++ )
	AllViewsPairedFlag = ( AllViewsPairedFlag  &  depth_view_present_flag[ i ]  &  
		texture_view_present_flag[ i ] )

Replace subclause J.7.3.2.1.5 with the following (additions indicated with turquoise highlights):

| seq_parameter_set_3davc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( NumDepthViews > 0 ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| for( i = 0; i < NumDepthViews; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc ) { |  |  |
| depth_ranges( NumDepthViews, 2, 0 ) |  |  |
| vsp_param( NumDepthViews, 2, 0 ) |  |  |
| } |  |  |
| reduced_resolution_flag | 0 | u(1) |
| if( reduced_resolution_flag ) { |  |  |
| depth_pic_width_in_mbs_minus1 | 0 | ue(v) |
| depth_pic_height_in_map_units_minus1 | 0 | ue(v) |
| depth_hor_mult_minus1 | 0 | ue(v) |
| depth_ver_mult_minus1 | 0 | ue(v) |
| depth_hor_rsh | 0 | ue(v) |
| depth_ver_rsh | 0 | ue(v) |
| } |  |  |
| depth_frame_cropping_flag | 0 | u(1) |
| if( depth_frame_cropping_flag ) { |  |  |
| depth_frame_crop_left_offset | 0 | ue(v) |
| depth_frame_crop_right_offset | 0 | ue(v) |
| depth_frame_crop_top_offset | 0 | ue(v) |
| depth_frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| grid_pos_num_views | 0 | ue(v) |
| for( i = 0; i < grid_pos_num_views; i++ ) { |  |  |
| grid_pos_view_id[ i ] | 0 | ue(v) |
| grid_pos_x[ grid_pos_view_id[ i ] ] | 0 | se(v) |
| grid_pos_y[ grid_pos_view_id[ i ] ] | 0 | se(v) |
| } |  |  |
| slice_header_prediction_flag | 0 | u(1) |
| seq_view_synthesis_flag | 0 | u(1) |
| } |  |  |
| alc_sps_enable_flag | 0 | u(1) |
| enable_rle_skip_flag | 0 | u(1) |
| if( !AllViewsPairedFlag ) { |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) |  |  |
| if( texture_view_present_flag[ i ] ) { |  |  |
| num_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l0[ i ]; j++ ) |  |  |
| anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l1[ i ]; j++ ) |  |  |
| anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) |  |  |
| if( texture_view_present_flag[ i ] ) { |  |  |
| num_non_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l0[ i ]; j++ ) |  |  |
| non_anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_non_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l1[ i ]; j++ ) |  |  |
| non_anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

Add at the end of subclause J.7.4.2.1.5:

For num_anchor_refs_l0[ i ], anchor_ref_l0[ i ][ j ], num_anchor_refs_l1[ i ], anchor_ref_l1[ i ][ j ], num_non_anchor_refs_l0[ i ], non_anchor_ref_l0[ i ][ j ], num_non_anchor_refs_l1[ i ], and non_anchor_ref_l1[ i ][ j ] the semantics specified in subclause H.7.4.2.1.4 apply with the substitution of texture view component for view component. When num_anchor_refs_l0[ i ], anchor_ref_l0[ i ][ j ], num_anchor_refs_l1[ i ], anchor_ref_l1[ i ][ j ], num_non_anchor_refs_l0[ i ], non_anchor_ref_l0[ i ][ j ], num_non_anchor_refs_l1[ i ], and non_anchor_ref_l1[ i ][ j ] are not present, they are inferred to have the same values as the respective syntax elements in the seq_parameter_set_mvcd_extension( ) syntax structure in the same subset_seq_parameter_set_rbsp( ) syntax structure that also contains this seq_parameter_set_3davc_extension( ) syntax structure.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The authors are not aware of University of Science and Technology of China having any current or pending patent rights relating to the technology described in this contribution.