| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0105 |
| --- | --- |

| Title: | Adaptive depth access for VSP enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to change the reference points in VSP from the current always four corner points to four or one point to improve coding efficiency. It is reported that the gain of proposed method is 0.08, 0.09 and 0.05 in texture, video and synthesis respectively.

# Introduction

The contribution of JCT-3V G0035 [1] has proposed one point access per VSP subblock instead of the four corner access in order to simplify the depth access. Then, it was found that one point access was better than four corner access in terms of overall coding efficiency. However in the meeting, the coordination of the point (x+2, y+2) was questioned and the loss of one sequence in GT fly was concerned.  The proposal itself was not adopted because the simplification impact was considered not so large.

In this contribution, we focus on coding improvement aspect and found an improved depth access method.  This proposal addresses (x+2, y+2) coordination issue and GT fly’s coding efficiency concern related to the former contribution.

# Proposed Method

In the proposed method, the depth access per sub-block is selected depending on the PU as follows.

- Four corner access (If PU is 2Nx2N)

- One center access (Otherwise, PU is not 2Nx2N)

Fig. 1 Proposed VSP depth access in the sub block

Note: Because this depth access decision is PU-level, there should be no significant complexity increase.

# WD change (relative to JCT3V-H1001_v2)

I.8.5.5.2 Derivation process for a disparity sample array

The array disparitySamples is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH ) − 1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW ) − 1 ), inclusive, the following applies:

    - The variable maxDep is set equal to −1 and modified as specified in the following:

xSubB = sBx * nSubBlkW
			ySubB = sBy * nSubBlkH

If nPSW is equal to nPSH, the following apply,

xP0 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB )
				yP0 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB ) 
				xP1 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB + nSubBlkW − 1 )
				yP1 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB + nSubBlkH − 1 )
				maxDep = Max( maxDep, refDepPels[ xP0 ][ yP0 ] )
				maxDep = Max( maxDep, refDepPels[ xP0 ][ yP1] )
				maxDep = Max( maxDep, refDepPels[ xP1][ yP0 ] )
				maxDep = Max( maxDep, refDepPels[ xP1][ yP1 ] )

Otherwise ( nPSW is not equal to nPSH), the following apply,

xP0 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB/2 )
				yP0 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB/2 ) 
				maxDep = refDepPels[ xP0 ][ yP0 ]

    - The values of the array depthSamples are modified as specified in the following:

for ( yOff = 0; yOff < nSubBlkH; yOff++ )
				for( xOff = 0; xOff < nSubBlkW; xOff++ ) {
					x = xSubB + xOff 
					y = ySubB + yOff 
					disparitySamples[ x ][ y ] = DepthToDisparityB[ refViewIdx ][ maxDep ]

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | -0.02% | -0.02% | -0.03% | 100.2% | 106.1% | 101.0% |
| Kendo | 0.0% | -0.2% | 0.0% | -0.04% | -0.06% | -0.09% | 100.3% | 101.9% | 101.9% |
| Newspaper_CC | 0.0% | -0.1% | 0.0% | -0.02% | -0.02% | 0.01% | 100.7% | 99.2% | 100.4% |
| GT_Fly | 0.0% | -0.2% | -0.1% | -0.05% | -0.06% | -0.03% | 101.0% | 101.3% | 100.3% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.1% | -0.04% | -0.05% | 0.01% | 100.5% | 100.2% | 99.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | -0.02% | -0.02% | 0.03% | 100.2% | 99.7% | 100.3% |
| Undo_Dancer | 0.0% | -1.2% | -1.1% | -0.37% | -0.37% | -0.27% | 100.4% | 100.0% | 99.9% |
| Shark | 0.0% | -0.3% | -0.2% | -0.09% | -0.08% | -0.03% | 100.2% | 100.3% | 100.0% |
| 1024x768 | 0.0% | -0.1% | 0.0% | -0.03% | -0.04% | -0.04% | 100.4% | 102.4% | 101.1% |
| 1920x1088 | 0.0% | -0.4% | -0.3% | -0.12% | -0.12% | -0.06% | 100.4% | 100.3% | 100.0% |
| average | 0.0% | -0.3% | -0.2% | -0.08% | -0.09% | -0.05% | 100.4% | 101.1% | 100.4% |

# Conclusion

It is proposed to change depth access in VSP sub-block. Because the proposed small change achieves 0.09 % and 0.05 % coding gain in video and synthesis respectively, it is recommended to adopt this method in the next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- T. Ikai, “VSP depth access simplification”, JCT3V-G0035, 7th Meeting: San José, US, 11–17 Jan. 2014.