| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0106 |
| --- | --- |

| Title: | Further VSP inheritance simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to simplify VSP inheritance by referring VSP flag at only one candidate (A1). It is reported that BD rate change is -0.01, -0.01 and 0.00 % respectively.

# Introduction

VSP inheritance is effective because it adaptively inserts the VSP candidates in early position of merge candidate list if there are VSP usages in the neighbouring blocks. Technique has already been simplified by removing DV inheritance and resolving additional line buffer issue by restricting CTU inter-boundary access for upper candidate [1]. However further simplification can be found. Thus it is proposed in this contribution.

# Proposed Method

The proposal refers neighbouring VSP flag at only one candidate (A1). With this change, the VSP inheritance access decreases from 5 (A1, B1, B0, A0, B2) to 1.

Fig. 1 A1 only access in VSP inheritance (proposed)

# WD change (relative to JCT3V-H1001_v2)

- I.8.5.3.2.17 Derivation process for a view synthesis prediction flag

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current prediction block,

- a merge candidate indicator N, specifying the merge candidate.

Outputs of this process are:

- a variable mergeCandIsVspFlag specifying, whether the merge candidate is a view synthesis prediction  merge candidate.

- The variable mergeCandIsVspFlag is derived as specified in the following:

- If N is equal to VSP, mergeCandIsVspFlag is set equal to 1,

- Otherwise, if N is equal to A1, B1, B0, A0, or B2, the following applies:

  - The luma position ( xN, yN ) is specified in Table Error: Reference source not found-1 depending on N.

  - If one of the following conditions is true, the variable mergeCandIsVspFlag is set equal to VspModeFlag[ xNxPb − 1 ][ yNyPb + nPbH − 1 ].

    - N is equal to A1 or A0

    - N is equal to B0, B1, or B2 and ( yN  >>  Log2CtbSizeY ) is equal to ( yCb >> Log2CtbSizeY )

  - Otherwise, ,mergeCandIsVspFlag is set equal to 0.

- Otherwise, mergeCandIsVspFlag is set equal to 0.

Table Error: Reference source not found-1 – Specification of xN and yN depending on N

| N | A1 | B1 | B0 | A0 | B2 |
| --- | --- | --- | --- | --- | --- |
| xN | xPb − 1 | xPb + nPbW − 1 | xPb + nPbW | xPb − 1 | xPb − 1 |
| yN | yPb + nPbH − 1 | yPb − 1 | yPb − 1 | yPb + nPbH | yPb − 1 |

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.00% | 0.01% | 0.00% | 99.9% | 102.2% | 100.6% |
| Kendo | 0.0% | -0.1% | -0.1% | -0.05% | -0.05% | -0.01% | 100.5% | 102.9% | 102.2% |
| Newspaper_CC | 0.0% | 0.0% | -0.1% | -0.01% | 0.01% | -0.03% | 100.6% | 99.8% | 100.8% |
| GT_Fly | 0.0% | -0.1% | -0.1% | -0.03% | -0.04% | 0.00% | 101.2% | 100.3% | 99.6% |
| Poznan_Hall2 | 0.0% | 0.1% | 0.0% | 0.02% | 0.04% | 0.00% | 99.8% | 99.6% | 99.7% |
| Poznan_Street | 0.0% | 0.1% | -0.1% | 0.00% | -0.01% | 0.03% | 100.4% | 99.8% | 100.0% |
| Undo_Dancer | 0.0% | 0.1% | 0.1% | 0.00% | 0.01% | 0.00% | 100.6% | 100.7% | 100.1% |
| Shark | 0.0% | -0.2% | 0.0% | -0.04% | -0.04% | -0.03% | 100.1% | 100.8% | 100.3% |
| 1024x768 | 0.0% | -0.1% | 0.0% | -0.02% | -0.01% | -0.01% | 100.3% | 101.6% | 101.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 0.00% | 100.4% | 100.2% | 100.0% |
| average | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 0.00% | 100.4% | 100.8% | 100.4% |

# Conclusion

It is proposed to simplify VSP inheritance. Because it removes number of VSP inheritance access with no loss, it is recommended to adopt this method into the next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 4,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-H1001, 8th Meeting: Valencia, ES, 29 Mar. – 4 Apr. 2014.