| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0107 |
| --- | --- |

| Title: | Partition decision unification for DBBP and VSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a unified partition decision for DBBP and VSP. The proposed method utilizes the unified partition method which uses four corner of the target block. The BD-rate change is reported as -0.01 %, -0.01 % and -0.01% in texture, video and synthesis respectively.

# Introduction

The DBBP partition simplification in JCT3V-H0073 [1] was adopted at the last meeting, understanding the technique was the common part of several simplifications and can be agreeable among proponents as the first step simplification. However the DBBP partition decision is considered still complex and DBBP partitioning is different from VSP partitioning although its purpose are the same. The united partitioning of DBBP and VSP has been suggested in JCT3V-H0065. The proposal is follow up for this context.

# Proposal

We propose to use the same partitioning decision for DBBP and VSP. Partitioning method was the same as JCT3V-H0068 [3], which originally proposed to simplify DBBP partitioning.

# WD change (relative to JCT3V-H1001_v2)

- I.8.5.5.2 Derivation process for a disparity sample array

- …

- Depending on partIdc, the variables nSubBlkW, nSubBlkH and horSplitFlag are derived as specified in the following:

- If partIdc is equal to 0, the variables nSubBlkW, nSubBlkH, and horSplitFlag are set equal to nPSW ,nPSH, and 0, respectively.

- Otherwise, if partIdc is equal to 1, the variables nSubBlkW, nSubBlkH, and horSplitFlag are set equal to 1, 1, and 0, respectively.

- Otherwise (partIdc is equal to 2), the following applies:

  - The variable minSubBlkSizeFlag is derived as specified in the following:

      - minSubBlkSizeFlag = ( nPSW % 8  !=  0)  | |  ( nPSH % 8   !=  0 )	(Error: Reference source not found-1)

  - Depending on the value of minSubBlkSizeFlag, the following applies:

    - If minSubBlkSizeFlag is equal to 1, the following applies:

        - horSplitFlag =  ( nPSH % 8  !=  0 )		(Error: Reference source not found-2)

    - Otherwise (minSubBlkSizeFlag is equal to 0), the following applies:

        - xP0 = Clip3( 0, pic_width_in_luma_samples − 1, xTL )	(Error: Reference source not found-3)

        - yP0 = Clip3( 0, pic_height_in_luma_samples − 1, yTL )	(Error: Reference source not found-4)

        - xP1 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + nPSW − 1 )	(Error: Reference source not found-5)

        - yP1 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + nPSH  − 1 )	(Error: Reference source not found-6)

    - horSplitFlag = ( refDepPels[ xP0 ][ yP0 ] < refDepPels[ xP1 ][ yP1 ] )
	  = =  ( refDepPels[ xP1 ][ yP0 ] < refDepPels[ xP0 ][ yP1] ) )	(Error: Reference source not found-7)

    - horSplitFlag = ( abs(refDepPels[ xP0 ][ yP0 ] − refDepPels[ xP1 ][ yP0 ] )
	< abs( refDepPels[ xP0 ][ yP0 ] − refDepPels[ xP0 ][ yP1] ) )	(Error: Reference source not found-8)

- I.8.5.7 Derivation process for a modified partitioning mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block.

The derivation process for a depth predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sampling interval sampInt equal to 4, the sample location ( xTb, yTb ) equal to ( xCb, yCb ), and the block size nTbS equal to nCbS as inputs, and the output is a binary partition pattern wedgePattern[ x ][ y ] and a flag horSplitFlag.

For p in the range of 0 to 1, inclusive and i in the range of 0 to 1, inclusive, partSum[ p ][ i ] is set equal to 0.

for( y = 0; y < nCbS ; y += 4 )
		for( x = 0; x < nCbS ; x += 4 ) { 
			segFlag = wedgePattern[ x ][ y ]
			partSum[ 0 ][ ( x < ( nCbS  >>  1 ) ) ? segFlag : !segFlag ]++
			partSum[ 1 ][ ( y < ( nCbS  >>  1 ) ) ? segFlag : !segFlag  ]++	(Error: Reference source not found-9)
		}

The variable partFlag is derived as specified in the following:

partFlag = 0
	maxPartSum = 0
	for( p = 0; p < 2; p++ )
		for( i = 0; i < 2; i++ ) {
			if( partSum[ p ][ i ] > maxPartSum ) {	(Error: Reference source not found-10)
				maxPartSum = partSum[ p ][ i ]
				partFlag = p
			}
		}

The variables x0 and y0 are derived and the variable PartMode is modified depending on horSplitFlagpartFlag as specified in the following:

  - x0 = horSplitFlagpartFlag ? xCb : xCb + nCbS / 2

  - y0 = horSplitFlagpartFlag ? yCb + nCbS / 2 : yCb

  - PartMode = partFlaghorSplitFlag ? SIZE_2NxN : SIZE_Nx2N

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.03% | 0.03% | 0.00% | 100.0% | 103.0% | 101.5% |
| Kendo | 0.0% | -0.2% | 0.0% | -0.04% | -0.09% | -0.09% | 100.2% | 98.8% | 101.1% |
| Newspaper_CC | 0.0% | 0.1% | 0.0% | 0.01% | 0.03% | -0.02% | 100.7% | 97.9% | 101.2% |
| GT_Fly | 0.0% | 0.1% | -0.1% | 0.00% | 0.01% | -0.02% | 101.1% | 99.6% | 99.9% |
| Poznan_Hall2 | 0.0% | -0.2% | -0.1% | -0.04% | -0.03% | -0.02% | 99.9% | 101.3% | 100.0% |
| Poznan_Street | 0.0% | 0.1% | -0.1% | -0.01% | 0.00% | 0.02% | 100.4% | 99.8% | 99.9% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | -0.01% | 0.00% | 0.02% | 100.7% | 100.7% | 99.7% |
| Shark | 0.0% | -0.1% | 0.1% | 0.00% | 0.00% | 0.01% | 100.2% | 99.8% | 100.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.04% | 100.3% | 99.9% | 101.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | -0.01% | 0.00% | 0.00% | 100.5% | 100.3% | 99.9% |
| average | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | -0.01% | 100.4% | 100.1% | 100.4% |

# Conclusion

It is proposed to simplify DBBP filtering, which utilize one dimensional filtering decided by PartMode. Because coding efficiency impact is negligible with this simplification, it is recommended to adopt this method into the next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Zhang, K. Zhang, J. An, H. Huang, J.-L. Lin, S. Lei, “Simplification on depth-based block partitioning (DBBP)”, JCT3V-H0073, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.

- T. Ikai, “DBBP simplification”, JCT3V-H0065, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.

- M. W. Park, J. Y. Lee, C. Kim, “Simplification of DBBP in 3D-HEVC”, JCT3V-H0068, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.