| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0108 |
| --- | --- |

| Title: | Disparity vector unification for DBBP and VSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a unification of DMM and VSP for disparity vector (DV). As JCT3V-H0069 reported, DBBP uses DoNBDV for depth access, which requires depth access twice as opposed to other related component such as VSP, which uses NBDV for depth access. In this proposal, we propose to use DoNBDV in DBBP and VSP only when the PU size is larger than 8x8, otherwise we use NBDV. It is asserted that the proposal reduces memory bandwidth complexity in DBBP with unified depth access between VSP and DBBP. It was reported that the gain of proposed method is 0.03 %, 0.03 % and 0.02 % in texture, video and synthesis respectively.

# Introduction

Both DBBP and VSP require depth access while different disparity vector is used. Specifically DBBP uses DoNBDV and VSP uses NBDV, resulting design inconsistency regarding depth access.

We think DoNBDV usage, which requires depth access twice, is problematic but the twice depth access may be not so problem when it is larger than 8x8 PU. Specifically, complexity of 16x8 DoNBDV access is comparable to that of two 8x8 NBDV access. So it is not so much problem. In this understanding, we suggest to reduce the worst case complexity to disallow DoNBDV in 8x8 block and we suggest to allow DoNBDV in 16x8 or larger block for coding efficiency. Then DBBP and VSP can be unified with the worst case complexity reduction.

Fig.1 Different depth access in DBBP and VSP (current)

# Proposal

The disparity vector is decided depending on PU size for DBBP and VSP

- DoNBDV (if nPbW + nPbH is larger than 16)

- NBDV (Otherwise)

Fig.1 Unified depth access in DBBP and VSP (proposed)

It is asserted that the unification has the following benefits.

- In 8x8 CU, both DBBP and VSP doesn’t fetch depth block twice. This design consistency may help implementation and encoder decision.

- Reduce memory bandwidth of DBBP.

- Keep VSP’s good characteristics of relatively small complexity, thanks to the specified MVs, small number of operations and restricted memory bandwidth, by continuing to restrict DoNBDV usage in 8x8 CU. No additional complexity increase in VSP part.

- Small gain

# WD change (relative to JCT3V-H1001_v2)

I.8.5.3.2.13 Derivation process for a view synthesis prediction merge candidate

- The variable availableFlagVSP is set equal to 0, the variables predFlagL0VSP and predFlagL1VSP are set equal to 0, and the variables refIdxL0VSP and refIdxL1VSP are set equal to −1.

- For X in the range of 0 to 1, inclusive, the following applies:

  - For i in the range of 0 to NumRefPicsLX − 1, inclusive, the following applies:

    - When availableFlagVSP is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xPb ][ yPb ], the following applies:

      - The variable predFlagLXVSP, mvLXVSP, refIdxLXVSP, and availableFlagVSP are modified as specified in the following:

          - predFlagLXVSP = 1		(Error: Reference source not found-1)

          - mvLXVSP = nPbW + nPbH > 16 ? MvRefinedDisp[ xPb ][ yPb ] : MvDisp[ xPb ][ yPb ]		(Error: Reference source not found-2)

          - refIdxLXVSP = i		(Error: Reference source not found-3)

          - availableFlagVSP = 1		(Error: Reference source not found-4)

      - The derivation process for the luma motion vectors for view synthesis prediction merge candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the reference index refIdxLXVSP, the disparity vector mvLXVSP, and the view order index RefViewIdx[ xPb ][ yPb ] as inputs.

I.8.5.3.2.13.1 Derivation process for luma motion vectors for view synthesis prediction merge candidate

The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xPb, yPb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPbW , the variable nPbH and the variable partIdc equal to 2 as inputs, and the output is the array disparitySamples of size (nPbW)x(nPbH) and the flag horSplitFlag.

- I.8.5.8 Derivation process for a depth predicted contour pattern

…

The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations ( xP, yP ) equal to ( xTb, yTb ), the disparity vector mvDisp equal to nTbS + nTbS > 16 ? MvRefinedDisp[ xTb ][ yTb ] : MvDisp[ xTb ][ yTb ], the view identifier refViewIdx equal to RefViewIdx[ xTb ][ yTb ], the variables nPbW and nPbH, equal to nTbS, and the variable partIdc equal to 1 as inputs, and the output is the array refSamples of size (nTbS)x(nTbS) and the flag horSplitFlag.

# Complexity Analysis

Table 1 shows the complexity analysis data. As shown in the table, the worst case memory bandwidth of DBBP decreases significantly from the current 128 % to 114 %. The VSP’s worst case memory bandwidth increase is small (by up to 2 %).

|  |  | Number of operation [%] |  | Data transfer rate [%] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3D-HEVC |  | Mult [%] | Add [%] | Partial depth access in DoNBDV |  |  |  | Full depth access in DoNBDV |  |  |  |
|  |  |  |  | 1x1 | 4x2 | 4x4 | 8x2 | 1x1 | 4x2 | 4x4 | 8x2 |
|  | MC | 100% | 100% | 101% | 103% | 105% | 106% | 110% | 113% | 111% | 114% |
|  | VSP (current) | 18% | 17% | 41% | 60% | 60% | 78% | 50% | 70% | 65% | 94% |
|  | DBBP (current) | 100% | 127% | 111% | 116% | 116% | 119% | 120% | 125% | 123% | 128% |
|  | VSP (proposal) | 18% | 17% | 41% | 61% | 60% | 78% | 59% | 72% | 66% | 94% |
|  | DBBP (proposal) | 100% | 127% | 110% | 113% | 111% | 114% | 110% | 113% | 111% | 114% |

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.04% | 100.1% | 104.1% | 100.1% |
| Kendo | 0.0% | -0.1% | 0.1% | -0.01% | -0.04% | -0.04% | 100.6% | 101.1% | 102.1% |
| Newspaper_CC | 0.0% | -0.2% | 0.0% | -0.03% | 0.02% | -0.01% | 100.6% | 100.0% | 100.8% |
| GT_Fly | 0.0% | -0.3% | -0.3% | -0.04% | -0.04% | -0.07% | 100.7% | 99.8% | 99.9% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.3% | -0.04% | -0.03% | -0.01% | 100.4% | 100.3% | 100.5% |
| Poznan_Street | 0.0% | 0.0% | -0.1% | -0.02% | -0.02% | 0.01% | 100.2% | 100.4% | 100.3% |
| Undo_Dancer | 0.0% | -0.1% | -0.5% | -0.06% | -0.04% | 0.02% | 100.6% | 100.3% | 100.2% |
| Shark | 0.0% | -0.2% | -0.4% | -0.06% | -0.05% | -0.03% | 100.3% | 99.7% | 100.9% |
| 1024x768 | 0.0% | -0.1% | 0.0% | -0.01% | -0.01% | -0.03% | 100.4% | 101.7% | 101.0% |
| 1920x1088 | 0.0% | -0.1% | -0.3% | -0.04% | -0.04% | -0.01% | 100.4% | 100.1% | 100.4% |
| average | 0.0% | -0.1% | -0.2% | -0.03% | -0.03% | -0.02% | 100.4% | 100.7% | 100.6% |

# Conclusion

It is proposed to unify disparity vector in DBBP and VSP.  Because this proposal reduces the worst case complexity with unification of DBBP and VSP, it is recommended to adopt this method into the next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. W. Park, J. Y. Lee, C. Kim, “Disparity Vector for DBBP in 3D-HEVC”, JCT3V-H0069, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.