| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0109_r2 |
| --- | --- |

| Title: | One dimensional DBBP boundary filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a simplification for DBBP filtering. The proposal replaces two dimensional horizontal and vertical filtering with one dimensional horizontal or vertical filtering, which decided on the basis of PartMode. It is reported that the BD-rate change is 0.01 %, 0.01 % and 0.01 % in texture, video, and synthesis respectively.

Revision1 added option2, which further simplifies the decision by replacing left/current/right (top/current/bottom) based decision with left/right (top/bottom) based decision. It is reported that the BD-rate change is 0.01 %, 0.01 % and 0.00 % in texture, video, and synthesis respectively.

Revision2 revised option2’s WD as attachment to respond a request of DBBP BoG.

- Introduction

Partition boundary filtering in DBBP of JCT3V-H0104 [1] was adopted at the Valencia meeting. Although the filtering have some coding gain, it imposes two dimensional (separable but sequential) filtering along with left, right, top and bottom neighbouring pixel checks. This kind of two dimensional filtering has much more complex compared to other pixel level filtering technique, e.g. SAO which only utilizes one dimensional filtering. Furthermore, if unit size is considered, DBBP filtering can be said to be more changing since DBBP filtering is CU based (smallest unit is 8x8) while SAO filtering is CTU based (smallest unit is 16x16).

# Proposal

We propose to use one dimensional filtering for each DBBP block. The filtering direction is decided based on partitioning mode, PartMode (2NxN or Nx2N).

# WD change (relative to JCT3V-H1001_v2)

- I.8.5.3.3.9 Decoding process for depth based block partition wise inter sample prediction

…

When partIdx is equal to 1, the arrays PredSamplesDbbpL, PredSamplesDbbpCb and PredSamplesDbbpCr are modified as follows:

- The derivation process for contour boundary filtered samples as specified in subclause  is invoked with, the luma coding block size block nCbSL, the current coding block size nCbSX set equal to nCbSL, the array segMask, the array predSamples of prediction samples equal to PredSamplesDbbpL as inputs and the output is assigned to the array PredSamplesDbbpL of luma prediction samples.

- The derivation process for contour boundary filtered samples as specified in subclause  is invoked with, the luma coding block size block nCbSL, the current coding block size nCbSX set equal to nCbSC, the array segMask, the array predSamples of prediction samples equal to PredSamplesDbbpCb as inputs and the output is assigned to the array PredSamplesDbbpCb of luma prediction samples.

- The derivation process for contour boundary filtered samples as specified in subclause  is invoked with, the luma coding block size block nCbSL, the current coding block size nCbSX set equal to nCbSC, the array segMask, the array predSamples of prediction samples equal to PredSamplesDbbpCr as inputs and the output is assigned to the array PredSamplesDbbpCr of luma prediction samples.

- I.8.5.3.3.9.1 Derivation process for contour boundary filtered samples

Inputs to this process are:

- a variable nCbSL specifying the size of the current luma coding block,

- a variable nCbSX specifying the size of the current coding block,

- an (nCbSL)x(nCbSL) array segMask

- an (nCbSX)x(nCbSX) array predSamples prediction samples

Outputs to this process are:

- an modified (nCbS)x(nCbS) array predSamples of luma prediction samples

The (nCbSX)x(nCbSX) array p is set equal to predSamples and the variable n is set equal to ( nCbSL / nCbSX ).

The values of predSamples are derived as specified in the following:

Option1

for ( y = 0; y < nCbSX; y++ )
		for( x = 0; x < nCbSX; x++ ) {
			tFlag = segMask[ n * x ][ Max( 0, n * ( y − 1 ) ) ]
			lFlag = segMask[ Max( 0, (n * ( x − 1 ) ) ][ n * y ]
			bFlag = segMask[ n * x ][ Min( n * ( y + 1 ), nCbSL − 1 ) ]
			rFlag = segMask[ Min( n * ( x + 1 ), nCbSL − 1 ) ][ n*y ]
			cFlag = segMask[ n * x ][ n * y ]
			filt = p[ x ][ y ]

If PartMode is PART_Nx2N, the following apply,
				if( ( lFlag  | |  cFlag  | |  rFlag )  &&  ( !lFlag  | |  !cFlag  | |  !rFlag ) )
					filt = ( p[ Max( 0, x − 1 ) ][ y ] + ( filt  <<  1 ) + p[ Min( x + 1, nCbSX − 1 ) ][ y ] )  >>  2
			Otherwise (PartMode is PART_2NxN), the following apply,

if( ( tFlag  | |  cFlag  | |  bFlag )  &&  ( !tFlag  | |  !cFlag  | |  !bFlag ) )
					filt = ( p[ x ][ Max( 0, y − 1 ) ] + ( filt  <<  1 ) + p[ x ][ Min( y + 1, nCbSX − 1 ) ] )  >>  2
			predSamples[ x ][ y ] = filt
		}

Option2 (added in revision2)

for ( y = 0; y < nCbSX; y++ )
		for( x = 0; x < nCbSX; x++ ) {
			tFlag = segMask[ n * x ][ Max( 0, n * ( y − 1 ) ) ]
			lFlag = segMask[ Max( 0, (n * ( x − 1 ) ) ][ n * y ]
			bFlag = segMask[ n * x ][ Min( n * ( y + 1 ), nCbSL − 1 ) ]
			rFlag = segMask[ Min( n * ( x + 1 ), nCbSL − 1 ) ][ n*y ]
			cFlag = segMask[ n * x ][ n * y ]
			filt = p[ x ][ y ]

If PartMode is PART_Nx2N, the following apply,

if ( lFlag != rFlag)
				if( ( lFlag  | |  cFlag  | |  rFlag )  &&  ( !lFlag  | |  !cFlag  | |  !rFlag ) )
					filt = ( p[ Max( 0, x − 1 ) ][ y ] + ( filt  <<  1 ) + p[ Min( x + 1, nCbSX − 1 ) ][ y ] )  >>  2
			Otherwise (PartMode is PART_2NxN), the following apply,

if ( tFlag != bFlag)
				if( ( tFlag  | |  cFlag  | |  bFlag )  &&  ( !tFlag  | |  !cFlag  | |  !bFlag ) )
					filt = ( p[ x ][ Max( 0, y − 1 ) ] + ( filt  <<  1 ) + p[ x ][ Min( y + 1, nCbSX − 1 ) ] )  >>  2
			predSamples[ x ][ y ] = filt
		}

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.02% | 0.05% | 0.05% | 100.2% | 104.6% | 101.1% |
| Kendo | 0.0% | 0.1% | 0.1% | 0.01% | -0.04% | -0.03% | 100.2% | 100.4% | 102.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | -0.03% | 100.9% | 99.1% | 100.6% |
| GT_Fly | 0.0% | 0.1% | 0.0% | 0.01% | 0.00% | 0.00% | 100.7% | 99.7% | 100.5% |
| Poznan_Hall2 | 0.0% | 0.1% | -0.2% | -0.03% | -0.04% | -0.02% | 99.9% | 100.3% | 100.3% |
| Poznan_Street | 0.0% | 0.2% | 0.1% | 0.05% | 0.05% | 0.05% | 100.2% | 99.7% | 99.9% |
| Undo_Dancer | 0.0% | 0.1% | 0.0% | 0.02% | 0.02% | 0.03% | 100.2% | 101.3% | 100.1% |
| Shark | 0.0% | 0.0% | 0.1% | 0.01% | 0.01% | 0.03% | 99.9% | 100.5% | 100.2% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 0.01% | 0.01% | 0.00% | 100.4% | 101.4% | 101.3% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.01% | 0.01% | 0.02% | 100.2% | 100.3% | 100.2% |
| Average | 0.0% | 0.1% | 0.0% | 0.01% | 0.01% | 0.01% | 100.3% | 100.7% | 100.6% |

Combination of I0109 option1 and I0107 (DBBP/VSP partition decision unification)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | -0.04% | 100.2% | 104.4% | 100.8% |
| Kendo | 0.0% | 0.0% | -0.1% | -0.03% | -0.07% | -0.06% | 100.4% | 101.6% | 101.3% |
| Newspaper_CC | 0.0% | 0.1% | 0.0% | 0.00% | 0.02% | 0.01% | 100.6% | 100.7% | 101.3% |
| GT_Fly | 0.0% | 0.1% | 0.1% | 0.02% | 0.02% | -0.03% | 100.9% | 100.5% | 100.0% |
| Poznan_Hall2 | 0.0% | -0.2% | -0.1% | -0.04% | -0.03% | 0.00% | 99.8% | 100.1% | 99.9% |
| Poznan_Street | 0.0% | 0.1% | 0.1% | 0.02% | 0.02% | 0.03% | 100.4% | 100.5% | 100.1% |
| Undo_Dancer | 0.0% | 0.1% | 0.0% | 0.01% | 0.01% | 0.05% | 100.3% | 100.8% | 99.8% |
| Shark | 0.0% | 0.0% | 0.1% | 0.00% | 0.01% | 0.03% | 100.1% | 100.5% | 100.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | -0.03% | 100.4% | 102.2% | 101.1% |
| 1920x1088 | 0.0% | 0.0% | 0.1% | 0.00% | 0.01% | 0.02% | 100.3% | 100.5% | 100.1% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.3% | 101.1% | 100.5% |

Option2

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.03% | 0.05% | 0.04% | 99.7% | 100.3% | 97.2% |
| Kendo | 0.0% | 0.1% | 0.1% | 0.01% | -0.04% | -0.02% | 98.3% | 94.1% | 95.6% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | -0.03% | 100.3% | 98.1% | 99.9% |
| GT_Fly | 0.0% | 0.1% | -0.1% | 0.00% | 0.00% | -0.01% | 100.8% | 100.6% | 99.9% |
| Poznan_Hall2 | 0.0% | 0.1% | -0.2% | -0.02% | -0.04% | -0.02% | 100.0% | 100.0% | 100.2% |
| Poznan_Street | 0.0% | 0.2% | 0.1% | 0.04% | 0.05% | 0.05% | 100.2% | 99.9% | 99.9% |
| Undo_Dancer | 0.0% | 0.1% | 0.1% | 0.02% | 0.02% | 0.02% | 100.2% | 100.2% | 100.1% |
| Shark | 0.0% | 0.0% | 0.1% | 0.00% | 0.00% | 0.01% | 99.9% | 100.0% | 100.2% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 0.01% | 0.01% | 0.00% | 99.5% | 97.5% | 97.6% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.01% | 0.01% | 0.01% | 100.2% | 100.1% | 100.1% |
| Average | 0.0% | 0.1% | 0.0% | 0.01% | 0.01% | 0.00% | 99.9% | 99.2% | 99.1% |

# Complexity analysis

Table2 shows DBBP’s complexity comparison between the current design and proposed design. As shown, in the current design reportedly the worst number of addiction decreases from the current 127 % to 115 %.

|  |  |  |  | Number of operation [per Pic] |  | Number of operation [%] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HEVC Main profile |  | CU | PU | Mult | Add | Mult [%] | Add [%] |
|  | MC | 64x64 | 64x64 | 42.1 | 37.3 | 74% | 75% |
|  |  | 32x32 | 32x32 | 44.3 | 39.1 | 78% | 78% |
|  |  | 16x16 | 16x16 | 48.5 | 42.8 | 85% | 86% |
|  |  | 8x8 | 8x8 | 57.0 | 50.0 | 100% | 100% |
|  |  | 8x8 | 8x4 | 37.0 | 31.5 | 65% | 63% |
|  |  | 8x8 | 4x8 | 28.5 | 24.3 | 50% | 49% |
| 3D-HEVC |  | CU | PU | Mult | Add | Mult [%] | Add [%] |
|  | DBBP | 64x64 | 64x64 | 42.1 | 57.3 | 74% | 115% |
|  |  | 32x32 | 32x32 | 44.3 | 59.1 | 78% | 118% |
|  |  | 16x16 | 16x16 | 48.5 | 62.8 | 85% | 126% |
|  |  | 8x8 | 8x8 | 57.0 | 70.0 | 100% | 140% |
|  |  | CU | PU | Mult | Add | Mult [%] | Add [%] |
|  | DBBP I0109 option1 | 64x64 | 64x64 | 42.1 | 49.1 | 74% | 98% |
|  |  | 32x32 | 32x32 | 44.3 | 50.9 | 78% | 102% |
|  |  | 16x16 | 16x16 | 48.5 | 54.5 | 85% | 109% |
|  |  | 8x8 | 8x8 | 57.0 | 61.8 | 100% | 124% |
|  |  | CU | PU | Mult | Add | Mult [%] | Add [%] |
|  | DBBP I0109 option2 | 64x64 | 64x64 | 42.1 | 46.6 | 74% | 93% |
|  |  | 32x32 | 32x32 | 44.3 | 48.4 | 78% | 97% |
|  |  | 16x16 | 16x16 | 48.5 | 52.0 | 85% | 104% |
|  |  | 8x8 | 8x8 | 57.0 | 59.3 | 100% | 119% |
|  |  | CU | PU | Mult | Add | Mult [%] | Add [%] |
|  | DBBP I0088 | 64x64 | 64x64 | 42.1 | 49.3 | 74% | 99% |
|  |  | 32x32 | 32x32 | 44.3 | 51.1 | 78% | 102% |
|  |  | 16x16 | 16x16 | 48.5 | 54.8 | 85% | 110% |
|  |  | 8x8 | 8x8 | 57.0 | 62.0 | 100% | 124% |

- Conclusion

It is proposed to simplify DBBP filtering, which utilize one dimensional filtering decided on the basis of PartMode. Because coding efficiency impact is negligible with this simplification, it is recommended to adopt this method in next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Y. Lee, M. Mishurovskiy, M. W. Park, C. Kim, “Partition boundary filtering in DBBP”, JCT3V-H0104, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.

# Appendix

|  | Luma comparison | Luma add | Chroma comparison | Chroma add |
| --- | --- | --- | --- | --- |
| Current HTM | NxNx8 | NxNx7 | N/2xN/2x10 | N/2xN/2x10 |
| I0109 option1 | NxNx5+1 | NxNx4 | N/2xN/2x6+1 | N/2xN/2x5 |
| I0109 option2 | NxNx4+1 | NxNx3 | N/2xN/2x5+1 | N/2xN/2x4 |
| I0088 | NxNx6 | NxNx5 | N/2xN/2x4 | 0 |

Current HTM (luma): Add = NxNx8 comparison + NxNx7 addiction

Current HTM (chroma): Add = N/2xN/2x10 comparison + N/2xN/2x10 addiction

Contour part(luma only): NxNx1 comparison NxNx1 addiction

The values of the binary partition pattern contourPattern[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable threshVal specifying a threshold for the segmentation of refSamples is derived as specified in the following:

    - The variable sumRefVals is set equal to 0.

    - For x = 0..nTbS − 1 and y = 0..nTbS − 1, when ( x % sampInt ) and ( y % sampInt ) are both equal to 0, the following applies:

        - sumRefVals  +=  refSamples[ x ][ y ]		(Error: Reference source not found-1)

    - The variable threshVal is set equal to ( sumRefVals  >>  ( 2 * log2( nTbS / sampInt ) ) )

  - The variable contourPattern[ x ][ y ] with x, y =0..nTbS − 1 specifying a binary partition pattern is derived as specified in the following:

    - For x = 0..nTbS − 1 and y = 0..nTbS − 1, the following applies:

        - contourPattern[ x ][ y ] = ( refSamples[ x ][ y ] > threshVal )	(Error: Reference source not found-2)

DBBP blending part(luma): (NxN comparison)

DBBP blending part(chroma): (2*N/2xN/2 comparison)

for ( y = 0; y < nCbSL; y++ )
		for( x = 0; x < nCbSL; x++ ) {
			if( segMask[ x ][ y ]  = =  ( partIdx  ! =  segMask[ 0 ][ 0 ] ) )
				PredSamplesDbbpL[ x ][ y ] = predSamplesL[ x ][ y ]
				if( ( x % 2  = =  0 )  &&  ( y % 2  = =  0 ) ) {
					PredSamplesDbbpCb[ x / 2 ][ y / 2 ] = predSamplesCb[ x / 2 ][ y / 2 ]
					PredSamplesDbbpCr[ x / 2 ][ y / 2 ] = predSamplesCr[ x / 2 ][ y / 2 ]
				}
		}

DBBP Filtering part (luma): NxNx4+NxNx2 comparison NxNx2+NxNx4 addiction

DBBP Filtering part (chroma): N/2xN/2x4+2*N/2xN/2x2 comparison N/2xN/2x2+2*N/2xN/2x4 addiction

for ( y = 0; y < nCbSX; y++ )
		for( x = 0; x < nCbSX; x++ ) {
			tFlag = segMask[ n * x ][ Max( 0, n * ( y − 1 ) ) ]
			lFlag = segMask[ Max( 0, (n * ( x − 1 ) ) ][ n * y ]
			bFlag = segMask[ n * x ][ Min( n * ( y + 1 ), nCbSL − 1 ) ]
			rFlag = segMask[ Min( n * ( x + 1 ), nCbSL − 1 ) ][ n*y ]
			cFlag = segMask[ n * x ][ n * y ]
			filt = p[ x ][ y ]
			if( ( lFlag  | |  cFlag  | |  rFlag )  &&  ( !lFlag  | |  !cFlag  | |  !rFlag ) )
			if( ( lFlag  -  cFlag )  +  ( cFlag  -  rFlag ) )
				filt = ( p[ Max( 0, x − 1 ) ][ y ] + ( filt  <<  1 ) + p[ Min( x + 1, nCbSX − 1 ) ][ y ] )  >>  2

if( ( tFlag  | |  cFlag  | |  bFlag )  &&  ( !tFlag  | |  !cFlag  | |  !bFlag ) )
			if( ( tFlag  -  cFlag ) + ( cFlag  -  bFlag ) )

filt = ( p[ x ][ Max( 0, y − 1 ) ] + ( filt  <<  1 ) + p[ x ][ Min( y + 1, nCbSX − 1 ) ] )  >>  2			predSamples[ x ][ y ] = filt
		}

Proposal(Option1)

Proposal option1 (luma): Add = NxNx5+1 comparison + NxNx4 addiction

Proposal option1 (chroma): Add = N/2xN/2x6+1 comparison + N/2xN/2x5 addiction

Contour part(luma only): NxNx1 comparison NxNx1 addiction

DBBP blending part(luma): (NxN comparison)

DBBP blending part(chroma): (2*N/2xN/2 comparison)

DBBP Filtering part (luma): NxNx2+NxNx1+1 comparison NxNx1+NxNx2 addiction

DBBP Filtering part (chroma): N/2xN/2x2+1+2*N/2xN/2 comparison N/2xN/2x1+2*N/2xN/2x2 addiction

for ( y = 0; y < nCbSX; y++ )
		for( x = 0; x < nCbSX; x++ ) {
			tFlag = segMask[ n * x ][ Max( 0, n * ( y − 1 ) ) ]
			lFlag = segMask[ Max( 0, (n * ( x − 1 ) ) ][ n * y ]
			bFlag = segMask[ n * x ][ Min( n * ( y + 1 ), nCbSL − 1 ) ]
			rFlag = segMask[ Min( n * ( x + 1 ), nCbSL − 1 ) ][ n*y ]
			cFlag = segMask[ n * x ][ n * y ]
			filt = p[ x ][ y ]
			if( ( lFlag  | |  cFlag  | |  rFlag )  &&  ( !lFlag  | |  !cFlag  | |  !rFlag ) )
			If PartMode is PART_Nx2N, the following apply,

if( ( lFlag  -  cFlag )  +  ( cFlag  -  rFlag ) )
				filt = ( p[ Max( 0, x − 1 ) ][ y ] + ( filt  <<  1 ) + p[ Min( x + 1, nCbSX − 1 ) ][ y ] )  >>  2

Otherwise (PartMode is PART_2NxN), the following apply,

if( ( tFlag  | |  cFlag  | |  bFlag )  &&  ( !tFlag  | |  !cFlag  | |  !bFlag ) )
			if( ( tFlag  -  cFlag ) + ( cFlag  -  bFlag ) )

filt = ( p[ x ][ Max( 0, y − 1 ) ] + ( filt  <<  1 ) + p[ x ][ Min( y + 1, nCbSX − 1 ) ] )  >>  2			predSamples[ x ][ y ] = filt
		}

Proposal(Option2)

Proposal option1 (luma): Add = NxNx4+1 comparison + NxNx3 addiction

Proposal option1 (chroma): Add = N/2xN/2x5+1 comparison + N/2xN/2x4 addiction

Contour part(luma only): NxNx1 comparison NxNx1 addiction

DBBP blending part(luma): (NxNx1 comparison)

DBBP blending part(chroma): (2*N/2xN/2 comparison)

DBBP Filtering part (luma): NxN+NxNx1+1 comparison NxNx2 addiction

DBBP Filtering part (chroma): N/2xN/2+1+2*N/2xN/2 comparison 2*N/2xN/2x2 addiction

for ( y = 0; y < nCbSX; y++ )
		for( x = 0; x < nCbSX; x++ ) {
			tFlag = segMask[ n * x ][ Max( 0, n * ( y − 1 ) ) ]
			lFlag = segMask[ Max( 0, (n * ( x − 1 ) ) ][ n * y ]
			bFlag = segMask[ n * x ][ Min( n * ( y + 1 ), nCbSL − 1 ) ]
			rFlag = segMask[ Min( n * ( x + 1 ), nCbSL − 1 ) ][ n*y ]
			cFlag = segMask[ n * x ][ n * y ]
			filt = p[ x ][ y ]
			if( ( lFlag  | |  cFlag  | |  rFlag )  &&  ( !lFlag  | |  !cFlag  | |  !rFlag ) )
			If PartMode is PART_Nx2N, the following apply,

if( ( lFlag  -  rFlag ))
				filt = ( p[ Max( 0, x − 1 ) ][ y ] + ( filt  <<  1 ) + p[ Min( x + 1, nCbSX − 1 ) ][ y ] )  >>  2

Otherwise (PartMode is PART_2NxN), the following apply,

if( ( tFlag  | |  cFlag  | |  bFlag )  &&  ( !tFlag  | |  !cFlag  | |  !bFlag ) )
			if( ( tFlag  -  bFlag ) )

filt = ( p[ x ][ Max( 0, y − 1 ) ] + ( filt  <<  1 ) + p[ x ][ Min( y + 1, nCbSX − 1 ) ] )  >>  2			predSamples[ x ][ y ] = filt
		}

I0088

Proposal option1 (luma): Add = NxNx6 comparison + NxNx5 addiction

Proposal option1 (chroma): Add = N/2xN/2x4 comparison

Contour part(luma only): NxNx1 comparison NxNx1 addiction

DBBP blending part(luma): (NxNx1 comparison)

DBBP blending part(chroma): (2*N/2xN/2 comparison)

DBBP Filtering part (luma): NxNx2+NxNx2 comparison NxNx4 addiction

for ( y = 0; y < nCbSX; y++ )
		for( x = 0; x < nCbSX; x++ ) {
			tFlag = segMask[ n * x ][ Max( 0, n * ( y − 1 ) ) ]
			lFlag = segMask[ Max( 0, (n * ( x − 1 ) ) ][ n * y ]
			bFlag = segMask[ n * x ][ Min( n * ( y + 1 ), nCbSL − 1 ) ]
			rFlag = segMask[ Min( n * ( x + 1 ), nCbSL − 1 ) ][ n*y ]
			cFlag = segMask[ n * x ][ n * y ]
			filt = p[ x ][ y ]
			if( ( lFlag  | |  cFlag  | |  rFlag )  &&  ( !lFlag  | |  !cFlag  | |  !rFlag ) )

if( ( lFlag  -  rFlag ))
				filt = ( p[ Max( 0, x − 1 ) ][ y ] + ( filt  <<  1 ) + p[ Min( x + 1, nCbSX − 1 ) ][ y ] )  >>  2

if( ( tFlag  | |  cFlag  | |  bFlag )  &&  ( !tFlag  | |  !cFlag  | |  !bFlag ) )
			if( ( tFlag  -  bFlag ) )

filt = ( p[ x ][ Max( 0, y − 1 ) ] + ( filt  <<  1 ) + p[ x ][ Min( y + 1, nCbSX − 1 ) ] )  >>  2			predSamples[ x ][ y ] = filt
		}