| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0114 |
| --- | --- |

| Title: | Improvement on merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Junghak Nam Sehoon Yea 19 Yangjae-daero 11gil Seocho-gu, Seoul 137-893 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, an IC flag can be parsed for merge blocks according to the merge index. In this method in D0060[1], the IC flag is only parsed when the merge index is larger than zero. On the other side, MLC(merge list construction) for dependent views is performed according to the IC flag in H0070[2]. The MLC process can skip IvMC candidate if the IC flag is equal to 1. Coincidence of these two methods can make the design undesirable, not being chosen the first spatial candidate if the IC flag is set to 1. Therefore, it is proposed to solve this problem with two solutions. The first solution is to make D0060 disabled, and it is reportedly shown that the coding loss is 0.16% from synthesis view. The second solution is to remove H0070. It is found out that it losses 0.03% of coding gain from synthesis view.

# Problem Statement

In the current 3D-HEVC, the IC flag and the merge mode can affect from each other. Firstly, in the IC flag parsing dependency prevention method, the IC flag parsing process for merge blocks has been adopted to remove parsing dependency. When the current block is coded with merge mode, the IC flag is parsed when its index is larger than zero. In the other related method in the adaptive disabling IvMV, IvMV candidate can be skipped for the merge block when the IC flag is set to one. The coincidence of these two methods causes an undesirable situation when the current block is coded with merge mode, the merge index is larger than 0, and the IC flag is set to one. In this condition, the first merge candidate cannot be IvMV because of the adaptive disabling IvMV method, which means the first candidate is a spatial candidate. However, because of the the IC flag parsing dependency prevention method, the first candidate cannot be chosen although it might be the best candidate, described as Figure 1.

Figure 1  An undesirable condition in the merge mode when IC flag is set to 1

# Proposed method

Since the coincidence of the two methods makes the first spatial merge candidate untouchable, disabling either D0060 or H0070 can solve the problem. One proposed solution is to disable the IC flag parsing dependency prevention method, D0060. By making D0060 disabled, the IC flag for merge blocks are always parsed. Even though H0070 makes the spatial candidate to be positioned first, it can be chosen as the best candidate, depicted in Error: Reference source not found. The other solution is to make the adaptive disabling IvMV method, H0070, removed described as Error: Reference source not found. In the second solution the IC flag is only parsed when merge index is greater than zero. However, there is a chance that the first spatial candidate can be chosen as the best candidate since a temporal inter-view candidate (IvMC) is not skipped.

Figure 2  Solution 1 : Available merge candidates in the given condition if D0060 is off

Figure 3 Solution 2 : Available merge candidates in the given condition if H0070 is off

# Experimental result

From the following experimental results, it is found out that the first solution has coding loss of 0.15% from synthesis view, while removing H0070 brings 0.03% of coding loss from synthesis view. It seems because when the D0060 is disabled, the IC flags are always sent for all merge blocks, which may introduce bitrate overhead.

## The experimental result when D0060 is disabled

## The experimental result when H0070 is disabled

# Conclusion

In the current 3D-HEVC, the first spatial merge candidate in dependent views is considered as always disabled when the IC flag is set to 1 because of the coincidence of D0060 and H0070. By simply removing one of them, the problem discussed in this proposal can be solved. For this reason, it is recommended to remove either of them so that 3D-HEVC is properly performed.

# Reference

- T. Ikai, “3D-CE5.h related: Removal of parsing dependency in illumination compensation,”  Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Incheon, KR, Apr .2013

- M. W. Park, J. Y. Lee, B. Choi, Y. Cho, C. Kim, “3D-CE1: Results on Adaptive Disabling Inter-view Motion Vector Candidates,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-H0070, Valencia, ES, Mar. 2014.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).