| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0114 |
| --- | --- |

| Title: | Bug-fix on merge mode for IC |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Junghak Nam Sehoon Yea 19 Yangjae-daero 11gil Seocho-gu, Seoul 137-893 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- The newly added parts compared to 3D-HEVC working draft 3 are highlighted in green and the removed parts are marked with strikethrough.

# Solution 1

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| .... |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if( nuh_layer_id > 0 && !DepthFlag && !MvHevcCompatibilityFlag ) { |  |
| slice_ic_enable_flag | u(1) |
| if( slice_ic_enable_flag ) |  |
| slice_ic_disable_merge_zero_idx_flag | u(1) |
| } |  |
| ... |  |
| } |  |

- I.7.4.7.1        General slice segment header semantics

...

slice_ic_disable_merge_zero_idx_flag equal to 1 specifies that ic_flag is not present in the coding units with partitioning mode equal to PART_2Nx2N of the current slice when merge_flag is equal to 1 and merge_idx of the prediction unit in the coding unit is equal to 0. slice_ic_disable_merge_zero_idx_flag equal to 0 specifies that ic_flag might be present in the coding units with partitioning mode equal to PART_2Nx2N of the current slice when merge_flag is equal to 1 and merge_idx of the prediction unit in the coding unit is equal to 0. When not present, slice_ic_disable_merge_zero_idx_flag is inferred to be equal to 0.

...

- I.7.4.9.5.2        Coding unit extension semantics

...

- The variable icEnableFlag is set equal to 0 and when slice_ic_enable_flag is equal to 1 and PartMode is equal to 2Nx2N and CuPredMode[ x0 ][ y0 ] is not equal to MODE_INTRA, the following applies:

- If merge_flag[ x0 ][ y0 ] is equal to 1, the following applies:

    - icEnableFlag = ( merge_idx[ x0 ][ y0 ] != 0 )  | |  !slice_ic_disable_merge_zero_idx_flag 	(Error: Reference source not found-1)

- Otherwise (merge_flag[ x0 ][ y0 ] is equal to 0), the following applies:

  - With X being replaced by 0 and 1, the variable refViewIdxLX is set equal to ViewIdx( RefPicListLX[ ref_idx_lX[ x0 ][ y0 ] ] ).

  - The flag icEnableFlag is derived as specified in the following:

      - icEnableFlag = 	
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L0  &&  refViewIdxL1  !=  ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y0 ]  !=  Pred_L1  &&  refViewIdxL0  !=  ViewIdx)	(Error: Reference source not found-2)

- ...

# Solution 2

- I.8.5.3.2.10        Derivation process for inter-view merge candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0 and mpi_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagIvMC, availableFlagIvMCShift and availableFlagIvDC specifying whether the inter-view merge candidates are available,

- the reference indices refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC,

- the prediction list utilization flags predFlagLXIvMC, predFlagLXIvMCShift and predFlagLXIvDC,

- the motion vectors mvLXIvMC, mvLXIvMCShift and mvLXIvDC.

The availability flags availableFlagIvMC, availableFlagIvMCShift and availableFlagIvDC are set equal to 0, and for X in the range of 0 to 1, inclusive, the variables predFlagLXIvMC, predFlagLXIvMCShift, predFlagLXIvDC are set equal to 0, the variables refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC are set equal to −1,  and both components of mvLXIvMC, mvLXIvMCShift and mvLXIvDC are set equal to 0.

When ic_flag is equal to 0, tThe temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - Depending on PartMode, the following applies:

      - [ Ed. (GT): Subsequent to the Valencia meeting, proponents of H0205 provided a revised specification text changing below condition "PartMode is equal to PART_2Nx2N" to "dbbp_flag[ xPb ][ yPb ] is equal to 0". However, although this was discussed at the meeting, it is not clear from the meeting notes, whether this was finally adopted. Further clarification is required. Current software uses the changed condition. ]

    - If PartMode is equal to PART_2Nx2N, the derivation process for inter layer predicted sub prediction block motion vector candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ] and the disparity vector MvRefinedDisp[ xPb ][ yPb ] as inputs, and the outputs are, with X being in the range of 0 to 1, inclusive, the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

    - Otherwise (PartMode is not equal to PART_2Nx2N), the derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector MvRefinedDisp[ xPb ][ yPb ] as inputs, and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

      - availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(Error: Reference source not found-3)

      - predFlagL0IvMC = availableFlagL0IvMC		(Error: Reference source not found-4)

      - predFlagL1IvMC = availableFlagL1IvMC		(Error: Reference source not found-5)

When DepthFlag is equal to 0 and ic_flag is equal to 0, the shifted temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies:

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector MvRefinedDisp[ xPb ][ yPb ] + ( nPbW *2 + 4, nPbH *2 + 4 ) as inputs, and the outputs are the flag availableFlagLXIvMCShift, the motion vector mvLXIvMCShift and the reference index refIdxLXIvMCShift.

  - The availability flag availableFlagIvMCShift, and the prediction utilization flags predFlagL0IvMCShift and predFlagL1IvMCShift are derived by

      - availableFlagIvMCShift = availableFlagL0IvMCShift | | availableFlagL1IvMCShift	(Error: Reference source not found-6)

      - predFlagL0IvMCShift = availableFlagL0IvMCShift	(Error: Reference source not found-7)

      - predFlagL1IvMCShift = availableFlagL1IvMCShift	(Error: Reference source not found-8)

When DepthFlag is equal to 0, the disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies:

    - The derivation process for a disparity inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ], the disparity vector MvRefinedDisp[ xPb ][ yPb ], and the prediction list indication X, as inputs, and the outputs are the flag availableFlagLXIvDC, the motion vector mvLXIvDC, and the reference index refIdxLXIvDC.

  - The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

      - availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC 	(Error: Reference source not found-9)

      - predFlagL0IvDC = availableFlagL0IvDC		(Error: Reference source not found-10)

      - predFlagL1IvDC = availableFlagL1IvDC		(Error: Reference source not found-11)