| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0115 |
| --- | --- |

| Title: | AHG5 : Performance evaluation for 3D-HEVC and its coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Sehoon Yea 19 Yangjae-daero 11gil Seocho-gu, Seoul South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this document, the overall coding performance of the current 3D-HEVC is shown. Firstly, 3D-HEVC is compared with simulcast HEVC (HEVC version 1) and MV-HEVC. After that, the coding performances of each coding tools integrated in 3D-HEVC are assessed. For these evaluations, HTM-10.0r1 is used. In order to test the simulcast HEVC and MV-HEVC, all 3D-HEVC coding tools are disabled. For the coding tools evaluation, each coding tool is disabled and compared with the anchor. As a result, 3D-HEVC achieved 54.84% of coding performance from synthesis view, while the coding performance of MV-HEVC is 45.44% from synthesis view, compared to the simulcast HEVC.

# Introduction

Since the 98th MPEG, in November 2011, 3D-HEVC has developed based on the best proposal from HHI[1]. Until now, many coding tools have been evaluated and adopted to 3D-HEVC. In company with the standardization process of 3D-HEVC, MV-HEVC standardization is also finished. The key difference between MV-HEVC and 3D-HEVC is that 3D-HEVC provides under CTU level coding tools, which exploits motion parameters from the reference view, as well as special coding tools for depth coding. In this contribution, the comparison of coding performance between MV-HEVC and 3D-HEVC is provided. Also, the coding tool evaluation of 3D-HEVC is reported.

# Comparison of 3D-HEVC with HEVC and MV-HEVC

In order to confirm the coding performance of 3D-HEVC, it is compared with HEVC. For easier experiment, HTM-10.0r1 is used for simulcast HEVC, by modifying the configuration of coding structure - all coding tools are disabled and the inter-view prediction is prevented for all views. In the same way, MV-HEVC can be tested by disabling all 3D-HEVC coding tools, according to JCT3V-F0122[2]. The complexity estimates for the encoding, decoding and rendering are calculated on the Linux cluster, which may not be accurate because of parallel simulations.

The experimental result of 3D-HEVC performance against the simulcast HEVC is shown in Table 1. It is shown that the coding performance of dependent views is approximately 75% in average and it is achieved 54.8% of savings from the synthesis view. For the complexity, it is shown that the encoder time decreases. This is because fast encoding algorithms in HTM are turned off for simulcast case.

Table 1 Simulcast HEVC vs. 3D-HEVC (HTM-10.0r1)

The coding performance of MV-HEVC against the simulcast HEVC is shown in Table 2. The overall coding performances from the dependent views and synthesis view are approximately 10% lower than the ones of 3D-HEVC.

Table 2 Simulcast HEVC vs. MV-HEVC (HTM-10.0r1)

The coding performance of 3D-HEVC against MV-HEVC is shown in Table 3. 3D-HEVC has earned 17.1% of coding performance in the synthesis view by adopting coding tools under CTU level.

Table 3 MV-HEVC vs. 3D-HEVC (HTM-10.0r1)

# 3D-HEVC Tool evaluations

## Texture coding tools

Inter-view motion vector prediction algorithm turned out to be the best texture coding algorithm in 3D-HEVC, shown in Table 4. Please note that IvMV coding performance can increase with DoNBDV.

Table 4  IvMC for texture disabled vs. 3D-HEVC (HTM-10.0r1)

The coding performance of ARP, advanced residual prediction, turned out as the second best texture coding tool in 3D-HEVC. 3D-HEVC decreases 1.07% of coding performance from the synthesis view when ARP is disabled, while it consumes computing resources at the decoder.

Table 5  ARP disabled vs. 3D-HEVC (HTM-10.0r1)

Depth oriented neighbor block disparity vector, DoNBDV, has assisted 0.47% of coding gains to 3D-HEVC. When the DoNBDV is tunred off, it is replaced to NBDV for IvMV, IvDC and DBBP, the tools which exploits the benefit of DoNBDV.

Table 6  DoNBDV disabled vs. 3D-HEVC (HTM-10.0r1)

When IC, Illumination compensation algorithm in 3D-HEVC is turned off, 0.43% of coding gains coding loss is observed, like shown in Table 7. Since computer generated sequences, GT_Fly, Undo_Dancer and Shark, have less illumination mismatch than other natural sequences, the coding gain from IC seems less.

Table 7  IC disabled vs. 3D-HEVC (HTM-10.0r1)

It is shown that 3D-HEVC achieved 0.32% of coding performance from view synthesis prediction. It is supposed that the sequences with higher quality of depths can have more benefits from VSP.

Table 8  VSP disabled vs. 3D-HEVC (HTM-10.0r1)

From depth based block partitioning, 3D-HEVC earns 0.08% of coding gains from synthesis view. Like VSP, the coding performance in the dependent view varies depending on the sequences which have better quality of depths.

Table 9  DBBP disabled vs. 3D-HEVC (HTM-10.0r1)

## Depth coding tools

Depth coding tools can be classified as intra tools and inter coding tools. For accurate coding performance assessment, intra tools are evaluated under all intra configuration as well.

Motion parameter inheritance algorithm turned out to be the best coding tool for depth in 3D-HEVC. According to JCT3V-G0119[3], it is achieved 0.23% of additional coding gains from Sub-PU.

Table 10  MPI disabled vs. 3D-HEVC (HTM-10.0r1)

The intra coding tool for depth coding, 3D-HEVC gains 1.58% of coding performance by using DMM1 under CTC. It seems DMM1 needs more bitrates, but it helps to enhance the synthesized video BD-performance.

Table 11  DMM1 disabled vs. 3D-HEVC (HTM-10.0r1) under CTC

Also, 3D-HEVC gains 3.62% of coding performance by using DMM1, with the need of a lot of computing resources for encoding, under AI configuration.

Table 12  DMM1 disabled vs. 3D-HEVC (HTM-10.0r1) under AI

For all intra configuration, only intra SDC can be turned on. It is achieved 1.23% of coding loss if SDC is turned off, like shown in Table 13.

Table 13  SDC disabled vs. 3D-HEVC (HTM-10.0r1) under AI

3D-HEVC can get 1.20% of additional coding performance with segment-wise depth coding for inter prediction block scheme. Like DMM1, it is necessary to use more bitrate for Inter SDC, but it achieves good synthesize BD performance.

Table 14  Inter SDC disabled vs. 3D-HEVC (HTM-10.0r1)

By adopting depth lookup table, 3D-HEVC gets 0.15% of coding gains in the synthesis view, shown in Table 15.

Table 15  DLT disabled vs. 3D-HEVC (HTM-10.0r1) under CTC

It is found out that DLT under AI has brought 0.48% of coding gains to 3D-HEVC, while it has earned 0.15% of coding gains under CTC.

Table 16  DLT disabled vs. 3D-HEVC (HTM-10.0r1) under AI

It seems that from disparity derived depth algorithm, 3D-HEVC earns 0.12% of coding gains in the synthesis view.

Table 17  DDD disabled vs. 3D-HEVC (HTM-10.0r1)

For depth inter coding, IvMC is also supported with the disparity converted the value of 128. By using IvMC for depth, 3D-HEVC can achieve 0.09% of the additional coding performance.

Table 18  IvMC disabled vs. 3D-HEVC (HTM-10.0r1)

Using depth modeling mode 4, 3D-HEVC decreases negligible coding performance under the CTC.

Table 19  DMM4 disabled vs. 3D-HEVC (HTM-10.0r1) under CTC

It is shown that DMM4, which models the depth partition in a block based on a collocated texture block, brings 0.17% of coding efficiency to 3D-HEVC, while it does not achieve any coding gain under CTC.

Table 20  DMM4 disabled vs. 3D-HEVC (HTM-10.0r1) under AI

## Coding tools applied both texture and depth

From Sub-PU algorithm, 3D-HEVC can achieve totally 1.44% of coding gain. Since JCT3V-G0119 reported the coding gain of Sub-PU MPI as 0.23%, approximately 1.21% of coding gain is earned from the texture.

Table 21    Sub-PU disabled vs. 3D-HEVC (HTM-10.0r1)

3D-HEVC can achieve 5.91% of coding gains compared that the IvMV is disabled for both for texture and depth. In Table 4, it is already shown that IvMV for texture has achieved 5.63% of coding gains. Therefore, most of the coding gains from IvMV is not from the depth coding, but from the texture coding.

Table 22    IvMV disabled vs. 3D-HEVC (HTM-10.0r1)

# Conclusion

In this document, the current coding performance of 3D-HEVC has been evaluated. When it is compared to the simulcast HEVC, it achieves approximately 54% of coding gains, while MV-HEVC gets about 45% of coding gains against the simulcast HEVC. Moreover, coding tools adopted in 3D-HEVC are assessed. For the texture coding, IvMC is the best algorithm to compress, with 5.63% of BD performance from synthesis view. For the depth coding, MPI turned out to be the best method with 2.39% of BD performance from synthesis view, under CTC.

# Reference

- H. Schwarz, C. Bartnik, S. Bosse, H. Brust, T. Hinz, H. Lakshman, D. Marpe, P. Merkle, K. Müller, H. Rhee, G. Tech, M. Winken, T. Wiegand, “Description of 3D Video Coding Technology Proposal by Fraunhofer HHI (HEVC compatible, configuration A),” ISO/IEC JTC1/SC29/WG11/M22570, Geneva, CH, Nov. 2011.

- Y. Chen, L. Zhang, X. Zhao, M. Karczewicz, “HTM support of depth coding in MV-HEVC,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-F0122, Geneva, CH, Oct. 2013.

- Y. Chen, H. Liu, L. Zhang, “CE2:Sub-PU based MPI,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-G0119, San Jose, US, Jan. 2014.