| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0116 |
| --- | --- |

| Title: | AHG5: Coding results for flexible coding order (FCO) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam and Sehoon Yea | Email: | junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, flexible coding order (FCO) scheme is only available in a software which has several bugs and makes compile errors when the macro for FCO is activated. The draft text is implicitly designed for a texture-first coding order. However, a depth-first coding might be potentially increased to a coding gain by exploiting collocated depth information like the texture-first coding is doing using texture information in the same view. In this proposal, it is proposed to support FCO functionality in 3D- HEVC. The experimental results show a little coding gain on video only and 1% coding loss on synthesized video under the common test conditions.

# Introduction

In the current 3D-HTM software, texture picture is firstly coded before depth picture in the same view. So, coding gain of the depth picture might be improved by inter-component prediction utilizing reconstructed samples, motion/disparity information, and block partitioning from reconstructed texture. In addition, 3D-HTM supports flexible coding order (FCO) to study coding performance of non-CTC configurations. In similar with texture-first coding, coding performance of depth-first coding could be potentially improved by exploiting reconstructed depth information for the texture pictures of dependant views except texture picture of base view.

To support FCO, inter-component tools for the depth pictures cannot be used obviously. It is list of depth inter-component tools;

- Motion parameter inheritance (MPI)

- Depth derived disparity (DDD)

- Depth modeling mode 4 (DMM4)

- Quad-tree limitation (QTL/PC)

MPI, DMM4, and QTL/PC tools are used for both base and dependant depth pictures while DDD tool is only conducted on dependant depth picture. Even though coded by FCO, depth picture of base view is coded after texture picture to compatible with HEVC Version 1. Therefore, inter-component tools can be still used for the base depth picture except DDD tool, which turns off depth pictures for the all view.

In addition, texture coding tools using a virtual depth which is derived from the reconstructed depth in a reference view might be changed. It is list of texture coding tools referencing the virtual depth;

- Depth oriented NBDV (DoNBDV)

- View synthesized prediction (VSP)

- Depth based block partitioning (DBBP)

Depth-first coding by FCO can be used the collocated depth of the current view instead of using the depth of a reference view. As mentioned, coding gains for these texture tools could be improved because of referencing the more accurate depth values.

Unfortunately, the current HTM software makes compile errors when the macro for FCO is activated.

# Proposed method

This contribution offers FCO functionality solutions in 3D-HEVC .

## MPI

Even though FCO is used, MPI tool could be used for depth picture in the base view. Modifications for MPI to support FCO are as follows:

1) For dependant depth coding, mpi_flag[layerId] in VPS sets to 0 specifies that motion parameter inheritance is not used.

2) Checking process to determine whether the corresponding texture for MPI tool is available or not

## DDD

In the current 3D-HEVC, DDD and MPI tools are controlled by a mpi_flag[layerId] at the same time. However, DDD tool is not used for all depth pictures when FCO coding is applied while MPI tool is still used for the base depth picture. Modifications for DDD to support FCO are as follows:

1) Add ddd_flag[layerId] syntax element in VPS. ddd_flag[layerId] equal to 0 specifies disparity derived depth coding is not used for the layer with nuh_layer_id equal to layerId.

2) Checking process to determine whether the corresponding texture for DDD tool is available or not.

## DMM4

DMM4 is not used for all depth coding when FCO coding is applied. The HTM already includes checking process for an availability of the corresponding texture. However, when macro for FCO is activated, compile error occurs because of a legacy-code of the past.

1) Fix a compile error

## QTL/PC

Even though FCO is used, QTL/PC tool could be used for depth coding in the base view. Modifications for QTL/PC to support FCO are as follows:

1) For dependant depth coding, QTL/PC flag in SPS sets to 0 specifies that quad-tree limitation is not used. Note: QTL/PC syntax elements in SPS will be moved to VPS.

2) Checking process to determine whether the corresponding texture for QTL/PC is available or not.

## DoNBDV and VSP

When FCO coding is applied, DoNBDV and VSP processes use the collocated depth of the current view instead of using the virtual depth of a reference view. The scheme was adopted by JCT3V-E0163. There is no change for FCO coding in the software.

1) Checking process to determine whether collocated depth is used or base depth is used.

## DBBP

When FCO coding is applied, DBBP process can use the collocated depth of the current view instead of using the virtual depth of a reference view as DoNBDV and VSP. Certainly, related source code is already implemented in 3D-HTM. However, it seems to be having a bug in the case of FCO. In the software, caller function of getVirtualDepthBlock function always uses constant 0 value as the first argument. In the case of FCO, constant 0 value for the first argument means top-left position of LCU.

1) Bug fix to get virtual depth block from a co-located block position in a LCU instead of a top-left position of a LCU.

2) Checking process to determine whether collocated depth is used or base depth is used.

## Camera parameter

In the current 3D-HTM, camera parameters are transmitted in texture picture, and an associated depth picture inherits camera parameters from the texture picture. However, VSP and DoNBDV that require camera parameters cannot work properly for the depth picture when depth-first coding order is used. Draft text can support to explicitly signal camera parameter in depth picture.

1) Camera parameters are transmitted in depth picture, and a texture picture inherits camera parameters from the depth picture of the same view.

# Experimental results

The proposed method is integrated on 3D-HTM 11.0. To evaluate the proposed FCO, five tests are conducted. Table 1 shows the test conditions. The inter-component tools for depth picture are commonly used for all tests. MPI, DMM4, QTL/PC are only applied to a depth picture of base view while DDD turns off all tests.

Table 1. Test conditions

|  | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 |
| --- | --- | --- | --- | --- | --- |
| MPI, DMM4, QTL/PC | Yes (Base depth) | Yes (Base depth) | Yes (Base depth) | Yes (Base depth) | Yes (Base depth) |
| DDD | No | No | No | No | No |
| DoNBDV | Yes (Base depth) | Yes (collocated depth) | Yes (Base depth) | Yes (Base depth) | Yes (collocated depth) |
| VSP | Yes (Base depth) | Yes (Base depth) | Yes (collocated depth) | Yes (Base depth) | Yes (collocated depth) |
| DBBP | Yes (Base depth) | Yes (Base depth) | Yes (Base depth) | Yes (collocated depth) | Yes (collocated depth) |

Test 1 is conducted to evaluate coding effects by restricting the inter-component tools. There is no change for texture coding tools like DoNBDV, VSP and DBBP. Test 2 incurs approximately 0.06% coding gains for dependant views while synthesized PSNR shows 1.32% coding loss, as shown in Table 3.

Table 2. Test 1 vs. HTM 11.0

Test 2 to Test 4 are experimented to evaluate coding performance of each texture coding tool by using the collocated depth of the current view instead of using the virtual depth of a reference view. Test 2, Test 3, and Test 4 evaluate DoNBDV, VSP, and DBBP, respectively.

Test 2 incurs approximately 0.31% coding gains for dependant views while synthesized PSNR shows 1.13% coding loss, as shown in Table 3.

Table 3. Test 2 vs. HTM 11.0

Test 3 incurs approximately 0.31% coding gains for dependant views while synthesized PSNR shows 1.06% coding loss, as shown in Table 4.

Table 4. Test 3 vs. HTM 11.0

Test 4 incurs approximately 0.15% coding gains for dependant views while synthesized PSNR shows 1.23% coding loss, as shown in Table 5.

Table 5. Test 4 vs. HTM 11.0

As shown in Table 3 to Table 5, coding performance of texture coding tools are improved by FCO consistently. Especially, the tools show a big gain at the sequences having accurate depth data.

Test 5 evaluate overall coding performance of FCO. As shown in Table 6, Test 5 incurs approximately 0.34% coding gains for dependant views while synthesized PSNR shows 1% coding loss.

Table 6. Test 5 vs. HTM 11.0

# Conclusion

This contribution proposes to support FCO functionality in the current 3D-HTM. It is recommended to include the proposed method to the next 3D-HTM.

# Patent rights declaration(s)

LG Electronics Co., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).