| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0117 |
| --- | --- |

| Title: | CE2 related: Reduction of the number of intra prediction modes for depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, JungHak Nam, Sehoon Yea, | Email: | jin78.heo@lge.com, junghak.nam@lge.com sehoon.yea@lge.com, |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents an intra prediction mode for depth map coding. The proposed method reduces the number of intra prediction modes for depth map because the depth map has different characteristics compared to the texture video: 1) little variation within a block 2) simple or smooth region. It means that 35 intra prediction modes used in the conventional intra coding are unnecessarily expensive for depth intra coding. Therefore, this proposal reduced the number of conventional intra prediction modes for depth map coding. The proposed method has no coding loss under the common test condition (CTC) and all-intra configuration (AI) with negligible encoding/decoding complexity.

# Proposed Method

Both texture and depth data use 35 options: 33 directional prediction modes, a DC mode and a planar mode in the conventional intra coding. Figure 1 shows the number of 33 intra prediction modes.

Figure 1. Conventional intra prediction mode directions

For depth intra coding, the proposed method reduces the number of conventional intra prediction mode directions. As a result, total 11 directional prediction modes are selected. Figure 2 shows the 9 possible intra prediction directions and the mapping between the intra prediction direction and the intra prediction mode number, respectively.

(a) 9 intra prediction directions                  (b) Mapping between intra prediction

direction and intra prediction mode

Figure 2. Intra prediction mode for depth map coding

# Experimental Results

The proposed method is integrated into HTM 11.0 reference software and all simulations are conducted under common test conditions (CTC) and all-intra configuration (AI). Table 1 and Table 2 show the results under CTC and AI, respectively.

Table 1 Results of the number of 11 intra prediction modes for depth map under CTC

Table 1 Results of the number of 11 intra prediction modes for depth map under AI

# Conclusion

The contribution method reduced the number of intra prediction modes for depth map from 35 intra prediction directions to 11 intra prediction directions. The proposed method provides no coding losses under the common test condition (CTC) and all-intra configuration (AI) with negligible encoding/decoding time.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).