| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0118 |
| --- | --- |

| Title: | CE2: Advanced boundary chain coding for depth intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungdong Seo Moonmo Koo Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 jungdong.seo@lge.com moonmo.koo@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This proposal provides a case study complexity assessment of the advanced boundary chain coding (ABC). The ABC is an improved coding tool based on the region boundary coding which separates a block into two partitions following an arbitrary contour described by a chain code. The improvement lies in both a more versatile description of arbitrary contours while achieving more efficient encoding of the associated side-information for chain description. The method of complexity assessment is based on JCT3V-H0127 which provides a complexity analysis for depth intra coding modes. This proposal also presents the experimental results of ABC mode with and without CABAC. The proposed method with CABAC reportedly shows 0.3% and 0.3% BDR gains in synthesized views under CTC and AI configurations, respectively. The coding gains of ABC without CABAC are 0.2% and 0.2% under CTC and AI configurations, respectively.

# Introduction

In the Valencia meeting, Core Experiment (CE) 2 on depth intra coding was organized to investigate depth intra coding tools proposed at the meeting. Advanced boundary chain coding (ABC) is one of the tools [1]. It separates a block into two partitions following an arbitrary contour described by chain codes. To satisfy the mandates of CE2, this proposal provides a case study of complexity assessment on ABC in 3D-HEVC.

# Complexity Assessment

In this proposal, the method of complexity assessment is based on JCT3V-H127 [2]. It follows the analysis method to extract the intrinsic algorithmic complexity that is platform-independent and transparent to platforms. At first the flowchart should be defined to assess the complexity, and then the complexity metrics were assessed in accordance with the flowchart. The metrics includes number of operations, data storage requirement, and data transfer rate.

## Algorithm description

According to the draft text for ABC, the flowchart of ABC was constructed as shown in Fig. 1. In ‘Determine partition pattern’ block, edge information, including the edge_start_side, edge_start_position, num_edge_codes, and edge_code are transmitted from bitstream, and the partitioning is processed by using the information. For the segmented regions, the predicted values are calculated from neighboring samples in ‘Derive and assign depth partition value’. ‘Segmental depth intra coding’ process is the same as the process for DMM1 and DMM4. The complexity metrics of ABC are presented and details are described in the following sections.

Fig. 1 Flowchart of ABC

## Number of operations

Since the ‘Derive and assign depth partition value’ and ‘Segmental depth intra coding’ process of ABC are identical to that of DMM4, the number of operation of ‘Determine partition pattern’ was analyzed. The pseudo code based on the draft text for ABC is as follows:

curx = (edge_start_side == 0 ?) edge_start_position : ( (edge_start_side == 1 ?) 0 : N - 1 ) cury = (edge_start_side == 0 ?) 0 : ( (edge_start_side == 1 ?) edge_start_position : edge_start_position ) prevEdgeIdx = (edge_start_side == 0 ?) 2 : ( (edge_start_side == 1 ?) 0 : 1 ) For 0 <= y < N: For 0 <= x < N-1: vertEdgeInfo[x][y] = 0 For 0 <= y < N-1: horizEdgeInfo[y] = 0 For 0 <= i <= num_edge_codes: { If(i == num_edge_codes) If( (cury == 0, and PrevEdgeIdx == 1) or (cury == N-1, and PrevEdgeIdx == 0) or (curx == 0, and PrevEdgeIdx == 2) or (curx == N-1, and PrevEdgeIdx == 3) ) CurEdgeCode = 3 Otherwise, CurEdgeCode = 0 Otherwise, CurEdgeCode = edge_code[i] AbsoluteEdgeIdx = AbsoluteEdgeIdxTable[PrevEdgeIdx][CurEdgeCode] EdgePatternIncIdx_x = EdgePatternIncIdxTable[AbsoluteEdgeIdx][0] EdgePatternIncIdx_y = EdgePatternIncIdxTable[AbsoluteEdgeIdx][1] EdgeMark = EdgeMarkingTable[PrevEdgeIdx][CurEdgeCode] If(EdgeMark & 0x1 == 1) vertEdgeInfo[curx-1][cury] = 1 If(EdgeMark & 0x2 == 1, and curx == 0) horizEdgeInfo[cury-1] = 1 If(EdgeMark & 0x4 == 1) vertEdgeInfo[curx][cury] = 1 If(EdgeMark & 0x8 == 1, and curx == 0) horizEdgeInfo[cury] = 1 curx += EdgePatternIncIdx_x cury += EdgePatternIncIdx_y PrevEdgeIdx = AbsoluteEdgeIdx } startPartitionIdx = 0 For 0 <= y < N: { partitionIdx = startPartitionIdx If(y < N-1) If(horizEdgeInfo[y] == 1) startPartitionIdx = !startPartitionIdx For 0 <= x < N: { partitionPattern[x][y] = partitionIdx If(x < N-1) If(vertEdgeInfo[x][y] == 1) partitionIdx =! partitionIdx } }

In the pseudo code, addition (subtraction), not operation (logical operation) and LUT access are employed to obtain the partition pattern. The number of each operation is as follows:

- Number of additions: 4N+2

  - ( log2(N)+2 + 2 ) bits addition operation is applied.

  - The maximum value of ‘num_edge_codes’ is 2N.

  - In the worst case, the addition operation is repeated 2N+1 times.

- Number of logical operations: N2 – 1

  - (N-1) + N × (N-1) = N2-1

- Number of LUT access: 3N2 + 14N + 5

  - AbsoluteEdgeIdxTable (3bits × 8 × 5): 2N+1

  - EdgePatternIncIdxTable (2bit × 8 × 2): 2N+1

  - EdgeMarkingTable (4bit × 8 × 5): 2N+1

  - vertEdgeInfo (1bit × (N-1) × N): N × (N-1) (init. write) + 2 × (2N+1) (write) + N × (N-1) 
(read) = 2N2 + 2N + 2

  - horizEdgeInfo (1bit × (N-1)): (N-1) (init. write) + 2 × (2N+1) (write) + (N-1) (read) = 6N

  - partitionPattern (1bit × N × N): N2 (write)

Table 1 Number of operations of one PU

| Process |  | Add/Sub |  | LUT access |  | Logical operation (Not) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DLT |  |  |  |  |
|  |  | ON | OFF | ON | OFF |  |
| Determine partition pattern |  | 4N+2 |  | 3N2 + 14N + 5 |  | N2 - 1 |
| Derive and assign depth partition value |  | 3+N2 | 3+N2 | 3+3N2 | 3+N2 | N/A |
| Segmental depth intra coding | ON | N2 | N2 | 4N2 | N2 | N/A |
|  | OFF | N2 | N/A | 2N2 | N/A | N/A |
| Total | ON | 2N2 + 4N + 5 | 2N2 + 4N + 5 | 10N2 + 14N + 8 | 4N2 + 14N + 8 | N2 - 1 |
|  | OFF | 2N2 + 4N + 5 | N2 + 4N + 5 | 8N2 + 14N + 8 | 4N2 + 14N + 5 | N2 - 1 |

## Storage requirement

Input buffer of ABC should store edge information, above and left reference samples, and output buffer stores the reconstructed block. Some intermediate data should be stored, such as current positions and previous edge index. The tables for decoding process are also stored. The algebraic expression of storage requirement according PU size is listed in Table 2, and numeric expression of storage requirement without segmental depth intra coding process and with the process at different PU size are presented in Table 3 and Table 4, respectively. the numeric expression for the segmental depth intra coding process is based on JCT3V-H127.

Table 2 Algebraic expressions of storage requirement (N is PU size)

| Data | Number of bits |
| --- | --- |
| edge_start_side | 2 |
| edge_start_position | log2(N) |
| num_edge_codes | log2(2N) |
| edge_code | 3 x 2N |
| curx | log2(N)+2 |
| cury | log2(N)+2 |
| prevEdgeIdx | 3 |
| AbsoluteEdgeIdxTable | 120 |
| EdgePatternIncIdxTable | 32 |
| EdgeMarkingTable | 160 |
| vertEdgeInfo | N2 - N |
| horizEdgeInfo | N - 1 |
| startPartitionIdx | 1 |
| partitionIdx | 1 |
| Total | N2 + 6N + log2(2N) + 3log2(N) + 322 |

Table 3 Numeric expression of storage requirement

| PU size | Total bits |
| --- | --- |
| 4 × 4 | 371 |
| 8 × 8 | 447 |
| 16 × 16 | 691 |
| 32 × 32 | 1,559 |
| 64 × 64 | 4,827 |

Table 4 Numeric expression of storage requirement with segmental depth intra coding process

| PU size | Total bits |
| --- | --- |
| 4 × 4 | 256 + 371 = 627 |
| 8 × 8 | 688 + 447 = 1135 |
| 16 × 16 | 2416 + 691 = 3107 |
| 32 × 32 | 9328 + 1559 = 10887 |
| 64 × 64 | 37072 + 4827 = 41899 |

## Data transfer rate

Except for segmental depth intra coding process, edge information is the only data to be transferred between ABC and other modules. The algebraic expressions of data transfer rate established according to PU size are listed in Table 5. The numeric results without segmental depth intra coding process and with the process are presented in Table 6 and 7, respectively. The numeric data for the segmental depth intra coding process is based on the JCT3V-H127.

Table 5 Algebraic expressions of data transfer rate (N is PU size)

| Data | Number of bits |
| --- | --- |
| edge_start_side | 2 |
| edge_start_position | log2(N) |
| num_edge_codes | log2(2N) |
| edge_code | 3 x 2N |
| Total | 6N + log2(2N) + log2(N) + 2 |

Table 6 Numeric expression of data transfer rate at different PU size

| PU size | Total bits |
| --- | --- |
| 4 x 4 | 31 |
| 8 x 8 | 57 |
| 16 x 16 | 107 |
| 32 x 32 | 205 |
| 64 x 64 | 399 |

Table 7 Numeric expression of data transfer rate with segmental depth intra coding process

| PU size | Bits/PU |
| --- | --- |
| 4 × 4 | 240 + 31 = 271 |
| 8 × 8 | 672 + 57 = 729 |
| 16 × 16 | 2400 + 107 = 2507 |
| 32 × 32 | 9312 + 205 = 9517 |
| 64 × 64 | 37056 + 399 = 37455 |

## Analysis of coded bins with CABAC

For ABC mode, some edge information, including edge_start_side, edge_start_position, and edge_codes, is encoded with CABAC, and the other information, edge_start_position, is encoded as bypass. Table 8 shows the average number of bins encoded with CABAC on each PU size. About 64 bins are used to encode a 32×32 PU with CABAC for ABC mode, and about 9 bins are used for a 4×4 PU. However, the number of encoded bins with CABAC may not have a significant effect on the whole process, because ABC mode is selected rarely. The ratio of coded bins for ABC mode to total bins encoded is presented in Table 9. About 1.18% of whole CABAC bins are encoded by CABAC for ABC, and 1% of whole bins of 3D-HEVC are employed for the mode.

Table 8 Number of bins with CABAC for ABC mode at different PU size

|  | Balloons | Kendo | Poznan Hall2 | Newspaper | Poznan Street | GT_fly | Undo dancer | Shark | Avg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32×32 | 67.23 | 66.40 | 66.12 | 64.65 | 68.27 | 67.11 | 64.75 | 59.03 | 65.44 |
| 16×16 | 34.38 | 34.64 | 34.14 | 33.90 | 34.92 | 35.58 | 33.79 | 33.15 | 34.31 |
| 8×8 | 17.47 | 18.15 | 17.92 | 18.30 | 18.58 | 19.32 | 18.09 | 18.33 | 18.27 |
| 4×4 | 9.01 | 9.16 | 9.19 | 9.48 | 9.38 | 9.15 | 9.14 | 9.67 | 9.27 |

Table 9 Ratio of encoded bins for ABC mode (%)

|  | Balloons | Kendo | Poznan Hall2 | News paper | Poznan Street | GT_fly | Undo dancer | Shark | Avg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CABAC bins for ABC/ Total CABAC bins | 1.07 | 1.80 | 0.60 | 1.59 | 1.59 | 0.88 | 0.82 | 1.52 | 1.23 |
| Bins for ABC/ Total bins | 0.90 | 1.47 | 0.54 | 1.34 | 1.34 | 0.75 | 0.71 | 1.28 | 1.04 |

## Overall summary

Table 10, 11, and 12 show the numerical representations of required resource for a 64x64 CTU, and table 13 shows an overall reports on complexity of ABC and DMM coding tools. In the tables, the red letter indicates the worst case.

In terms of number of operations, ABC mode requires similar number of addition and logical operations with DMM4 and more look up table assess than DMM series. However, it is possible to remove the tables for decoding of ABC mode by substituting logic modules. The logic module for replacing the tables can be implemented in 88 gates. In terms of storage size and transfer rate, ABC requires the similar amount to DMM1, and that is smaller than the amount of DMM4.

Table 10 Number of operations of depth intra modes for one 64×64 CTU

|  | DMM 1 |  | DMM 4 |  | ABC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PU size | Add/Sub | LUT access | Add/Sub/ Comp | LUT access | Add/Sub | LUT access | Logical operation |
| 4×4 | 8,960 | 26,368 | 17,152 | 29,440 | 13,568 | 57,344 | 3,840 |
| 8×8 | 8,384 | 25,024 | 16,576 | 28,864 | 10,560 | 48,640 | 4,032 |
| 16×16 | 8,240 | 24,688 | 16,432 | 28,720 | 9,296 | 44,672 | 4,080 |
| 32×32 | 8,204 | 24,604 | 16,396 | 28,684 | 8,724 | 42,784 | 4,092 |
| 64×64 | 8,204 | 24,604 | 16,396 | 28,684 | 8,462 | 42,784 | 4,095 |

Table 11 Storage requirement of depth intra modes for one 64×64 CTU

| PU size | DMM 1 | DMM 4 | ABC |
| --- | --- | --- | --- |
| 4×4 | 256 | 384 | 627 |
| 8×8 | 688 | 1,200 | 1,135 |
| 16×16 | 2,416 | 4,464 | 3,107 |
| 32×32 | 9,328 | 17,520 | 10,887 |
| 64×64 | 37,072 | 69,840 | 41,899 |

Table 12 Data transfer rate of depth intra mode for one 64×64 CTU

| PU size | DMM 1 | DMM 4 | ABC |
| --- | --- | --- | --- |
| 4×4 | 64,256 | 90,112 | 69,376 |
| 8×8 | 43,712 | 71,680 | 46,656 |
| 16×16 | 38,576 | 67,072 | 40,112 |
| 32×32 | 37,292 | 65,920 | 38,068 |
| 64×64 | 37,069 | 65,728 | 37,455 |

Table 13 worst case complexity of depth intra mode at 64×64 CTU

| Complexity Metrics | DMM 1 | DMM 4 | ABC |
| --- | --- | --- | --- |
| Number of operations (logical) | 0 | 0 | 4,095 |
| Number of operations (Add/Sub/Comp) | 8,960 | 17,152 | 13,568 |
| Number of operations (LUT access) | 26,368 | 29,440 | 57,344 |
| Storage requirement (Bits) | 9,328 | 17,520 | 10,887 |
| Data transfer rate (Bits per 6464 block) | 61,440 | 90,112 | 69,376 |

# Experimental results

The proposed method was integrated into HTM-11.0 reference software and two test sets were conducted under random access (CTC) and intra-only configuration. [3]

## Test 1: results of ABC

In Test 1 ABC was implemetned with CABAC for encoding the edge information. ABC provides 0.3 % and 0.3% coding gains in synthesized views under CTC and AI configurations.

Table 14 Results of ABC in CTC

Table 15 Results of ABC in AI

## Test2: results of ABC without CABAC

In Test 2, CABAC for edge information of ABC was removed. The proposed method provides 0.2 % and 0.2% coding gains in synthesized views under CTC and AI configurations.

Table 16 Results of ABC without CABAC in CTC

Table 17 Results of ABC without CABAC in AI

# Conclusion

This proposal provides a case study complexity assessment of the advanced boundary chain coding (ABC). The complexity assessment is based on JCT3V-H0127 and performed in terms of number of instruction, storage requirement, and data transfer rate. This proposal also presents the experimental results of ABC. The BDR gains of the proposed method are 0.3% and 0.3% in synthesized views under CTC and AI configurations, respectively

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JCT3V-H121, “Advanced boundary chain coding for depth intra mode”, Valencia, ES, 29 March – 4 April 2014.

[2] JCT3V-H0127, “AHG8: Complexity Assessment on Depth Intra Modes”, Valencia, ES, 29 March – 4 April 2014.

[3] JCT3V-H1001, “3D-HEVC Draft Text 4,” Valencia, ES, 29 March – 4 April 2014.