| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0119 |
| --- | --- |

| Title: | Cross check of separate enabling flag for intra coding tools (JCT3V-I0085) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philipp Merkle | Email: | philipp.merkle@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Cross check results for JCT3V-I0085 on separate enabling flag for intra coding tools by Samsung are reported. Performance evaluation results for CTC configuration reported by the proponent were verified.

# Introduction

In JCT3V-I0085, Samsung proposes separate enabling flags for intra coding tools, namely intraSDC, DMM1, DMM4 and DLT. As cross-checker, we received their source code and simulation results for CTC configuration according to JCT3V-G1103. The source code was compiled and the corresponding simulations were conducted within our lab on a clustered system with Linux OS and GCC 4.5.5 compiler.

# Results

Encoding and decoding YUV files (md5sum) matched. More details of the cross check results are summarized in Table 1.

Table 1. Result for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | 98.8% | 97.9% | 97.1% |
| Kendo | 0.0% | 0.0% | 0.00% | 98.9% | 97.5% | 98.5% |
| Newspapercc | 0.0% | 0.0% | 0.00% | 99.7% | 98.5% | 98.7% |
| GhostTownFly | 0.0% | 0.0% | 0.00% | 99.9% | 104.6% | 98.2% |
| PoznanHall2 | 0.0% | 0.0% | 0.00% | 101.5% | 100.1% | 99.6% |
| PoznanStreet | 0.0% | 0.0% | 0.00% | 100.1% | 99.9% | 99.9% |
| UndoDancer | 0.0% | 0.0% | 0.00% | 100.1% | 100.3% | 100.9% |
| Shark | 0.0% | 0.0% | 0.00% | 99.8% | 99.1% | 99.1% |
| 1024x768 | 0.0% | 0.0% | 0.00% | 99.1% | 98.0% | 98.1% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | 100.3% | 100.8% | 99.5% |
| average | 0.0% | 0.0% | 0.00% | 99.9% | 99.8% | 99.0% |

# Conclusion

Samsung provided the source code of their proposal for HTM-11.0 together with a short explanation on how to enable and compile the proposed algorithm. All the modifications to the reference software were encapsulated in preprocessor statements for conditional compilation. Overall the implementation of the proposed algorithm is done in a clean and maintainable way and does not interfere with other coding tools. The proposed specification text changes clearly specify the proposed algorithm.

The cross check results were matched to those provided by the proponent.